| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D612 WG11 Number: m19508 |
| --- | --- |

| Title: | Description of CE12: Deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Andrey Norkin1 (primary coordinator) Jeongyeon Lim2 Byeungwoo Jeon3 (coordinator) Xun Guo4 Jizheng Xu5 Matthias Narroschke6 (coordinator) Teruhiko Suzuki7 | Email: | andrey.norkin@ericsson.com jeongyeon@sktelecom.com bjeon@skku.edu xun.guo@mediatek.com jzxu@microsoft.com matthias.narroschke@eu.panasonic.com teruhikos@jp.sony.com |
| Source: | Ericsson1, SK Telecom2, SKKU3, Mediatek4, Microsoft7, Panasonic8, Sony9 |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE) is to enhance deblocking filtering techniques in the HEVC Test Model. Techniques covered by this CE are deblocking filter improvements and deblocking filter parallelization. This is not limited to improving coding efficiency, subjective quality and parallelization, but also reducing complexity and harmonization of the various schemes that are technically feasible.

Contributions in this CE are divided into two subsets:

Subset 1 includes contributions that aim at improving objective and/or subjective quality of the current HEVC deblocking filter. For these proposals, the impact of modifications on subjective and objective quality is evaluated. The complexity of the proposed schemes relative to the current HEVC deblocking is also evaluated. In addition, subjective viewing shall be conducted according to the suggestion of viewing experts.

Subset 2 of the current CE includes parallelization techniques for deblocking filters. For these techniques, degree of parallelization is considered as well as their impact on objective and subjective quality and decoding complexity. Memory storage requirements are also to be evaluated.

The software basis for this CE is HM 2.0 that will be announced available on February 11, 2011, and the experiments will use the test conditions in JCTVC-D600.

# Participants

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 | Andrey Norkin (primary coordinator) Shawmin Lei Xun Guo Yu-Wen Huang Jizheng Xu Xiaoyan Sun Matthias Narroschke (coordinator) Jeongyeon Lim Jungyoup Yang Byeungwoo Jeon (coordinator) Kenneth Andersson Teruhiko Suzuki Masaru Ikeda Tomoo Yamakage Vadim Seregin In Suk Chong Geert Van Der Auwera | Ericsson MediaTek MediaTek MediaTek Microsoft Microsoft Panasonic SK Telecom SKKU SKKU Ericsson Sony Sony Toshiba Samsung Qualcomm Qualcomm | andrey.norkin@ericsson.com shawmin.lei@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com jzxu@microsoft.com xysun@microsoft.com Matthias.Narroschke@panasonic.de jeongyeon@sktelecom.com binbak1@ece.skku.ac.kr bjeon@skku.edu kenneth.r.andersson@ericsson.com teruhikos@jp.sony.com Masaru.Ikeda@jp.sony.com tomoo.yamakage@toshiba.co.jp v.seregin@samsung.com ichong@qualcomm.com geertv@qualcomm.com | Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent Proponent |

# Experimental Conditions

## Software

HM 2.0 shall be used (will be provided on February 11, 2011).

## Test Sequences, Bit Rates and Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-D600 will be used. This includes the newly added sequences in Class A. Providing results for additional sequences is encouraged but not mandated. Results for all six test cases shall be provided.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and/or complexity as well as subjective quality.

Complexity assessment is performed on the base of decoding complexity measurement. For complexity assessment, HM 2.0 software with the anchor configuration and the proposal implemented on the HM 2.0 software shall be used. For decoder complexity measurements, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

Subjective viewing before or during the Geneva meeting should be performed and results provided during the meeting in order to ensure that the proposals do not cause degradation of subjective quality. The test sequences and test conditions should be determined before the beginning of cross-check.

# Description of Core Experiment

## Subset 1: Deblocking filters improving objective or/and subjective performance

### Functionality Addressed

- Reduction of blocking artifacts

- Improvement of coding efficiency

- Improvements in subjective quality

### Expected Gains

- Subjective quality improvement

- BD-rate reduction

### Evaluation Criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement, memory storage, etc.)

- Subjective video quality

Note 1: HM 2.0 shall be used as the anchor.

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and time-slot for informal viewing are available at the Geneva meeting, informal viewing will be conducted. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the Geneva meeting are decided before the cross-check start date. The selection should be conducted by CE12 participants including non-proponents.

Note 3: The proponents are encouraged to provide additional results for separate tools (groups of tools) in their proposals if the proposal consists of several independent modifications of the HM software. The possibility to turn on and off these groups of tools should be provided in the software. This specifically applies to adjustments of the thresholds and tables as well as to modifications to the encoder control and implementations, which could also be applied to improve the anchor. The cross-checkers should have a possibility to assess, which part of the coding gain comes from the thresholds/tables adjustment or from modifications to the encoder control or implementations. The cross-checkers are encouraged to perform the subjective viewing of the separate tools/combinations of tools in order to ensure that removing or adding certain tools from the proposal does not degrade the subjective quality.

### Mode-dependent deblocking filter (JCTVC-D163, MediaTek)

#### General Concept

This contribution describes MediaTek’s proposal, which is based on deblocking filter in HM1.0 and a previous proposal in Guangzhou meeting (C142). There are three major modifications in deblocking filtering: mode dependent deblocking filter (MDDF), modified filtering conditions and thresholds (MFCT) and modified filter operation (MFO). In MDDF, intra mode information is used to define different boundary types (BT), which are used to adjust filter strength and directions. In MFCT, several conditions have been modified: the conditions which determine if to perform filtering of an edge or not, and the conditions which determine whether to use a weak filter or a strong filter. The thresholds and filter decisions used are similar to ones in H.264 (α and β thresholds). The corresponding thresholds have been modified relative to H.264. In MWF, filtering operations for luma weak filter and chroma filter are modified.

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Content-adaptive deblocking (JCTVC-D137, Microsoft)

Figure 1 Extra smoothing deblocking (Mode B). A vertical boundary of a luminance block is taken for example.  are the pixels to be filtered. In this example, the filtered value of q4 is obtained by averaging .

There are two deblocking modes in the proposed scheme, the HEVC deblocking mode (Mode A), which performs exactly the same filtering as in the current HEVC deblocking, and the extra smoothing deblocking mode (Mode B), which performs a strong and long distance filtering across certain transform unit (TU) boundaries.

Deblocking Mode B is applied in large smooth regions with small variation. In the proposed scheme, TU boundaries satisfy the following conditions will be processed with deblocking Mode B.

- The two adjacent TU’s related to the TU boundary are both in intra-coded CU’s;

- The sum of luminance variance in the two TU's is smaller than a certain threshold (set to 8 in the proposed scheme);

- The smaller transform size between the two TU's is larger than a certain value (set to 8 in the proposed scheme);

- The larger transform size between the two TU's is larger than a certain value (set to 16 in the proposed scheme).

Other TU boundaries will be processed with deblocking Mode A.

For deblocking Mode B, the filtering is performed along lines perpendicular to the TU boundary. As shown in Figure 1, suppose  are the pixels in the same row or column of two adjacent TU’s, during filtering their values are modified as

This is a heavy and long distance smoothing across the TU boundary and pixels in large smooth regions are preferred to be involved. For deblocking Mode B, no boundary strength and filter thresholds are needed.

Once the deblocking mode for each TU boundary is determined, the two different deblocking modes are simultaneously employed in a raster-scan order at the TU level for both luminance and chroma components. The reconstructed frame is obtained in a single deblocking pass.

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Intra deblocking filter by SKT/SKKU (JCTVC- D334, SKKU)

#### Key concepts

In the current design of deblocking filter in HM1.0, if blocks P and Q are intra coded, the boundary between them is always filtered. However, in the proposed method, a boundary between intra blocks is not filtered if intra prediction modes of the blocks P and Q are the same and if they don’t contain any non-zero transform coefficients. It is based on the understanding that in this case, the blocks are expected to be very similar in their contents and the blocking artifact may not be conspicuous. Other filtering decision conditions are the same as the design of deblocking filter in HM1.0. If the boundary between blocks P and Q is decided to be filtered by the filtering decision process, the proposed method controls the strength of deblocking filter for intra coded blocks as follows. If blocks P and/or Q are intra-coded and their prediction modes are different, the same filtering process of HM1.0 is performed with tC plus 2 since the boundary between them may have strong blocking artifacts. It is to note that one of two types of filtering (weak and strong filtering) is applied by using the same decision criterion in HM1.0. Otherwise, intra prediction modes of P and Q are considered to jointly control the strength of filtering. Different intra prediction modes of P and Q indicate that those two blocks are predicted in different directions, thus the effects of intra prediction and quantization of them are expected to be non-identical. In this case, the same filtering process in HM1.0 is performed with tC plus 1. Otherwise, if intra prediction modes of P and Q are the same, P and Q are most likely to be in the same region. Therefore, in this case, a weak filtering with tC is only applied. Otherwise, a weak smoothing filter with tC is only applied.

Since deblocking filtering is applied to vertical and horizontal block boundaries, vertical and horizontal intra prediction modes are considered to check the difference between boundary direction and prediction direction. Other filtering operations are the same as in HM1.0.

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Development of HEVC deblocking filter (JCTVC-D377, Ericsson)

This proposal contains modifications to the HEVC deblocking filter. These modifications are: a different way to calculate deblocking filter coefficients, filtering restrictions and filtering length adaptation to block sizes. The tc and β thresholds have also been adjusted.

The proposed filtering operations use different deltas when modifying pixels at the second position from the block boundary. This is different from the current HEVC filter which uses the same delta. The filter uses additional restrictions on number of pixels from the block boundary to be modified. Pixels p1 and q1 (at the second position from the block border) are modified only if abs() < thr1. This condition differentiates blocking artifacts from natural edges and reduces computational complexity of the filter.

Another condition limits filtering strength if there is no blocking artifact. The second pixel from the block boundary is modified if abs() > thr2. This condition limits the amount of filtering applied to the second pixel if there are no blocking artifacts (the delta for the first pixel is close to zero).

The filter modifies different number of pixels depending on the sizes of the blocks adjacent to the block boundary. The smaller block size of the blocks adjacent to the block border is used for the decision. For smaller blocks, smaller number of pixels from the block boundary can be modified.

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Participants

| Participant | Contact |
| --- | --- |
| MediaTek | Xun Guo xun.guo@mediatek.com |
| Microsoft | Jizheng Xu jzxu@microsoft.com Xiaoyan Sun xysun@microsoft.com |
| SKT/SKKU | Byeungwoo Jeon bjeon@skku.edu Jungyoup Yang binbak1@ece.skku.ac.kr |
| Ericsson | Andrey Norkin andrey.norkin@ericsson.com |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com |
| Toshiba | Tomoo Yamakage tomoo.yamakage@toshiba.co.jp |
| Samsung | Vadim Seregin v.seregin@samsung.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| MediaTek (D163) | Ericsson, Sony |
| SKT/SKKU (D334) | Microsoft |
| Microsoft (D137) | MediaTek |
| Ericsson (D377) | SKT/SKKU, Toshiba, Samsung, MediaTek |

## Subset 2 Parallel deblocking filters

### Functionality Addressed

- Filter design friendly to parallel processing

## Expected Gains

- More flexibility of parallel processing without subjective and objective quality degradation

## Evaluation Criteria

- BD-rate relative to the anchor(s)

- Complexity (decoding time measure, memory storage, etc.)

- Impact on subjective quality

- Degree of achieved parallelization

Note 1: HM 2.0 shall be used as the anchor.

Note 2: Cross-checking includes informal viewing of proposals. All cross-checkers shall report the informal viewing results (without scoring) and their equipments (e.g., monitor, etc). If a room, equipment, and a time-slot for informal viewing are available in the Geneva meeting, informal viewing will be conducted. The selection of test sequences, QP values, and coding conditions for the potential informal viewing in the Geneva meeting are decided before the cross-check start date. The selection should be conducted by CE12 participants including non-proponents.

Note 3: For parallel processing category, impact on picture quality is evaluated. It will be evaluated if parallel processing capability is supported without any picture quality degradation. The complexity impact is also evaluated. For example the size of extra memory is evaluated.

Note 4: The proponents are encouraged to provide additional results for separate tools (groups of tools) in their proposals if the proposal consists of several independent modifications of the HM software. The possibility to turn on and off these groups of tools should be provided in the software. This specifically applies to adjustments of the thresholds and tables as well as to modifications to the encoder control and implementations, which could also be applied to improve the anchor. The cross-checkers should have a possibility to assess, which part of the coding gain comes from the thresholds/tables adjustment or from modifications to the encoder control or implementations. The cross-checkers are encouraged to perform the subjective viewing of the separate tools/combinations of tools in order to ensure that removing or adding certain tools from the proposal does not degrade the subjective quality.

### Parallel Deblocking Filter (JCTVC-D263, Sony)

#### General concept

In the current HM deblocking filter, horizontal filter (across vertical edge) is dependent on vertical filter (across horizontal edge) between two consecutive Largest Coding Units (LCU) because the filtering is first performed across vertical edges, then across horizontal edges in LCU. The proposed technique provides possibilities for parallel processing in deblocking filtering.

#### Parallelism of filtering

In the proposed deblocking filter process, the order of horizontal filtering (across vertical edge) and vertical filtering (across horizontal edge) is changed. The changes are as follows. First, the filter operates across all the vertical edges in processing frame, then across all the horizontal edges in the frame. As a result horizontal filtering (across vertical edge) is independent of vertical filtering (across horizontal edge), so both horizontal and vertical filtering can be parallelized.

Figure 2: Proposed filter process in LCU unit

#### Parallelism of filtering decisions for block boundary

As shown in Figure 3 (a), filtering decision across horizontal edge uses both 3rd and 6th lines in conventional filter, and the decision in #n LCU which uses red pixels depends on horizontal filtering across vertical edge in #n LCU. It is proposed that filtering decisions use 4th and 5th lines in order to remove the dependency as shown in Figure 3 (b). These lines (4th and 5th) consist of not filtered pixels but only reconstructed pixels. The proposed modifications of filtering decisions allow to perform filtering decisions for all block edges in the frame before the actual filtering and in parallel.

Figure 3: Edge judgment of block boundary for deblocking filter

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Parallel filter decision for de-blocking (JCTVC-D214, Panasonic)

The basic idea of this proposal is to perform all decisionsd1,v, d1,h, d2,v, d2,h, Di,v, Di,h based on the non-deblocked signal of the current coding unit. This dissolves dependencies between the horizontal and vertical filtering which are currently limiting the degree of parallelization.

In order to deblock a coding unit of the size 8x8 samples according to the proposal, the following 4 sequential steps are necessary:

- Performing the decisions d1,v, d2,v,d1,h, and d2,h

- Performing the decisions Di,v, with i=1,…,8 and Di,h, with i=1,…,8

- Performing the filtering operations Fi,v, with i=1,…,8 and Fi,h, with i=1,…,4

- Performing the filtering operations Fi,h, with i=5,…,8

Figure 4 to Figure 7 illustrate these 4 sequential steps. Furthermore they show the samples used for the individual operations.

Figure 4: Illustration of samples required to perform the decisionsd1,v, d1,h, d2,v, d2,h according to the proposal.

Figure 5: Illustration of samples required to perform the decisionsDi,v, Di,haccording to the proposal.

Figure 6: Illustration of samples required to perform the filtering operations F1,v−F8,vand F5,h−F8,h according to the proposal.

Figure 7: Illustration of samples required to perform the decisions according to the proposal.

As the performance of all decisionsd1,v, d1,h, d2,v, d2,h, Di,v, Di,h based on the non-deblocked signal of the current coding unit dissolves the dependencies, the parallel processing capabilities are increased. This is illustrated in Figure 8. As a consequence, the number of sequential operations in the critical path is reduced from 44 of the HM 0.9 to only 32. This is a reduction of around 30%.

Figure 8: Illustration of dependencies and the number of sequential operations in the critical path according to the proposal.

#### Bitstream organization

No need to modify the bitstream syntax (i.e. only require the modification of decoding process).

### Participants

| Participant | Contact |
| --- | --- |
| Panasonic | Matthias Narroschke Matthias.Narroschke@eu.panasonic.com |
| Sony | Teruhiko Suzuki teruhikos@jp.sony.com Masaru Ikeda Masaru.Ikeda@jp.sony.com |
| Qualcomm | In Suk Chong ichong@qualcomm.com Geert Van Der Auwera geertv@qualcomm.com |

Cross-check assignment

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony (D263) | Panasonic, Qualcomm |
| Panasonic (D214) | Sony, Qualcomm |

# Time-line and Responsibilities

2011-Feb-11:	Final CE description uploaded

2011-Mar-2:	Cross-checking begins, proponents provide software and results to cross-checkers

2011-Mar-3:	Proponents report results to CE12 coordinators

2011-Mar-9:	Cross-checkers report results to CE12 coordinators

2011-Mar-11:	Input documents uploaded

# Previous Contributions

JCTVC-D163 	[J. An, X. Guo, Q. Huang, Y.-W. Huang, S. Lei (MediaTek)] CE8 Subtest1: Improved Deblocking Filter.

JCTVC-D137	[Z. Shi, Z. Xiong, X. Sun, J. Xu (USTC, Microsoft)] CE8, Subset 1: Report of Content-Adaptive Deblocking.

JCTVC-D334	[J. Yang, K. Won, H. Yang, B. Jeon, J. Lim (SK Telecom and Sungkyunkwan University)] CE8 subset 1: Results of intra deblocking filter testing by SKKU/SKT (JCTVC-C130).

JCTVC-D377	[A. Norkin, K. Andersson, R. Sjöberg (Ericsson)] Development of HEVC deblocking filter.

JCTVC-D263	[M. Ikeda, J. Tanaka, T. Suzuki (Sony)] Parallel deblocking filter.

JCTVC-D214	[M. Narroschke, H. Sasai, T. Wedi (Panasonic)] Reduction of operations in the critical path of the deblocking filter.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, Jan. 2011.

[2]	K. McCann, B. Bross, S. Sekiguchi, “Description of HEVC Test Model 1”, JCTVC-C402, Guangzhou, October 2010.

[3]	T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “HEVC Text Specification Working Draft 1”, JCTVC-C403, Guangzhou, October 2010.