| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D614 WG11 Number: m19505 |
| --- | --- |

| Title: | Description of Core Experiment 14: Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Shawmin Lei1 (Coordinator) Marta Karczewicz2 (Coordinator) TK Tan3 Wenpeng Ding4 Guichun Li5 | shawmin.lei@mediatek.com martak@qualcomm.com tktan.p@rd.nttdocomo.co.jp wpding@bjut.edu.cn g1li@scu.edu |  |
| Source: | MediaTek1, Qualcomm2, DOCOMO3, Beijing Univ. of Technology4, Santa Clara Univ5 |  |  |

_____________________________

# Introduction

This CE includes five proponents which are S. Lei (MediaTek), TK Tan (NTT DOCOMO, Inc.), W. Ding (Beijing University of Technology), G. Li (Santa Clara University), and L. Liu (HiSilicon). The goal of this Core Experiment (CE) is to improve the intra mode coding. There are two Subtests in this CE: the most probable mode signaling for luma (Subtest1) and intra mode derivation with multiple predictor sets (Subtest2). For Subtest1, two possible candidate prediction modes from the left and the above blocks of the current block would be used instead of using the minimum one. If the two candidates are different and one of them is the same with the current mode, one additional flag is transmitted after the most probable mode to indicate which one would be used as a predictor. The proponents of the Subtest 1 include S. Lei (MediaTek), TK Tan (NTT DOCOMO, Inc)., and W. Ding (Beijing University of Technology). For Subtest2, Multiple Predictor Sets (MPS) are used to improve the intra mode coding. The proponents of the Subtest 2 include G. Li (Santa Clara University) and L. Liu (HiSilicon).

# Participants List

| Company | Participants |
| --- | --- |
| MediaTek | Shawmin Lei, shawmin.lei@mediatek.com Xun Guo, xun.guo@mediatek.com Mei Guo, mei.guo@mediatek.com |
| DOCOMO | TK Tan , ttk@pacific.net.sg |
| Beijing University of Technology | Wenpeng Ding, wpding@bjut.edu.cn |
| Qualcomm | Marta Karczewicz, martak@qualcomm.com Muhammed Coban, mcoban@qualcomm.com |
| Santa Clara University | Guichun Li, g1li@scu.edu |
| HiSilicon | Lingzhi Liu, lzliu@huawei.com |
| Samsung | Jianle Chen, jianle.chen@samsung.com |
| Institute for Infocomm Research | Chuohao Yeo, chyeo@i2r.a-star.edu.sg Hui Li Tan, hltan@ i2r.a-star.edu.sg |
| Canon | Edouard Francois, edouard.francois@crf.canon.fr |
| Sharp | Andrew Segall, asegall@sharplabs.com |
| Toshiba | Akiyuki Tanizawa, akiyuki.tanizawa@toshiba.co.jp Taichiro Shiodera, taichiro.shiodera@toshiba.co.jp |

# Description of Tools

## Subtest1: The Most Probable Mode Signaling for Luma

In the 4th JCT-VC meeting in Daegu, Korea, three proposals (JCTVC-D166 [2], JCTVC-D235 [3], JCTVC-D253 [4]) from three proponents as listed in Section 3.1.1 are essentially the same. The goal of this subtest is to verify the performance of this essentially the same proposal under the new HM2.0 and to make sure that it is harmonized with the newly adopted LCEC technique for intra mode coding described in JCTVC-D366 [5].

### Proponents

| Proponent | Document | Contact | Cross-checker |
| --- | --- | --- | --- |
| MediaTek | JCTVC-D166 | Xun Guo, xun.guo@mediatek.com | Qualcomm, Marta Karczewicz, martak@qualcomm.com I2R, Chuohao Yeo, chyeo@i2r.a-star.edu.sg Toshiba, Akiyuki Tanizawa akiyuki.tanizawa@toshiba.co.jp Samsung, Jianle Chen, jianle.chen@samsung.com |
| DOCOMO | JCTVC-D235 | TK Tan , ttk@pacific.net.sg |  |
| Beijing University of Technology | JCTVC-D253 | Wenpeng Ding, wpding@bjut.edu.cn |  |

### Algorithm Description

Fig. 1 Illustration of neighboring PUs to the left and to the top of the current PU

In this contribution, the most probable mode of luma prediction unit (PU) is adaptively selected from the modes of two neighboring blocks instead of directly using the one with smaller mode index in HM1.0. Specifically, when the intra mode of current PU (ModeC) is being coded, the modes of the left PU (ModeA) and the upper PU (ModeB), as depicted in Fig. 1, are used to generate the most probable mode of ModeC in Section 3.1.2.1. If ModeC is equal to candModeList[MostProbableIndex] then the MostProbableMode of value 1 is sent followed by the MostProbableIndex. MostProbableIndex can be 0 or 1, which represents the smallest one and the largest mode in the candModeList[x], respectively. Note that MostProbableIndex is equal to the final cIdx in Section 3.1.2.2 in this case.

Otherwise ModeC is not in the candModeList and the MostProbableMode of value 0 is sent followed by the remMode. The remMode is derived in Section 3.1.2.2.

The derivation of the ModeC in the decoder from the remMode received is described in section 3.1.2.3.

The above algorithm is defined by the following processes

#### Candidate modes list derivation. (encoder and decoder)

The inputs to this process are the prediction modes ModeA and ModeB of the partition above and to the left of the current partition, respectively.

The output of this process is a candidate mode list candModeList[x] containing up to two possible candidate prediction modes.

The candidate prediction modes are ModeA and ModeB.  If a candidate prediction mode is not available, then it is replaced by the DC_pred mode.

The candModeList[x] is derived as follows

- If both candidate prediction modes are the same, then this candidate prediction mode is added to candModeList[0].

- Otherwise, both candidate prediction modes are added to the candidate modes list with the smaller of the two candidates at candModeList[0] and the larger at candModeList[1].

#### Remainder mode derivation. (encoder)

The inputs to this process are the candidate modes list candModeList[x], the number of candidate modes in the list numCand and the prediction mode of the current partition ModeC.

The output of this process is the remainder mode remMode.

The remMode is derived by applying the following procedure

remMode = ModeC

for (cIdx = numCand-1; cIdx >= 0; cIdx--)

if (remMode > candModeList[cIdx]) remMode--

#### Prediction mode derivation from remainder mode. (decoder)

The inputs to this process are the candidate modes list candModeList[x], the number of candidate modes in the list numCand and the remainder mode of the current partition remMode.

The output of this process is the prediction mode of the current partition ModeC.

The ModeC is derived by applying the following procedure

ModeC = remMode

for (cIdx = 0; cIdx < numCand; cIdx++)

if (ModeC >= candModeList[cIdx]) ModeC++

## Subtest2: Intra Mode Derivation with Multiple Predictor Sets (MPS) technology (JCTVC-D026 and JCTVC-C111, Santa Clara University)

### Proponents

| Proponent | Document | Contact | Cross-checker |
| --- | --- | --- | --- |
| Santa Clara University | JCTVC-D026 (JCTVC-C111) | Guichun Li, g1li@scu.edu | Mediatek, Shawmin Lei, shawmin.lei@mediatek.com Canon, Edouard Francois, edouard.francois@crf.canon.fr |

### Algorithm Description

The Intra Mode Derivation method currently used in TMuC is getting the minimum value of the prediction mode of the above prediction unit and the left prediction unit. If any of above or left block is not available, the predicted current mode will be DC.

MostProbableMode = Min ( AboveIntraMode, LeftIntraMode );

In MPS intra technology, intra prediction modes are first grouped into 4 subsets. Within each subset, there are 9 modes. The basic group, Group 0, has the same predictors as in H.264/AVC. All other groups can be obtained by rotating the basic group for a certain angle.

Each intra prediction mode number can be represented by the combination of a group number and an index number within the group.

|  | AboveIntraMode [ AboveGroup, AboveIdx ] |
| --- | --- |
| LeftMode [ LeftGroup, LeftIdx ] | MPM = ModeTable [Min(LeftGroup, AboveGroup), Min(LeftIdx,AboveIdx)] |

Fig. 2 - Left: Current Intra Mode Prediction Method Used in TMuC

Right: Proposed Intra Mode Prediction Method

The proposed intra mode prediction method in utilizes the Group Numbers and the Group Indices. The predicted current mode is determined by the minimum Group Number and the minimum Group Index from the above block and the left block, as shown in Fig. 2. Similarly, if any of above or left block is not available, the predicted current mode will be DC Mode. This method is only applied on PUs with maximum mode number of 34.

ModeProbableMode = ModeTable [ Min ( AboveGroup, LeftGroup ), Min ( AboveIdx, LeftIdx ) ];

If any of the predicted Group Number or Group Index is not the same as the actual value, the flag and the corresponding bits will be sent. Otherwise, only a flag for Group Number or a flag for Index Number is sent.

Fig. 3 – flag and bits for Group Number; flag and bits for Index Number

# Software and Configuration

## Software

The CE will use the HM2.0 software that is recommended by the Software Ad Hoc group.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configurations and conditions that are specified in JCTVC-D600. The priority will be testing two all-intra configurations. The four random-access and low-delay configurations also will be checked to insure no ill effects to the inter configurations.

# Time-line and Responsibilities

- 2011/2/11	Final CE-description

- 2011/3/3 	Completion of tool integration, start of testing

- 2011/3/9 	Cross-check completion

- 2011/3/10	Input Document upload

# Definition of Performance Measurement Criteria

- Coding performance: BD-rate

- Complexity: Running time

# References

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- M. Guo, X. Guo and S. Lei, “Improved Intra Mode Coding,” JCTVC-D166, Daegu, Korea, 20-28 January, 2011.

- S. Kanumuri, TK Tan and Frank Bossen, “Enhancements to Intra Coding,” JCTVC-D235, Daegu, Korea, 20-28 January, 2011.

- W. Ding, X. Che, Y. Shi, B. Yin, “Improved Intra Mode Coding by Multiple Mode Candidates,” JCTVC-D253, Daegu, Korea, 20-28 January, 2011.

- M. Karczewicz, X. Wang, W. Chen, and A. Fuldseth, “CE5: Improved intra prediction mode coding with LCEC,” JCTVC-D366, Daegu, Korea, 20-28 January, 2011.

- G. Li, L. Liu, N. Ling, J. Zheng, and P. Zhang, “CE6.a.5: Santa Clara University and HiSilicon Report on Block Based Intra Prediction ,” JCTVC-D026, Daegu, Korea, 20-28 January, 2011.

- G. Li, L. Liu, N. Ling, J. Zheng, and P. Zhang, “Integration of Plane Mode in Unified Intra Prediction ,” JCTVC-C111, Guangzhou, China, 7-15 October, 2010.