| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E004 WG11 Number: m19775 |
| --- | --- |

| Title: | JCT-VC AHG report: Slice support development and characterization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki Japan Michael Horowitz eBrisk Video, Inc. 1055 W Hastings St, Suite 1450 Vancouver, BC V6E 2E9, Canada Andrew Segall Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 U.S.A. | Tel: Email: | +46761153563 rickard sjoberg@ericsson com +1.778.331.3432 michael@ebriskvideo.com +1 360 817 8487 asegall@sharplabs.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the slice support development and characterization ad-hoc activities between the 4th and the 5th JCT-VC meeting and the input documents to this meeting related to this ad-hoc group.

# Mandate

Table 1	- AHG mandates

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Slice support development and characterization (jct-vc@lists.rwth-aachen.de) - Assist the software development and HM software technical evaluation ad hoc group with integration of basic slice (independently-decodable sequences of largest coding units in raster-scan order) and "entropy slice" support into the HM 2.1 software. - Identify issues relating to the draft text description of slice functionality - Consider spatial granularity of slice boundaries - Study technical proposals relating to slice structured coding - Study the coding efficiency and loss resilience impact of slice-structured coding - Identify and discuss additional issues relating to slice-structured coding | R. Sjöberg (chair), Y. Chen, M. Horowitz, K. Kazui, A. Segall (vice chairs) |

# Basic slices and entropy slice implementation

Basic slices and entropy slices has been implemented on top of HM 2.0. A branch, HM-2.0-dev-slices, was used for adding slices on top of HM 2.0.

Table 2	- AHG history

| February 18 | AHG kick-off message sent |
| --- | --- |
| March 1 | First version of slices announced on reflector (revision 596) |
| March 2 | Bug-fix, second version of slices released (rev 600) |
| March 3 | HM-2.0-dev-slices code merged into HM-2.0-dev |
| March 4 | Bug-fix, third version released (rev 609) |

The only change in rev 609 was to set AD_HOC_SLICES_TEST_OUTOFORDER_DECOMPRESS to 0.

Table 3 shows the BDR performance of the HM-2.0-dev-slices code (revision 611) using 1500 byte slices. HM 2.0 was used as anchor.

Table 3	- Performance of 1500 byte slices (HM-2.0-dev-slices rev 609) compared to HM 2.0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 5.0 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.5 |
| Class C | 5.3 | 4.6 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.4 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.6 | 6.4 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |
| Enc Time[%] | 115% |  |  | 122% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.6 | 9.6 | 8.9 | 5.4 | 5.9 | 6.7 |
| Class B | 4.7 | 5.8 | 5.3 | 4.8 | 5.1 | 4.8 |
| Class C | 4.1 | 4.9 | 5.0 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 2.9 | 2.6 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.3 | 5.8 | 5.5 | 4.3 | 4.4 | 4.5 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.5 | 7.0 | 5.5 | 3.7 | 3.7 | 3.4 |
| Class C | 2.8 | 4.9 | 4.9 | 2.8 | 2.6 | 2.4 |
| Class D | 1.7 | 2.6 | 2.6 | 1.8 | 1.7 | 2.2 |
| Class E | 1.5 | 1.9 | 1.2 | 1.3 | 3.1 | 3.3 |
| All | 2.5 | 4.4 | 3.8 | 2.6 | 2.8 | 2.8 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The complexity numbers above come from cluster simulations using different machines and are therefore not reliable. The encoder complexity for the all-Intra configuration is high due to the fact that the encoder re-encodes LCUs rather frequently for Intra pictures at low QPs to reach the slice byte target.

There are some known issues with the current slice implementation. These are:

- The slice code always puts ALF syntax for the entire picture in the first slice

- The slice header is not accounted for when the code is configured for byte size limited slices

- AD_HOC_SLICES_TEST_OUTOFORDER_DECOMPRESS does not work with bit-stream buffering

- Slice header syntax elements differ between source code and Working Draft

# List of AHG related input documents

The following contributions related to slices were submitted to the current JCT-VC meeting:

JCTVC-E024 [Y.-W. Huang (MediaTek), I.-K. Kim (Samsung)] "CE4: Summary report of core experiment on slice boundary processing and fine granularity"

JCTVC-E042 [C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)] "Unified End-Of-Slice Detection for LCEC and CABAC"

JCTVC-E043 [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)] "CE4 Subset1: Leaf-CU-Aligned Slices"

JCTVC-E044 [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)] "CE4 Subset2: Slice Boundary Filter"

JCTVC-E045 [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)] "CE4 Subset3: Slice Common Information Sharing"

JCTVC-E063 [K.Kazui (Fujitsu)] "CE4 Subset3: Cross-verification of MediaTek's slice common information sharing by Fujitsu"

JCTVC-E087 [M. Shima (Canon)] "CE4-subset1: Cross-check result of Huawei's fine granularity slice partition"

JCTVC-E089 [M. Shima (Canon)] "CE4-subset2: Cross-check result of Huawei's intra coding improvements for slice boundary blocks"

JCTVC-E196 [F. Henry, D. Pateux (Orange Labs)] “Wavefront Parallel Processing”

JCTVC-E222 [E. Francois, C. Gisquet, G. Laroche, P. Onno, N. Ouedraogo] "Slices modifications for HEVC"

JCTVC-E260 [R. Sjoberg, P. Wennersten (Ericsson),] "CE4 Subset1: Ericsson fine granularity slices"

JCTVC-E272 [I.-K. Kim (Samsung),] "CE4: Cross-verification on slice-boundary filtering - Subset2 case1 and case2"

JCTVC-E281 [S. Wenger , J. Boyce, (Vidyo)] "Slice parameter set"

JCTVC-E283 [Blocks Y. Lin (HiSilicon), C. Lai (HiSilicon), J. Zheng (HiSilicon), L. Liu (HiSilicon)] "CE4 Subset2: Report of Intra Coding Improvements for Slice Boundary"

JCTVC-E298 [Q. Shen (Huawei), Q. Xie (Huawei), H. Yu (Huawei)] "CE4 Subset1: Report on fine granularity slice partition"

JCTVC-E301 [Q. Shen (Huawei), Q. Xie (Huawei), H. Yu (Huawei)] "CE4 Subset1: Cross-check report for Ericsson's Fine granularity slices"

JCTVC-E394 [P. Chen] "CE4: Cross-verification of MediaTek proposal on slice granularity (JCTVC-E043)"

JCTVC-E401 [P. Chen, Y. Chen, X. Wang, M. Karczewicz] "Slice Design with Fine Granularity"

JCTVC-E408 [A. Fuldseth (Cisco), M. Horowitz, S. Xu (eBrisk Video), M. Zhou (TI)] "Tiles"

JCTVC-E409 [K. Misra, A. Segall (Sharp)], “New results for periodic inits for wavefront coding functionality”

JCTVC-E412 [K. Misra, A. Segall (Sharp)] "Tiles for Parallel Decoding"

JCTVC-E414 [K. Misra, A. Segall] "Cross verification of Tiles (JCTVC-E408)"

JCTVC-E415 [K. Misra, A. Segall (Sharp)], “Cross verification of Wavefront Parallel Processing (JCTVC-E196)”

JCTVC-E421 [M. Coban] "Crosscheck of Cisco's Tiles Proposal (JCTVC-E408)"

JCTVC-E423 [F. Henry, D. Pateux (Orange Labs)] “Cross verification of new results for periodic inits for wavefront coding functionality (JCTVC-E409)”

# Recommendation

The Slice support development and characterization AHG recommends to review all slice related input document.