| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E007-r1 WG11 Number: m20084 |
| --- | --- |

| Title: | JCT-VC AHG report: Coding block structures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krit Panusopone (Motorola Mobility) Woo-Jin Han (Samsung Electronics) T. K. Tan (NTT Docomo) Thomas Wiegand (Fraunhofer HHI) | Email: | krit@motorola.com wjhan.han@samsung.com tktan.p@rd.nttdocomo.co.jp thomas.wiegand@hhi.fraunhofer.de |
| Source: | Coding block structures AhG |  |  |

_____________________________

# Abstract

The following report summarizes the Coding block structures Ad hoc activities between the 4th JCT-VC meeting in Daegu, Korea (20 to 28 January, 2011) and the 5th JCT-VC meeting in Geneva, Switzerland (16 to 23 March, 2011).

- Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Coding block structures (jct-vc@lists.rwth-aachen.de) - Study techniques of HM relating to coding block structure - Characterize the trade-offs involved in coding block structure issues, including complexity, redundancy and compression performance aspects - Identify opportunities for harmonization and simplification of coding block structure | K. Panusopone, W.-J. Han, T. K. Tan, T. Wiegand [chairs] | N |

- Status

Several changes were made to coding block structure at the Daegu meeting including removal of NxN PU partition type except for the smallest CU size and replacing 2NxN and Nx2N PU partition type with “partial merge” mode.

There were some activities relating to Coding block structure AhG occurred between the Daegu meeting and the Geneva meeting. CE-2 [1] studies several flexible motion partitioning methods which will impact PU partitioning. Several companies conducted independent investigations of RQT. One such study is a joint effort by Motorola Mobility, NEC, I2R, Texas Instrument to evaluate RQT in HM at various maximum depths and implicit TU signaling approach. Their findings were reported to the group thru JCTVC reflector and its coding performance results are shown in Table 1.

Table 1. Coding performance of RQT at various maximum depth and implicit TU for Luma component

| Column1 | Implicit | D=1 | D=2 | D=3 | D=4 |
| --- | --- | --- | --- | --- | --- |
| INHE | 0.4 | 0.4 | 0.1 | 0.0 | 0.0 |
| INLC | 0.0 | 0.0 | -0.1 | -0.4 | -0.3 |
| RAHE | 1.0 | 1.7 | 0.3 | 0.0 | -0.1 |
| RALC | 0.8 | 1.9 | -0.1 | -0.2 | -0.2 |
| LDHE | 1.1 | 2.3 | 0.3 | 0.0 | -0.1 |
| LDLC | 1.1 | 3.0 | -0.1 | -0.2 | 0.0 |

- List of related input documents

JCTVC-E081 [S. Liu , X. Zhang , S. Lei (MediaTek)] Syntax and Structure for Coding and Utilizing Partition Types

JCTVC-E083 [S. Liu , Z. Zhou , S. Lei (MediaTek)] Evaluations and Suggestions about TU Representation

JCTVC-E090 [X. Li , J. An , X. Guo , S. Lei (MediaTek)] Adaptive CU Depth Range

JCTVC-E104 [Y. H. Tan , C. Yeo , H. L. Tan , Z. Li (I2R)] RQT depth selection

JCTVC-E364 [K. Panusopone , X. Fang , L. Wang (Motorola Mobility)] Proposal on RQT root location

JCTVC-E365 [K. Panusopone (Motorola Mobility), K. Chono (NEC) , Y.H. Tan (I2R) , M. Zhou (TI)] Evaluation of RQT in HM and related TU representation

JCTVC-E377 [L. Guo , X. Wang , P. Chen , M. Karczewicz (Qualcomm)] Limiting Chroma Transform Depth in Residue Quad Tree (RQT)

JCTVC-E425 [Mischa Siekmann , Heiko Schwarz , Benjamin Bross , Detlev Marpe , Thomas Wiegand] Fast encoder control for RQT

- Recommendations

The recommendations of the Coding block structures AhG are to:

- Study TU tree partitioning method including RQT and alternative TU representation.

- Encourage more people to volunteer to contribute on improving coding efficiency and simplification of Coding block structures.

# References

[1] X Zheng , P. Bordes , P. Chen , I.-K Kim, “Core Experiment 2: Flexible Motion Partitioning,” JCTVC-C502r1, Guangzhou, CN, Oct. 2010