| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E008-r1 |
| --- | --- |

| Title: | JCT-VC AHG report: Reference pictures memory compression |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono (Chair) NEC Corporation Takeshi Chujoh Toshiba Corporation ChongSoon Lim Panasonic Corporation Ali Tabatabai Sony Corporation Minhua Zhou Texas Instruments Inc., USA | Email: | chono@ct.jp.nec.com takeshi.chujoh@toshiba.co.jp ChongSoon.Lim@sg.panasonic.com Ali.Tabatabai@am.sony.com zhou@ti.com |
| Source: | [Reference pictures memory compression AHG] |  |  |

_____________________________

# Abstract

This document summarizes the AHG activities between the 4th Meeting: Daegu, KR, 20-28 January, 2011 and the current 5th Meeting: Geneva, CH, 16-23 March 2011.

# Mandate

| AHG Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Reference pictures memory compression (jct-vc@lists.rwth-aachen.de) - Study motion compensation memory access bandwidth of HM design - Study clipping and rounding schemes of extended bit depth values for reference picture storage - Study reference picture memory compression schemes in HM design - Study data format alignment between reference picture memory compression and display processing - Study the visual quality impact of reference picture memory compression - Report on conclusions reached | K. Chono (chair), T. Chujoh, C. S. Lim, A. Tabatabai, M. Zhou (vice chairs) | N |

# Activities

Related to Mandate 1:

NEC integrated the run-time decoder-side MC memory access calculation module into HM2.0 branch:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-2.0-ahg-memory

Decoder-side MC memory access bandwidths of HM2.0 Anchor streams were measured and uploaded to the SVN site.

The following summary was reported to JCTVC reflector:

- In high efficient setting, decoder-MC memory access measures of HM2.0 are reduced by 21.8% and 24.1% for RA and LD cases compared to those of TMuC0.9;

- In low complexity setting, decoder-MC memory access measures of HM2.0 are increased by 25.5% and 24.1% for RA and LD cases compared to those of TMuC0.9.

Related to Mandate 2:

Toshiba integrated JCTVC-D152 fixed rounding using Bit Depth Increase (BDI) distortion metric into HM 2.1. The results of JCTVC-D152 and BDI off are summarized in Table 1 and Table 2. It was reported that even with the BDI distortion metric, Class E results with fixed rounding are inferior to those of BDI off. The results were cross-check by NEC in JCTVC-E187.

Table 1:	HE random access results of BDI off and JCTVC-D152 fixed-rounding

Table 2:	HE low delay results of BDI off and JCTVC-D152 fixed-rounding

Related to Mandate 3:

JCTVC-E133 proposes a memory compression scheme from 10-bit to 8-bit based on  a distortion definition. JCT-E133 also describes an adaptive scaling method with fixed length format as an example of the distortion-definition-based memory compression scheme.

JCTVC-E432 proposes a memory compression scheme from 10-bit to 8-bit based on the unified scaling using adaptive offset described in JCTVC-D035. JCTVC-E432 also proposes a file format for their memory compression scheme.

The results of JCTVC-E133 and JCTVC-E432 are summarized in Table 3andTable 4. Both memory compression schemes outperform BDI off and JCTVC-D154 fixed rounding in terms of BD-rates.

Table 3:	HE random access results of JCTVC-E133 and JCTVC-E432.

Table 4:	HE low delay results of JCTVC-E133 and JCTVC-E432

Related to Mandate 4:

There was no discussion among reflector. There is no contribution document.

Related to Mandate 5:

There was no discussion among reflector. There is no contribution document.

# Recommendations

The AHG recommends reviewing related input documents.

# List of related input documents

JCTVC-E133	Adaptive scaling for reference pictures memory compression

JCTVC-E187	Cross-verification report on memory bandwidth reduction MC filter (JCTVC-E129)

JCTVC-E432	Unified scaling for 10-bit to 8-bit reference frame compression

JCTVC-E463	Cross Verification for Unified scaling for 10-bit to 8-bit reference frame compression