| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E010 |
| --- | --- |

| Title: | JCT-VC AHG report: Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., Kazushi Sato Sony Corp. Gaëlle Martin-Cocher RIM | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1861 madhukar@ti.com Kazushi.Sato@jp.sony.com gmartincocher@rim.com |
| Source: | JCTVC Quantization AhG |  |  |

_____________________________

# Quantization AhG overview

The mandate of this AHG is as follows:

| Quantization (jct-vc@lists.rwth-aachen.de) - Study quantization issues in HM design, including sub-LCU dQP, Quantization matrix, etc. - Study trade offs and characteristics of quantization design, including coding efficiency and complexity - Study the impact on subjective quality improvement and coordinate preparations for subjective viewing test at Geneva, if necessary - Identify issues relating to tools tested only with RDOQ enabled. - Study new quantization schemes such as adaptive quantization level (AQL) with RDOQ on and RDOQ off - Study adequacy of current mapping of QP to quantizer step-size for rate control at different coding levels (LCU, slice, frame, etc.) | M. Budagavi (chair), K. Sato, G. Martin-Cocher (vice-chairs) | N |
| --- | --- | --- |

# Quantization related activity and emails

There were a few emails related to this AhG on the JCTVC reflector. A comment was made in the emails that quantization issues be considered in conjunction with transform design. CE10 on spatial transforms is already doing this.

# Quantization related contributions

There are several contributions to the Geneva meeting that are related to the mandates of the Ad Hoc Group. They can be broadly categorized as follows:

- Sub-LCU level delta QP

- Quantization matrices

- Others

## Sub-LCU level delta QP

There are nine contribution related to this topic. In the current HEVC design, QP is signaled at LCU level.  The topics covered in the contributions include:

- Need for delta QP support at sub-LCU level

- Syntax changes needed to support sub-LCU level delta QP,

- Prediction methods for calculation of delta QP using neighboring CU information,

- Simulation results of  sub-LCU level delta QP implementation.

| JCTVC-E051 | m19559 | Quantization: Sub-LCU Delta QP | T.-D. Chuang, C.-Y. Chen, Y.-L. Chang, Y.-W. Huang, S. Lei (MediaTek) |
| --- | --- | --- | --- |
| JCTVC-E198 | m19716 | Sub-LCU level delta QP signaling | M. Kobayashi, M. Shima (Canon) |
| JCTVC-E202 | m19720 | Support for Sub-LCU-Level QP in HEVC | Masato Shima (Canon), Sung-Chang Lim (ETRI), Yu-Wen Huang (MediaTek), Chong Soon Lim (Panasonic), Kazushi Sato (Sony), Madhukar Budagavi (TI), |
| JCTVC-E215 | m19735 | Prediction-based QP derivation | Hirofumi Aoki, Keiichi Chono, Yuzo Senda (NEC) |
| JCTVC-E217 | m19737 | Improved dQP Calculation Method | C. Pang, O. C. Au, F. Zou, J. Dai, X. Wen(HKUST), |
| JCTVC-E220 | m19741 | Preliminary Implementation on Sub-LCU-Level DeltaQP | Kazushi Sato, Jun Xu (Sony) |
| JCTVC-E238 | m19761 | Verification results of MediaTek's contribution on sub-LCU delta QP (JCTVC-E051) | M. Budagavi (TI), |
| JCTVC-E391 | m19927 | CU-Level QP Prediction | Muhammed Coban, Wei-Jung Chien, Marta Karczewicz (Qualcomm) |
| JCTVC-E436 | m19983 | Sub-LCU QP representation | C. Pang, O. C. Au, J. Dai, F. Zou, X. Wen(HKUST), |

## Quantization matrices

There are three contributions related to quantization matrices. The topics covered in the contributions include:

- Need for quantization matrix compression in HEVC

- Syntax changes needed to support quantization matrices,

- Quantization matrix compression methods,

- Simulation results of  quantization matrix implementation in HM software.

| JCTVC-E056 | m19564 | Necessity of Quantization Matrices Compression in HEVC | Kazushi Sato, Hironari Sakurai(Sony) |
| --- | --- | --- | --- |
| JCTVC-E073 | m19583 | Quantization matrix for HEVC | Junichi Tanaka, Yoshitaka Morigami, Teruhiko Suzuki (Sony) |
| JCTVC-E435 | m19982 | QuYK | Gergely Korodi, Dake He (RIM) |

## Others

| JCTVC-E091 | m19606 | Adaptive De-Quantization Offset | X. Li, X. Guo, S. Lei (MediaTek) |
| --- | --- | --- | --- |

# Recommendations

The AhG recommends to review all relevant input contributions.