| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E016 |
| --- | --- |

| Title: | JCT-VC AHG report: High-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Huawei Jill Boyce Vidyo | Email: yekui.wang@huawei.com Email: jill@vidyo.com |  |
|  | Ying Chen Qualcomm Kimihiko Kazui Fujitsu Thomas Schierl Fraunhofer HHI Rickard Sjöberg Ericsson T. K. Tan NTT DOCOMO | Email: cheny@qualcomm.com Email: kazui.kimihiko@jp.fujitsu.com Email: thomas.schierl@hhi.fraunhofer.de Email: rickard.sjoberg@ericsson.com Email: ttk@pacific.net.sg |  |
| Source: | High-level syntax AHG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the high-level syntax Ad Hoc Group between the 4th JCT-VC meeting held in Daegu in January 2011 and the current meeting in Geneva.

- Mandates

- Study NAL unit header, sequence parameter set, picture parameter set, and slice header syntax designs

- Study reuse of AVC SEI messages

- Study possible improvements to the reference picture list construction process

- Study possible simplifications and improvements to reference picture marking process (e.g., the need of the processes for generating and handling of "non-existing" pictures)

- Study the hypothetical reference decoder behaviour

- Assist in software development and text drafting for the high-level syntax in the HEVC design

- Reflector activity

The main topic of the discussion on the e-mail reflector related to whether to change nal_ref_idc in NAL unit header from two bits to one bit. There was one opinion expressed that two bits should be kept as they are, at least for now, unless the bit is clearly needed. Another opinion expressed was that due to the inclusion of temporal_id, the only use of the two bits for indication of transport priority in RTP payload format would disappear and therefore the bit would be purely wasted. We can keep this issue open at the moment unless there are new input contributions on this subject.

# Related contributions

The following contributions related to high-level syntax have been submitted to the current JCT-VC meeting (excluding those that seem to be closely related to other AHGs):

| JCTVC-E052 | Removal of duplicated combinations of reference picture indices for forward Bi-prediction | T.-D. Chuang , J.-L. Lin , Y.-W. Huang , S. Lei (MediaTek) |
| --- | --- | --- |
| JCTVC-E053 | Unified syntax of reference picture list reordering | C.-W. Hsu , J.-L. Lin , Y.-W. Huang , S. Lei (MediaTek) |
| JCTVC-E061 | Requirements of very low delay applications | K.Kazui , J.Koyama , S.Shimada , A.Nakagawa (Fujitsu) , |
| JCTVC-E280 | Picture orientation information | S. Wenger , J. Boyce , D. Hong , (Vidyo) |
| JCTVC-E281 | Slice parameter set | S. Wenger , J. Boyce , (Vidyo) |
| JCTVC-E309 | Parameter set updates using conditional replacement | S. Wenger , (Vidyo) , |
| JCTVC-E339 | On num_reorder_frames and max_dec_frame_buffering | R. Sjoberg (Ericsson) , |
| JCTVC-E345 | NAL unit header and sub-bitstream extraction | Y.-K. Wang (Huawei) |
| JCTVC-E346 | Reuse of AVC SEI messages | Y.-K. Wang (Huawei) |
| JCTVC-E348 | On reference picture list construction for uni-predicted partitions | Y.-K. Wang , Z. Wu (Huawei) , |
| JCTVC-E361 | Supporting one list only coding for HM | J. Lou , K. Minoo , D. Baylon , K. Panusopone , L. Wang (Motorola Mobility) |
| JCTVC-E375 | Design consideration for compability with AVC NAL units | ChongSoon Lim , Viktor Wahadaniah (Panasonic) , , |
| JCTVC-E399 | Sliding window for temporal scalability | Ying Chen , Marta Karczewicz , , |
| JCTVC-E400 | Comments on clean decoding refresh pictures | Ying Chen , Coban Muhammed , Peisong Chen , Marta Karczewicz , |
| JCTVC-E408 | Tiles | Arild Fuldseth , Michael Horowitz , Minhua Zhou , , |
| JCTVC-E412 | Tiles | Kiran Misra , Andrew Segall |
| JCTVC-E429 | On profile signaling | David Singer (Apple) |

# Conclusions

The high-level syntax AHG recommends:

- To review all contributions related to high-level syntax

- To coordinate with the scalable coding investigation AHG and the slice support development and characterization AHG contributions related to all groups