| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E022 |
| --- | --- |

| Title: | CE2: Summary of Core Experiment 2 on Flexible Motion Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (coordinator), Philippe Bordes, Peisong Chen, Il-Koo Kim (vice-coordinators) | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com philippe.bordes@technicolor.com peisongc@qualcomm.com ilkoo.kim@samsung.com |
| Source: | Huawei & HiSilicon Technologies, Technicolor, Samsung, Qualcomm |  |  |

_____________________________

# Abstract

This document summarizes the activity of CE2 related to Flexible Motion Partitioning. The description of the experiment can be found in JCTVC-D602r1. In this CE, three tools for motion partitioning have been studied: Asymmetric Motion Partitioning with Overlapped Block Motion Compensation (OBMC), Non-rectangular Motion Partitioning (NRMP) with OBMC and unified solution (AMP+NRMP+OBMC). An additional tool related to CE2 has also been evaluated: Asymmetric Motion Partitioning + OBMC + Non-Square TU. 8 companies have been involved as proponent or cross-checker.

# CE Documents

The following 8 documents are submitted for CE2:

- Summary document (this document)

  - JCTVC-E022[CE coordinators] CE2: Summary of Core Experiment 2 on Flexible Motion Partitioning

- Proposals and verification documents

Proposal 1: Asymmetric Motion Partitioning with OBMC  (original proposal: JCTVC-D367, JCTVC-D233)

  - Proposal :	JCTVC-E316 [Samsung and Qualcomm] CE2: Test results of asymmetric motion partition (AMP) with overlapped block motion compensation (OBMC)

  - Cross-check: 	JCTVC-E259 [HHI] CE2: Cross-check report of test 2a and 2b

Proposal 2: Non-rectangular Motion Partitioning (NRMP)  (original proposal: JCTVC- D230, JCTVC-D297)

  - Proposal :	JCTVC-E373 [Huawei & HiSilicon, Technicolor, Qualcomm] CE2: Test results of non-rectangular motion partitioning (NRMP) with overlapped block motion compensation (OBMC)

  - Cross-check: 	JCTVC-E252 [INRIA] CE2: Cross-check report of Non Rectangular Motion Partitioning (NMRP) + Overlapped Block Motion Compensation (OBMC), proposal JCTVC-EXXX

Proposal 3: unified solution (AMP+NRMP+OBMC) (original proposal: JCTVC- D602r1)

  - Proposal :	JCTVC-E374 [Huawei & HiSilicon, Qualcomm, Samsung, Technicolor] Unified solution of flexible motion partitioning

  - Cross-check: 	JCTVC- E259 [HHI] CE2: Cross-check report of test 2a and 2b

Additional proposal related to CE2: Asymmetric Motion Partitioning with OBMC and Non-Square TU

  - Proposal :	JCTVC-E376 [Tsinghua, Huawei & HiSilicon, Microsoft, USTC]  Asymmetric Motion Partition with OBMC and Non-Square TU

  - Cross-check: 	JCTVC-E349 [Qualcomm] CE2: Cross-verification of Asymmetric Motion Partitioning with OBMC and Non-Square TU from Tsinghua, Huawei &HiSilicon, Microsoft and USTC (JCTVC-EXXX)

# Proposed Tools

Two types of motion partitioning have been studied:

- Rectangular Motion Partitioning: the tool, known as Asymmetric Motion Partitioning (AMP) and proposed by Samsung, consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes.

- Non-rectangular Motion Partitioning: a square block is possibly divided into two non-rectangular partitions; At this category, Technicolor and Huawei & HiSilicon Technologies propose three different non-rectangular partitioning method: 2 diagonal-only partitioning, 6 diagonal-only partitioning, 6 not-only diagonal partitioning.

Then encoder speedup algorithm is applied to both AMP and NRMP. It can reduce encoding complexity and keep most of coding performance.

In addition, OBMC is proposed by Qualcomm as a solution to smooth the boundary between AMP PUs or between two non-rectangular partitions. OBMC was already used in the proposal JCTVC-D367 and JCTVC-D368.

# Summary of Results

Test conditions as specified in JCTVC-C600 were used. Only High Efficiency case was considered.

## Asymmetric Motion Partitioning (AMP) with OBMC

Proposal :	JCTVC-E316 [Samsung and Qualcomm]

AMP with OBMC (subtest 2a) – Reference: HM2.0

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.7 | -1.0 | -1.3 |  |  |  |
| Class B | -0.8 | -1.0 | -1.1 | -1.0 | -1.3 | -1.6 |
| Class C | -1.0 | -1.4 | -1.4 | -1.2 | -1.5 | -1.5 |
| Class D | -0.9 | -1.2 | -1.1 | -1.4 | -1.2 | -1.7 |
| Class E |  |  |  | -2.4 | -2.6 | -1.6 |
| All | -0.8 | -1.2 | -1.2 | -1.4 | -1.6 | -1.6 |
| Enc Time | 114% |  |  | 113% |  |  |
| Dec Time | 105% |  |  | 101% |  |  |

## Non-Rectangular Motion Partitioning (NRMP) with OBMC

Proposal :	JCTVC-E373 [Huawei & HiSilicon, Technicolor, Qualcomm]

NRMP with OBMC (2 diagonal-only partitioning, subtest 1a) – Reference: HM2.0

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.4 | -1.1 |  |  |  |
| Class B | -0.3 | -1.1 | -1.0 | -0.2 | -1.7 | -1.9 |
| Class C | -1,0 | -1,6 | -1,7 | -1.1 | -2.0 | -2.3 |
| Class D | -1,2 | -1,9 | -1,9 | -1.6 | -2.4 | -2.4 |
| Class E |  |  |  | -0.9 | -2.4 | -1.2 |
| All | -0.7 | -1.5 | -1.4 | -0.9 | -2.1 | -2.0 |
| Enc Time | 154% |  |  | 152% |  |  |
| Dec Time |  |  |  | 113% |  |  |

NRMP with OBMC (6 diagonal-only partitioning, subtest 1b) – Reference: HM2.0

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.4 | -1.5 |  |  |  |
| Class B | -0.4 | -1.2 | -1.3 | -0.3 | -2.1 | -2.3 |
| Class C | -1,2 | -1,9 | -2,0 | -1.3 | -2.3 | -2.6 |
| Class D | -1,3 | -2,0 | -2,0 | -1.7 | -2.9 | -2.6 |
| Class E |  |  |  | -1.1 | -2.7 | -1.8 |
| All | -0.8 | -1.6 | -1.7 | -1.1 | -2.5 | -2.4 |
| Enc Time | 173% |  |  | 172% |  |  |
| Dec Time |  |  |  | 118% |  |  |

NRMP with OBMC (6 not-only diagonal partitioning, subtest 1c) – Reference: HM2.0

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.8 | -2.0 |  |  |  |
| Class B | -0,6 | -1,5 | -1,5 | -0.7 | -2.3 | -2.8 |
| Class C | -1,5 | -2,4 | -2,5 | -1.7 | -2.9 | -3.1 |
| Class D | -1,6 | -2,2 | -2,3 | -2.1 | -2.7 | -3.1 |
| Class E |  |  |  | -2.1 | -3.2 | -2.2 |
| All | -1.1 | -2.0 | -2.0 | -1.6 | -2.7 | -2.8 |
| Enc Time | 175% |  |  | 175% |  |  |
| Dec Time |  |  |  | 119% |  |  |

## Unified solution of flexible motion partitioning

Proposal :	JCTVC-E374 [Huawei & HiSilicon, Qualcomm, Samsung, Technicolor]

AMP+NRMP+OBMC (subtest 2b) - Reference: HM2.0

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -1.1 | -2.2 | -2.3 |  |  |  |
| Class B | -1.0 | -1.9 | -1.9 | -1.2 | -2.9 | -3.3 |
| Class C | -1.9 | -2.8 | -2.8 | -2.1 | -3.2 | -3.3 |
| Class D | -1.9 | -2.8 | -2.8 | -2.6 | -3.6 | -3.5 |
| Class E |  |  |  | -3.0 | -4.2 | -3.7 |
| All | -1.4 | -2.4 | -2.4 | -2.1 | -3.4 | -3.4 |
| Enc Time | 174% |  |  | 171% |  |  |
| Dec Time | 117% |  |  | 110% |  |  |

## Additional proposal related to CE2

Proposal :	JCTVC-E376[Tsinghua, Huawei & HiSilicon, Microsoft, USTC]

AMP + OBMC + Non-Square TU –HM2.0

|  | Random access High Efficiency |  |  | Low Delay High Efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.3 | -1.5 |  |  |  |
| Class B | -0.9 | -1.8 | -1.6 | -1.4 | -3.5 | -3.7 |
| Class C | -1.2 | -2.0 | -2.0 | -1.6 | -3.1 | -3.2 |
| Class D | -1.0 | -1.6 | -1.6 | -1.9 | -3.2 | -3.2 |
| Class E |  |  |  | -3.4 | -5.0 | -4.0 |
| All | -1.0 | -1.7 | -1.7 | -2.0 | -3.6 | -3.5 |
| Enc Time | 114% |  |  | 114% |  |  |
| Dec Time | 100% |  |  | 101% |  |  |

|  | Random access Low Complexity |  |  | Low Delay Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.6 | -1.5 |  |  |  |
| Class B | -1.2 | -2.3 | -2.1 | -2.1 | -3.9 | -3.4 |
| Class C | -1.2 | -2.1 | -2.0 | -1.8 | -2.8 | -2.9 |
| Class D | -0.8 | -1.5 | -1.5 | -1.5 | -2.2 | -2.0 |
| Class E |  |  |  | -3.8 | -3.8 | -3.8 |
| All | -1.0 | -1.9 | -1.8 | -2.2 | -3.2 | -3.0 |
| Enc Time | 113% |  |  | 112% |  |  |
| Dec Time | 102% |  |  | 98% |  |  |

## Results analysis

The following table summarize the average BD-rate gain for Y, U, V component, encoding time and decoding time for the different proposals.

randomaccess

| subtest | Y-BD-rate | U-BD-rate | V-BD-rate | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| AMP+OBMC | -0.8% | -1.2% | -1.2% | 114% | 105% |
| NRMP+OBMC (1a) | -0.7% | -1.5% | -1.4% | 154% |  |
| NRMP+OBMC (1b) | -0.8% | -1.6^ | -1.7% | 173% |  |
| NRMP+OBMC (1c) | -1.1% | -2.0% | -2.0% | 175% |  |
| AMP+NRMP+OBMC (2b) | -1.4% | -2.4% | -2.4% | 174% | 117% |

lowdelay

| subtest | Y-BD-rate | U-BD-rate | V-BD-rate | Enc Time [%] | Dec Time [%] |
| --- | --- | --- | --- | --- | --- |
| AMP+OBMC | -1.4% | -1.6% | -1.6% | 113% | 101% |
| NRMP+OBMC (1a) | -0.9% | -2.1% | -2.0% | 152% | 113% |
| NRMP+OBMC (1b) | -1.1% | -2.5% | -2.4% | 172% | 118% |
| NRMP+OBMC (1c) | -1.6% | -2.7% | -2.8% | 175% | 119% |
| AMP+NRMP+OBMC (2b) | -2.1% | -3.4% | -3.4% | 171% | 110% |

# Conclusion

The CE participants suggest considering the different proposals for potential introduction into HM.

# References

- E. Francois, P. Chen, X. Zheng, “Description of Core Experiment 2: Flexible Motion Partitioning,” Doc. JCTVC-C099, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, October 2010

- K.McCann, W.-J.Han, I.-K.Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, Doc. JCTVC-A124, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Dresden, Germany, April 2010.

- P.Chen, R. Panchal W.-J. Chien, M.Karczewicz , (Qualcomm), “Overlapped block motion compensation in TMuC”, Doc. JCTVC-C251 (m18292), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- L. Guo, P.Chen, R. Panchal W.-J. Chien, M.Karczewicz , (Qualcomm), “CE2: Overlapped Block Motion Compensation for Geometry Partition Block”, Doc. JCTVC-D368 (m19143), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Edouard Francois, Philippe Bordes (Technicolor), Liwei Guo, Marta Karczewicz (Qualcomm), “CE2: Simplified Geometry Block Partitioning”, Doc. JCTVC-D230 (m18990), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- X.Zheng (HiSilicon), H.Yu (Huawei), “TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding”, Doc. JCTVC-C099 (m18122), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- P.Chen, L.Guo, M.Karczewicz , (Qualcomm), “Geometry motion partition”, Doc. JCTVC-C301 (m18516), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- Min-Su Cheon, Il-Koo Kim (Samsung) , L. Guo, P. Chen, M. Karczewicz (Qualcomm), “CE2: Asymmetric motion partition with overlapped block motion compensation”, Doc. JCTVC-D367 (m19142), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-D600 (m19497), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- K.McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

- Yoshinori Suzuki, TK Tan, “CE9: 3.1 PU Merge & Skip tools and proposed improvements”, Doc. JCTVC-D233, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011

# Annex: list of CE participants

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei/HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |
| HHI | Martin Winken, Martin.Winken@hhi.fraunhofer.de Benjamin Bross, benjamin.bross@hhi.fraunhofer.de |
| NTT Docomo | TK Tan, ttk@pacific.net.sg |
| INRIA | laurent.guillo@inria.fr;ronan.boitard@inria.fr |
| Technicolor | Philippe Bordes, philippe.bordes@technicolor.com |