| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 January, 2011 | Document: JCTVC-E023r1 WG11 Number: m19600 |
| --- | --- |

| Title: | CE3: Summary of CE3: interpolation for MC (Luma) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Elena Alshina | Email: | takeshi.chujoh@toshiba.co.jp elena_a.alshina@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This contribution is a summary of CE3: interpolation for MC (Luma).  Ten companies have been registered in CE3 and six proposed tools have been evaluated on the common condition with cross-verification after all.  Detailed results will be reported by proponents.

# Introduction

In this core experiment, interpolation filters for motion compensation of luminance pixels have been discussed.  There are following two purposes;

- Improvement of coding efficiency by interpolation filter

- Reduction of complexity and reference frame memory access bandwidth generally

# Document list

- Summary document

  - JCTVC-E023 [CE coordinators] Summary of CE3 : interpolation for MC (Luma)

- Proponent's documents

  - JCTVC-E078 [S.Matsuo, Y.Bandoh, T.Ito, S.Takamura, H.Jozawa (NTT)] CE3: Region-based adaptive interpolation filter

  - JCTVC-E128 [K.Kondo, T.Suzuki (Sony)] CE3: Results on Bi/Single MC interpolation filter

  - JCTVC-E134 [T.Chujoh, K.Kanou, T.Yamakage (Toshiba)] CE3: Non-uniform tap length filtering for bidirectional prediction

  - JCTVC-E188 [T.Yoshino, S.Naito] CE3 : Switching interpolation filter scheme

  - JCTVC-E284 [F.Kossentini, N.Mahdi (eBrisk Video)] An Adaptive Interpolation Filtering Technique

  - JCTVC-E358 [J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang (Motorola Mobility)] CE3: Report on Motorola Mobility's interpolation filter for HEVC

- Cross-checking documents

  - JCTVC-E195 [T.Yoshino, S.Naito] CE3:Cross-check for NTT's proposal on Region-Based Adaptive Interpolation Filter (JCTVC-E078)

  - JCTVC-E200 [K.Kondo, T.Suzuki (Sony)] CE3: Cross verification of non-uniform tap length filtering for bidirectional prediction (JCTVC-E134)

  - JCTVC-E205 [S.Matsuo, Y.Bandoh, T.Ito, S.Takamura, H.Jozawa] CE3: Cross-check for KDDI's proposal on Switching interpolation filter scheme (JCTVC-E188)

  - JCTVC-E244 [K.Ugur (Nokia)] CE3: Cross-check of Sony's proposal E128

  - JCTVC-E248 [S.Esenlik, T.Wedi (Panasonic)] CE3: Cross-check Results for the Proposal of eBrisk (JCTVC-E284)

  - JCTVC-E249 [H.Lakshman (Fraunhofer HHI)] CE3: Cross-check of Motorola Mobility's proposal JCTVC-E249

  - JCTVC-E347 [L.Guo, I.-S.Chong, M.Karczewicz (Qualcomm)] CE3:Cross-verification of SONY's Luma Interpolation Contribution (JCTVC-E128 and case 0 of JCTV-E129)

  - JCTVC-E351 [G.Van der Auwera, I.S.Chong, M.Karczewicz (Qualcomm)] CE3: Cross-Verification of Toshiba’s Contribution (JCTVC-E134) by Qualcomm

  - JCTVC-E355 [J.Lou, K.Panusopone, L.Wang (Motorola Mobility)] CE3: Cross-check report from Motorola Mobility for Toshiba's interpolation filter (JCTVC-E134)

  - JCTVC-E388 [E.Alshina, J.Chen, W.-J. Han (Samsung)] CE3: Cross-verification for Motorola Mobility’s test by Samsung

- Related documents

  - JCTVC-E129 [K.Kondo, T.Suzuki (Sony)] Memory bandwidth reduction MC filter

  - JCTVC-E189 [K.Chono, H.Aoki, Y.Senda] Cross-verification report on memory bandwidth reduction MC filter (JCTVC-E129)

  - JCTVC-E241 [K.Ugur, J.Lainema (Nokia)] Low complexity bi-predictive interpolation with 6-tap DCT-IF filter

  - JCTVC-E359 [J.Lou, K.Minoo, D.Baylon, K.Panusopone, L.Wang (Motorola Mobility)] Motorola Mobility's adaptive interpolation filter

  - JCTVC-E378 [E.Alshina, J.Chen, W.-J.Han (Samsung)] CE3: Cross-verification for Sony’s test (JCTVC-E129) by Samsung

  - JCTVC-E426 [E.Alshina, J.Chen, A.Alshin, N.Shlyakhov] Experimental results of 4 taps/5 bits Chroma DCTIF in HM2.0

# Proposed Tools

At all, six tools have been proposed.

Table 1  Proposals

| Proponent | Method |
| --- | --- |
| eBrisk Video | Adaptive interpolation filter |
| KDDI | Switching interpolation filters |
| Motorola Mobility | New filter coefficients of DCT-IF 8-tap |
| NTT | Region based adaptive interpolation filter |
| Sony | Bi/Single filters switching in FIF, 6-tap for single prediction and 8-tap bidirectional prediction. |
| TOSHIBA | Non-uniform tap length filtering, 8/6-tap for .bidirectional prediction |

# Cross-verification

Proponents provided their source code exchange to all participants.  Cross-checkers run the software independently and compared their results with proponent’s results.

Table 2  Cross checkers

| Proponents | Cross-checkers |
| --- | --- |
| eBrisk Video | Panasonic |
| KDDI | NTT, eBrisk Video |
| Motorola Mobility | Samsung, HHI |
| NTT | KDDI |
| Sony | Nokia, Qualcomm |
| TOSHIBA | Motorola Mobility, Sony |

# Experimental results

All experiments for all sequences based on the common conditions, JCTVC-D603 [1] have been conducted.  On this common condition, the following evaluation criteria are defined.

- Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate

- Complexity (encoding/decoding times, and use a common Excel sheet that counts number of operations instruction cycles and memory bandwidth compared to another)

As a discussion of participants, a common Excel sheet has been provided.

## Coding efficiency

Table 3 Summary of coding efficiency

| Proponent |  |  | NTT | SONY | TOSHIBA |  | KDDI | eBrisk Video | Motorola Mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contribution number |  |  | E078 | E128 | E134 |  | E188 | E284 | E358 |
| HE | RA | Y BD-rate (%) | -0.03 | 0.01 | 0.04 | 0.10 | 0.15 | -0.20 | 0.08 |
|  | LD |  | -0.10 | 0.05 | 0.30 | 0.16 | -0.09 | -0.10 | -0.08 |
| LC | RA |  | -0.42 | -0.57 | 0.30 | 0.01 | 0.04 | -1.10 | 0.29 |
|  | LD |  | -1.13 | -0.57 | 1.32 | -0.12 | -0.06 | -1.40 | -0.26 |
| All |  |  | -0.42 | -0.27 | 0.49 | 0.04 | 0.01 | -0.70 | 0.01 |

## Complexity analyses

Table 4 Summary of complexity analyses

| Proponent |  |  | NTT | SONY | TOSHIBA |  | KDDI | eBrisk Video | Motorola Mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contribution number |  |  | E078 | E128 | E134 |  | E188 | E284 | E358 |
| Proponent |  | Encoding Time (%) | 140% | 96% | 103% | 105% | 101% | 125% | 97% |
|  |  | Decoding Time (%) | 98% | 97% | 99% | 102% |  | 100% | 98% |
| Cross-checker |  | Encoding Time (%) |  | 72% | 104% | 103% |  |  | 99% |
|  |  | Decoding Time (%) |  | 106% | 99% | 99% |  |  | 102% |
|  |  | Encoding Time (%) |  | 96% | 101% | 102% |  |  | 98% |
|  |  | Decoding Time (%) |  | 100% | 97% | 101% |  |  | 103% |
| Single | Operations | Worst case / pixel | 115% | 69% | 100% | 100% | 100% | 115% | 88% |
|  |  | Average of operations / pixel | 114% | 72% | 100% | 100% | 100% | 114% | 93% |
|  | Bandwidth | Worst case / pixel | 100% | 67% | 100% | 100% | 100% | 100% | 100% |
|  |  | Average of bytes / pixel | 100% | 79% | 100% | 100% | 100% | 100% | 93% |
| Bi | Operations | Worst case / pixel | 115% | 100% | 92% | 88% | 100% | 115% | 88% |
|  |  | Average of operations / pixel | 114% | 103% | 94% | 91% | 100% | 114% | 93% |
|  | Bandwidth | Worst case / pixel | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | Average of bytes / pixel | 100% | 100% | 88% | 88% | 100% | 100% | 93% |
| Total | Operations | Worst case / pixel | 115% | 100% | 92% | 88% | 100% | 115% | 88% |
|  |  | Average of operations / pixel | 114% | 93% | 96% | 94% | 100% | 114% | 93% |
|  | Bandwidth | Worst case / pixel | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | Average of bytes / pixel | 100% | 93% | 92% | 92% | 100% | 100% | 93% |

# Conclusion

In this core experiment, six proposals have been tested and their performances have been verified.

# Reference

[1] T. Chujoh and E. Alshina, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-D603-v3, Daegu, January 2011.