| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E024 |
| --- | --- |

| Title: | CE4: Summary Report of Core Experiment on Slice Boundary Processing and Fine Granularity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Authors: | Yu-Wen Huang1 Il-Koo Kim2 | Emails: | yuwen.huang@mediatek.com ilkoo.kim@samsung.com |
| Source: | MediaTek1, Samsung2 |  |  |

_____________________________

# Abstract

This contribution is a summary of Core Experiment 4 (CE4) on slice boundary processing and fine granularity. In CE4, there are three subsets including Subset1: Slice granularity, Subset2: Slice boundary processing, and Subset3: Slice common information sharing, as described in JCTVC-D604. Three, two, and one proposal were evaluated in the three subsets respectively, and experiments were conducted according to the common test configurations in JCTVC-D600. Detailed results will be provided by each proponent individually.

- Introduction

In CE4, slice-related issues have been studied in the following subsets:

- Subset1: Slice granularity (three proposals)

- Subset2: Slice boundary processing (two proposals)

- Subset3: Slice common information sharing (one proposal)

Details of each proposal can be found in the documents listed in Section Error: Reference source not found.

# Document list

- Summary document

  - JCTVC-E024 [MediaTek/Samsung] CE4: Summary report of core experiment on slice boundary processing and fine granularity

- Proponents’ study documents and corresponding crosscheck reports

Subset1: Slice granularity

  - JCTVC-E043 [MediaTek] CE4 Subset1: Leaf-CU-aligned slices

    - JCTVC-E394 [Qualcomm] CE4: Cross-verification of MediaTek proposal on slice granularity and slice boundary processing (JCTVC-E043, JCTVC-E044)

  - JCTVC-E298 [Huawei] CE4 Subset1: Report on fine granularity slice partition

    - JCTVC-E087 [Canon] CE4-Subset1: Cross-check result of Huawei's fine granularity slice partition

  - JCTVC-E260 [Ericsson] CE4 Subset1: Ericsson fine granularity slices

    - JCTVC-E301 [Huawei] CE4 Subset1: Cross-check report for Ericsson's fine granularity slices

Subset2: Slice boundary processing

  - JCTVC-E044 [MediaTek] CE4 Subset2: Slice boundary filter

    - JCTVC-E272 [Samsung] CE4: Cross-verification on slice boundary filtering - Subset2 case1 and case2

    - JCTVC-E394 [Qualcomm] CE4: Cross-verification of MediaTek proposal on slice granularity and slice boundary processing (JCTVC-E043, JCTVC-E044)

  - JCTVC-E283 [HiSilicon] CE4 Subset2: Report of intra coding improvements for slice boundary blocks

    - JCTVC-E089 [Canon] CE4-Subset2: Cross-check result of Huawei's intra coding improvements for slice boundary blocks

Subset3: Slice common information sharing

  - JCTVC-E045 [MediaTek] CE4 Subset3: Slice common information sharing

    - JCTVC-E063 [Fujitsu] CE4 Subset3: Cross-verification of MediaTek's slice common information sharing by Fujitsu

# Cross-verification assignment

Table 1 shows the cross-verification assignment in CE4. Each Proponent sent software to the crosschecker(s). Cross-checkers ran software independently and compared their results with those provided by proponents.

Table 1  Cross-verification assignment

| Subset | Proponent | Cross-checkers |
| --- | --- | --- |
| 1 | MediaTek | Qualcomm |
|  | Huawei | Canon |
|  | Ericsson | Huawei |
| 2 | MediaTek | Samsung, Qualcomm |
|  | HiSilicon | Canon |
| 3 | MediaTeK | Fujitsu |

# Simulation results

Experiments are described in JCTVC-D604 [1], and the common test configurations in JCTVC-D600 [2] are used for conducting experiments.

## Subset1: Slice granularity

Table 2 shows the BD-rates and bit rate inaccuracy of Subset1. The first number in the cell is BD-rate. The second number in the cell is bit rate inaccuracy but not provided by all experiments. “N/A” means the experiments were not conducted. “Failed” means the decoder could not match the encoder.

Table 2  BD-rates (%) and bit rate inaccuracy ([%]) for 1500-byte slices (anchor: JCTVC-D600)

| Case | Software | Condition | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HM-2.0-dev-slices (rev 609) | LCU=64x64, LCU-aligned | 4.9 [12.3] | 4.3 [8.4] | 2.5 [5.6] | Failed | Failed | Failed |
| 2 | HM-2.0-dev-slices (rev 609) | LCU=32x32, LCU-aligned | 6.6 [3.8] | 7.2 [3.0] | 5.7 [2.7] | Failed | Failed | Failed |
| 3 | HM-2.0-dev-slices (rev 609) | LCU=16x16, LCU-aligned | 10.6 [1.2] | 18.0 [1.5] | 18.4 [1.6] | Failed | Failed | Failed |
| 4 | MediaTek (JCTVC-E043) | LCU=64x64, leaf-CU-aligned, 2-step addressing, EOS flag | 5.3 [2.0] | 4.6 [2.5] | 2.7 [3.0] | 5.8 [0.5] | 4.4 [0.9] | 2.5 [1.2] |
| 5 | MediaTek (JCTVC-E043) | LCU=64x64, leaf-CU-aligned, 2-step addressing, no EOS flag | 5.3 [2.0] | 4.5 [2.5] | 2.6 [3.0] | 5.8 [0.5] | 4.4 [0.9] | 2.5 [1.2] |
| 6 | Huawei (JCTVC-E298) | LCU=64x64, leaf-CU-aligned, 2-step addressing, EOS flag | 5.5 | 4.7 | 2.8 | 5.8 | 4.4 | 2.6 |
| 7 | Huawei (JCTVC-E298) | LCU=64x64, leaf-CU-aligned, 2-step addressing, no EOS flag | N/A | N/A | N/A | N/A | N/A | N/A |
| 8 | Ericsson (JCTVC-E260) | LCU=64x64, leaf-CU-aligned, 1-step addressing, EOS flag | 5.1 | 4.4 | 2.6 | 5.6 | 4.3 | 2.5 |
| 9 | Ericsson (JCTVC-E260) | LCU=64x64, leaf-CU-aligned, 1-step addressing, no EOS flag | 5.1 | 4.5 | 2.6 | 5.6 | 4.3 | 2.5 |

The cases can be divided into two categories: LCU-aligned and leaf-CU-aligned. It is reported that LCU-aligned slices will suffer significant coding efficiency loss or much higher bit rate inaccuracy. When case 1 is compared with case 5, the coding efficiency is essentially the same, but the bit rate inaccuracy for 64x64-LCU-aligned slices is 2.6-10.3% higher. When case 2 is compared with case 5, the coding efficiency for 32x32-LCU-aligned slices is 1.3-2.7% worse, and the bit rate inaccuracy is 0.3% lower to 1.8% higher. When case 3 is compared to case 5, the coding efficiency for 16x16-LCU-aligned slice is 5.3-15.8% worse, and the bit rate inaccuracy is 0.8-1.4% lower. It is suggested by all CE4 participants to adopt leaf-CU-aligned slices in HM.

However, it is still not clear if the 2-step addressing or the 1-step addressing should be used. Due to the very short time between the 4th JCT-VC meeting and the 5th JCT-VC meeting, HM2.1 could not be released, so cases 4-9 were not based on the same slice software. It is suggested by all CE4 participants to establish a BoG for reaching a consensus.

As for the end-of-slice flag, it is suggested by all CE4 participants to remove end-of-slice flags except for the last coding unit. A non-CE proposal, JCTVC-E042, is also related to this and proposed a solution for unification of end-of-slice detection between LCEC and CABAC, which is modified from JCTVC-D383.

## Subset2: Slice boundary processing

In JCTVC-E044, a slice boundary filter (SBF) is applied on slice boundaries after decoding multiple slices of a picture when slice-independent deblocking filter (DF) and adaptive loop filter (ALF) are used. For 1500-byte slices, the bit rate saving of the SBF-on case is reportedly 0.1-0.3% over the SBF-off. For two slices per picture, the bit rate saving of the SBF-on case is reportedly 0-0.1% over the SBF-off case. Table 3 summarizes the BD-rate performance. The subjective quality differences between the SBF-on and the SBF-off cases reportedly can be easily observed.

Table 3  BD-rates (%) of the slice boundary filter (SBF) proposed by MediaTek in JCTVC-E044 (anchor: JCTVC-D600)

| Case | Condition | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1500 bytes per slice, SBF off | 5.6 | 4.7 | 2.7 | 5.8 | 4.5 | 2.7 |
| 2 | 1500 bytes per slice, SBF on | 5.3 | 4.6 | 2.6 | 5.5 | 4.3 | 2.5 |
| 3 | 2 slices per picture, SBF off | 0.6 | 1.3 | 1.6 | 0.6 | 1.2 | 1.4 |
| 4 | 2 slices per picture, BF on | 0.5 | 1.3 | 1.6 | 0.6 | 1.1 | 1.3 |

In JCTVC-E283, two methods of intra prediction for slice boundary blocks are applied. Firstly, the unavailable reference pixels for boundary block are padded with the closest pixels to the upper-left corner from its available neighbor block, instead of with a fixed DC value. Secondly, for those positive directions in UDI, the unavailable reference pixels are generated by projecting the available reference pixels. Then the generated reference pixels further are used for UDI intra prediction. When the proposed intra prediction is compared by the intra prediction in HM, it is reported that 0.1%-0.8% bit-rate reduction is achieved for 1500-byte slices, as shown in Table 4.

Table 4  Results of intra prediction proposed by HiSilicon in JCTVC-E283 (anchor: original intra prediction in HM, 1500 bytes per slice)

| Case | Condition | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Proposed intra prediction off | 4.9 | 4.4 | 2.6 | 5.5 | 4.3 | 2.5 |
| 6 | Proposed intra prediction on | 4.3 | 4.1 | 2.5 | 4.7 | 4.0 | 2.4 |

## Subset3: Slice common information sharing

In the current HM, the ALF parameters are adapted at picture level. However, the entire ALF information is always sent in the first slice header even when a picture contains multiple slices, which is harmful for parallel slice decoding. In JCTVC-E045, for the sake of decoding multiple slices in parallel, an option is added to allow for moving ALF coefficients and slice common information to the picture parameter set (PPS) and distributing ALF CU on/off flags in separate slice headers. Compared with the current syntax in HM, the slice common information sharing technique reportedly show 0.1% bit rate increase and 0.1% bit rate decrease for HE and LC configurations respectively for 1500-byte slices, as shown in Table 5.

Table 5  BD-rates (%) of the slice common information sharing technique proposed by MediaTek in JCTVC-E045 (anchor: JCTVC-D600)

| Case | Condition | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1500 bytes per slice, sharing off | 4.9 | 4.4 | 2.6 | 5.5 | 4.3 | 2.5 |
| 2 | 1500 bytes per slice, sharing on | 4.8 | 4.6 | 2.9 | 5.3 | 4.2 | 2.6 |

# Conclusions

In CE4, six proposals were evaluated. The results are summarized according to coding efficiency. Based on the observed results in Subset1, it is suggested to adopt leaf-CU-aligned slices, to remove end-of-slice flags except for the last coding unit, and to establish a BoG on the addressing of slice starting positions. As for Subset2 and Subset3, it is suggested to review the results of the proposals with the JCT-VC committee and to make recommendation to adopt the proposed technologies in HM.

# References

- Y.-W. Huang and I.-K. Kim, “Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity,” Document of Joint Collaborative Team on Video Coding, JCTVC-D604, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.