| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E025 |
| --- | --- |

| Title: | CE5: Summary report of low complexity entropy coding improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE summary report |  |  |
| Author(s) or Contact(s): | Xianglin Wang Il Koo Kim Ping Wu | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

This document summarizes the activities in the Core Experiment CE5 on low complexity entropy coding (LCEC) improvements. A group of nine companies has registered for participation in CE5.

# Related documents

| Proposal | Title | Cross-check |
| --- | --- | --- |
| JCTVC-E319 (Sony) | Improvements on transform coefficients coding in LCEC | JCTVC-E245 (Cisco) JCTVC-E387 (Qualcomm) |
| JCTVC-E383 (Qualcomm) | Coefficient coding with LCEC for large block | JCTVC-E322 (Sony) |
| JCTVC-E404 (Qualcomm/Microsoft) | LCEC coded block flag coding under residual quadtree | JCTVC-E458 (Samsung) |
| JCTVC-E072 (ZTE) | Adaptive Coding of InterPredMode Syntax Elements | JCTVC-E393 (Qualcomm) |
| JCTVC-E160 (LGE) | Modified Joint Coding on Prediction Mode Signaling | JCTVC-E180 (ZTE) |
| JCTVC-E143 (Microsoft/ Qualcomm) | Counter based adaptation for LCEC | JCTVC-E159 (LGE) JCTVC-E178 (ZTE) JCTVC-E264 (MediaTek) |

# Summary of proposals

## Coefficient coding

3.1.a  JCTVC-E319, JCTVC-D261

In JCTVC-E319, for intra block, use dedicated mapping tables for Luma of 4x4, 8x8 and 16x16/32x32 in run mode coding. In addition to the single sorting table of HM2.0, 3 more sorting tables are used. It is reported that the BD-rate savings are 0.3% for Intra, 0.1% for random access and 0.1% for low delay.

It also extends the coefficient coding to 16x16 and 32x32 by: 1) Introduce tables to support large ranges of last nonzero transform coefficient position in 16x16 and 32x32 coefficient coding. 2) Extend run-mode table to support 16x16 and 32x32 coding, and the table is shared for Intra, Inter and Chroma blocks. It is reported that when extending the coefficient coding to 32x32 blocks the BD-rate savings are 2.4% for Intra, 1.5% for random access and 0.5% for low delay.

3.1.b  JCTVC-E383, JCTVC-D374

In JCTVC-E383, coefficient coding is extended to 16x16 and 32x32 block size, with special considerations in avoiding large size VLC table. The table size reduction mechanisms include: 1) In coding the last nonzero transform coefficient position, code number cn for {lev, last_pos} pair is calculated based on formula rather than look-up tables; 2) In level mode coding, for intra block, the same formula used for calculating code number cn for {lev, run}for 4x4 and 8x8 blocks is re-used for 16x16 and 32x32 block; for inter block, the code number calculation is also based on formula, which also trims the initial run-level table size at least by half.

It is reported that when extending the coefficient coding to 16x16 blocks, BD-rate savings are 2.5% for Intra, 1.1% for random access and 0.8% for low delay. When also extending the coding scheme to 32x32 blocks, BD-rate savings are 2.9% for Intra, 1.4% for random access and 1.0% for low delay.

## Coded block flag coding

JCTVC-E404, JCTVC-D375, JCTVC-D142

This is a joint proposal based on JCTVC-D375 and JCTVC-D142. According to the proposal, coded block flags and split flags are coded jointly to further improve coding efficiency. The original joint coding of Y, U and V coded block flag at CU root level is extended in two ways: 1) It further includes transform split flag if available; 2) The joint coding is not only performed at CU root level, but also extended to higher transform depth level so that the benefit of such joint coding can be maximized. It is reported that with the proposed coding scheme, there is slight BD-rate saving under default LC configurations. When using a maximum TU depth of 3, the BD rate savings are 0.3% for Intra, 0.5% for random access and 0.7% for low delay.

## Prediction mode coding and codeword adaptation

3.3.a  JCTVC-E072, JCTVC-D275

In this proposal, four syntax elements named split_flag_coding_type_depthX (where X = 0, 1, 2) and pred_mode_coding_type are introduced into P/B-slice header to indicate different coding types used in a current slice, where coding type is defined as type of coding order of the related elements, such as SkipFlag, MergeFlag, InterFlag, IntraFlag, split flag, etc. These flags are adaptively and jointly coded. The encoder determines the values of pred_mode_coding_type and split_flag_coding_type_depthX for the current slice according to the statistical results of the occurrence of the inter-frame prediction modes in each CU split layer obtained from the previous coded slices. It is reported that average BD-rate savings are 0.1% for random access and 0.3% for low delay, respectively.

3.3.b  JCTVC-E160, JCTVC-D084

In HM2.0, LCEC joint coding of prediction mode includes both prediction mode and prediction partition. According to this contribution, such joint coding is limited to prediction mode, with partition signaled separately. Additionally, initial mapping tables between symbol and codeword index are tuned according to the portion of each prediction mode. It is reported that the proposed method achieves BR-rate saving of 0.1% for random access and 0.1% for low delay case.

## Counter based adaptation for LCEC

JCTVC-E143, JCTVC-D140, JCTVC-D370

This is a joint proposal based on JCTVC-D140 and JCTVC-D370. According to the proposal, counters are used in VLC mapping adaptation to avoid unnecessary frequent swapping of codeword mapping associated with the current VLC adaptation in HM2.0. In the proposal, such counter based adaptation is applied to coded block flag coding, inter prediction mode coding and the joint coding of inter prediction direction and reference frame index. It is reported that the average BD-rate savings are 0.2% for Intra, 0.5% for random access and 0.8% for low delay.

# Conclusion

- Coding improvements are reported in every category listed section 3. It’s recommended that each document be reviewed and discussed in this meeting.

# References

- Qualcomm, “CE5: Improved coefficient coding with LCEC”, JCTVC-D374, Daegu, Korea, Jan. 2011.

- Samsung, “CE5: Efficient coefficient coding method for 16x16 and 32x32 transforms in LCEC mode”, JCTVC-D369, Daegu, Korea, Jan. 2011.

- Sony, “Improvements on transform coefficients coding in LCEC”, JCTVC-D261, Daegu, Korea, Jan. 2011.

- Microsoft, “A unified design of RQT cbf coding in LCEC”, JCTVC-D142, Daegu, Korea, Jan. 2011.

- Qualcomm, “LCEC coded block flag coding under residual quadtree”, JCTVC-D375, Daegu, Korea, Jan. 2011.

- Microsoft, “Adaptive coding order for skip and split flags in LCEC”, JCTVC-D140, Daegu, Korea, Jan. 2011.

- ZTE, “Adaptive coding of PredMode syntax elements”, JCTVC-D275, Daegu, Korea, Jan. 2011.

- LG Electronics, “Improvement on signaling method for prediction modes”, JCTVC-D084, Daegu, Korea, Jan. 2011.

- Qualcomm, “CE5: Improved coding of inter prediction mode with LCEC”, JCTVC-D270, Daegu, Korea, Jan. 2011.