| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E027_r2 WG11 Number: m19543 |
| --- | --- |

| Title: | CE7: Summary Report of Core Experiment on Alternative Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories Chuohao Yeo Institute for Infocomm Research Rajan Joshi Qualcomm Felix Fernandes Samsung Electronics Co., Ltd. | Email: | cohen@merl.com chyeo@i2r.a-star.edu.sg rajanj@qualcomm.com felix.f@sta.samsung.com |
| Source: | Mitsubishi Electric Research Laboratories, Institute for Infocomm Research, Qualcomm, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The purpose of Core Experiment 7 was to characterize the performance, in terms of both compression efficiency and complexity, of several transforms other than the 2-D integer DCT transform currently in HM. Some of these proposed transforms are secondary transforms applied after the HM transform, and some are used in place of the HM transform, under certain conditions. Some combinations of the proposed transforms are tested as well. This document and the accompanying spreadsheet provide a summary of activities and results for this core experiment.

# Introduction

The purpose of Core Experiment 7 (CE7) [1] is to investigate the performance of transforms other than the conventional integer DCT-like transform currently in HM (HM 2.0) [2]. The proposals submitted for CE7 along with their corresponding cross-verification documents and any related contributions are summarized in Table 1.

Table 1. CE7-related documents

| Tool # | Proponent (Tool Type) | CE7 Doc. | Cross-verification |  |  |  | Other related docs. | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R Huawei (Mode Dep. DCT/DST; Intra) | E098 | E075 Panasonic | E123 Samsung | E126 BBC |  |  | 2 variants |
| 2 | Samsung (Mode Dep. DCT/DST; Intra) | E125 | E099 I2R | E127 BBC | E461 Panasonic | E460 DOCOMO USA Labs |  | 4 variants |
| 3 | Peking Univ. |  |  |  |  |  |  | Withdrawn |
| 4 | Samsung (ROT; Intra) | E380 | E076 Panasonic | E420 Qualcomm |  |  |  | 4 variants |
| 5 | Panasonic (Secondary mode dep. transform; Intra) | E074 | E179 Toshiba | E379 Samsung |  |  |  |  |
| 6 | NHK (R-D adaptive selection of DCT or DST; Intra and Inter) | E107 | E413 MERL |  |  |  |  |  |
| 7 | HKUST |  |  |  |  |  |  | Withdrawn |

# Summary of proposed alternative transforms

The types of proposed transforms and methods generally fall into one or more of the following categories:

- Mode-dependent DCT/DST, where the horizontal and vertical transform depends upon the Intra prediction mode.

- Mode-dependent secondary 4x4 KLT, used horizontally and/or vertically depending upon mode.

- Secondary rotational transform, rate-distortion optimized selection.

- Adaptive selection of 2-D DCT or DST, rate distortion optimized selection.

Brief descriptions for each of the proposals are shown in Table 2. For more details, please refer to the document number associated with each tool.

Table 2. Brief descriptions of tools and their variants

| Tool # | Proponent | JCTVCDoc. | Tool Description |
| --- | --- | --- | --- |
| 1 | Qualcomm Toshiba I2R Huawei | E098 | (Intra) Horizontal and vertical transforms are HM DCT and/or odd type-III (type-VII) DST, depending upon Intra prediction mode. Two variants: 1: Coefficient scanning from HM 2.0; 2: “Weak” scans for low-complexity conditions. |
| 2 | Samsung | E125 | (Intra) Horizontal and vertical transforms are HM DCT and/or type-VII (odd type-III) DST, depending upon Intra prediction mode. Four experiments apply this technique to: 1: 4x4 & 8x8, DCT portion modified from HM; 2: 4x4, DCT portion modified from HM; 3: 4x4 & 8x8; 4: 4x4. |
| 3 |  |  | Withdrawn |
| 4 | Samsung | E380 | (Intra) Rate-distortion optimized selection of a secondary rotational transform, applied after the HM transform. Four variants: 1: ROT as secondary transform after HM transform; 2: Same as experiment 1, but ROT implemented via lifting; 3: ROT as secondary transform after Tool 2 (experiment 3) above; 4: Same, except ROT used as secondary transform only if Most Probable Mode for Intra prediction was used for current PU. |
| 5 | Panasonic | E074 | (Intra) For 4x4 blocks, Horizontal and vertical transforms are HM DCT and/or odd type-III (type-VII) DST, depending upon Intra prediction mode. For 8x8 and larger, the HM transform is performed first, followed by a secondary 4x4 KLT, which is applied horizontally and/or vertically (depending upon the Intra prediction mode) to the low-frequency coefficients. |
| 6 | NHK | E107 | (Intra and Inter) Rate-distortion optimized selection of HM DCT or type-II DST. The butterfly operations of the horizontal DCT and DST transforms are merged so that the DCT and DST coefficients are output simultaneously. |
| 7 |  |  | Withdrawn |

# Summary of Experimental Results

Table 3 summarizes the BD-Rate, encoding time, and decoding time summaries for each tool listed in the CE description. Entries highlighted in yellow, if present, indicate that only one contribution (either the proponent or cross-verifier) reported these values, so they have not yet been independently verified. LD-HE and LD-LC results are optional for tools that do not operate on Inter blocks. “N/A” stands for Not Applicable.

Table 3. Performance summaries for all proposed tools

|  | Overall BD-Rate change for the luma (Y) components (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (E098) | 2 (E125) | 3 | 4 (E380) | 5 (E074) | 6 (E107) | 7 |
| I-HE | -0.9 ¦ N/A | -1.0 ¦ -0.8 ¦ -1.0 ¦ -0.8 |  | -1.7 ¦ -1.6 ¦ -1.9 ¦ -1.4 | -1.1 | -0.5 |  |
| I-LC | -1.7 ¦ -2.5 | -1.8 ¦ -1.3 ¦ -1.8 ¦ -1.3 |  | -2.4 ¦ -2.3 ¦ -3.0 ¦ -2.3 | -1.7 | 0.1 |  |
| RA-HE | -0.4 ¦ N/A | -0.4 ¦ -0.3 ¦ -0.4 ¦ -0.3 |  | -1.0 ¦ -0.9 ¦ -1.0 ¦ -0.7 | -0.5 | -0.6 |  |
| RA-LC | -0.7 ¦ -1.1 | -0.7 ¦ -0.5 ¦ -0.7 ¦ -0.5 |  | -1.4 ¦ -1.3 ¦ -1.5 ¦ -1.0 | -0.7 | -0.1 |  |
| LD-HE |  |  |  |  |  | -0.6 |  |
| LD-LC |  |  |  |  |  | -0.4 |  |

|  | Encoding time as compared to reference (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (E098) | 2 (E125) | 3 | 4 (E380) | 5 (E074) | 6 (E107) | 7 |
| I-HE | 100 ¦ N/A | 102 ¦ 100 ¦ 102 ¦ 102 |  | 155 ¦ 163 ¦ 157 ¦ 123 | 101 | 141 |  |
| I-LC | 102 ¦ 104 | 104 ¦ 100 ¦ 100 ¦ 102 |  | 195 ¦ 204 ¦ 198 ¦ 130 | 104 | 156 |  |
| RA-HE | 100 ¦ N/A | 102 ¦ 103 ¦ 101 ¦ 103 |  | 110 ¦ 109 ¦ 108 ¦ 104 | 100 | 126 |  |
| RA-LC | 100 ¦ 100 | 102 ¦ 102 ¦ 104 ¦ 104 |  | 109 ¦ 106 ¦ 106 ¦ 102 | 100 | 113 |  |
| LD-HE |  |  |  |  |  | 123 |  |
| LD-LC |  |  |  |  |  | 111 |  |

|  | Decoding time as compared to reference (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Config | 1 (E098) | 2 (E125) | 3 | 4 (E380) | 5 (E074) | 6 (E107) | 7 |
| I-HE | 101 ¦ N/A | 100 ¦ 101 ¦ 100 ¦ 101 |  | 105 ¦ 106 ¦ 105 ¦ 103 | 102 | 100 |  |
| I-LC | 103 ¦ 104 | 101 ¦ 101 ¦ 103 ¦ 100 |  | 107 ¦ 110 ¦ 110 ¦ 105 | 105 | 100 |  |
| RA-HE | 100 ¦ N/A | 102 ¦ 100 ¦ 102 ¦ 100 |  | 107 ¦ 101 ¦ 101 ¦ 100 | 100 | 99 |  |
| RA-LC | 101 ¦ 101 | 100 ¦ 103 ¦ 103 ¦ 100 |  | 109 ¦ 101 ¦ 101 ¦ 100 | 100 | 100 |  |
| LD-HE |  |  |  |  |  | 100 |  |
| LD-LC |  |  |  |  |  | 102 |  |

# Summary of complexity

Detailed complexity metrics are included in a spreadsheet that accompanies this document. Tables 4-8 contain summaries. For each transform size, a summary of the average ops/pixel and worst-case ops/pixel is given. For proposed transforms that replace the HM transform, those metrics are for the proposed transform. For secondary transforms that are applied after the HM transform, those metrics include both transforms, giving an indication of the complexity of the full transform. In the lower part of the table, the metrics indicate how much complexity the proposed transform adds to HM. These values give an indication of how much more complex the proposed transform is over HM.

Table 4. Complexity summary for 4x4 transforms

| 4x4 |  | HM | 1 (E098) | 2 (E125) |  | 4 (E380) |  | 5 (E074) | 6 (E107) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  | DCT/DST | DCT/DST v1 | DCT/DST v2 | ROT | ROT (lifting) | Sec. Tx | Adaptive DCT/DST |
| Avg. Tx Ops/Pixel | Mults | 0.0 | 2.3 | 2.3 | 2.3 | 3.6 | 3.6 | 2.1 | 0.5 |
|  | Adds | 5.0 | 6.3 | 7.0 | 7.5 | 8.2 | 13 | 5.5 | 5.0 |
|  | Shifts | 2.0 | 2.3 | 3.0 | 3.5 | 3.6 | 7.2 | 1.5 | 2.0 |
| Worst-Case Tx Ops/Pixel | Mults | 0.0 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 8.0 | 1.0 |
|  | Adds | 5.0 | 7.5 | 9.0 | 10.0 | 9 | 15 | 9.0 | 5.0 |
|  | Shifts | 2.0 | 2.8 | 4.0 | 5.0 | 4 | 8.5 | 2.0 | 2.0 |
| Additional Avg. Tx Ops/pixel | Mults |  | 2.3 | 2.3 | 2.3 | 3.6 | 3.6 | 2.1 | 0.5 |
|  | Adds |  | 1.3 | 2.0 | 2.5 | 3.2 | 8 | 0.5 | 0.0 |
|  | Shifts |  | 0.3 | 1.0 | 1.5 | 1.6 | 5.2 | -0.5 | 0.0 |
| Additional Worst-Case Tx Ops/pixel | Mults |  | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 8.0 | 1.0 |
|  | Adds |  | 2.5 | 4.0 | 5.0 | 4 | 10 | 4.0 | 0.0 |
|  | Shifts |  | 0.8 | 2.0 | 3.0 | 2 | 6.5 | 0.0 | 0.0 |
| Additional Memory for transform | bytes |  | 9 | 5 | 5 | 72 | 108 | 32 | 64 |
| Additional R-D loops? (yes/no) | encoder |  | No | No | No | Yes | Yes | No | Yes |

Table 5. Complexity summary for 8x8 transforms

| 8x8 |  | HM | 1 (E098) | 2 (E125) |  | 4 (E380) |  | 5 (E074) | 6 (E107) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  | DCT/DST | DCT/DST v1 | DCT/DST v2 | ROT | ROT (lifting) | Sec. Tx | Adaptive DCT/DST |
| Avg. Tx Ops/Pixel | Mults | 0.0 | 8.0 | 8.0 | 8.0 | 3.6 | 3.6 | 0.7 | 0.0 |
|  | Adds | 10.0 | 13.0 | 12.5 | 13.0 | 13.2 | 18 | 10.7 | 10.5 |
|  | Shifts | 5.5 | 3.8 | 2.8 | 3.8 | 7.1 | 10.7 | 5.6 | 5.5 |
| Worst-Case Tx Ops/Pixel | Mults | 0.0 | 16.0 | 16.0 | 16.0 | 4.5 | 4.5 | 2.0 | 0.0 |
|  | Adds | 10.0 | 16.0 | 16.0 | 16.0 | 14 | 20 | 12.0 | 10.5 |
|  | Shifts | 5.5 | 5.5 | 4.5 | 5.5 | 7.5 | 12 | 5.8 | 5.5 |
| Additional Avg. Tx Ops/pixel | Mults |  | 8.0 | 8.0 | 8.0 | 3.6 | 3.6 | 0.7 | 0.0 |
|  | Adds |  | 3.0 | 2.5 | 3.0 | 3.2 | 8 | 0.7 | 0.5 |
|  | Shifts |  | -1.8 | -2.8 | -1.8 | 1.6 | 5.2 | 0.1 | 0.0 |
| Additional Worst-Case Tx Ops/pixel | Mults |  | 16.0 | 16.0 | 16.0 | 4.5 | 4.5 | 2.0 | 0.0 |
|  | Adds |  | 6.0 | 6.0 | 6.0 | 4 | 10 | 2.0 | 0.5 |
|  | Shifts |  | 0.0 | -1.0 | 0.0 | 2 | 6.5 | 0.3 | 0.0 |
| Additional Memory for transform | bytes |  | 64 | 64 | 64 | 72 | 0 | 32 | 288 |
| Additional R-D loops? (yes/no) | encoder |  | No | No | No | Yes | Yes | No | Yes |

Table 6. Complexity summary for 16x16 transforms

| 16x16 |  | HM | 1 (E098) | 2 (E125) |  | 4 (E380) |  | 5 (E074) | 6 (E107) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  | DCT/DST | DCT/DST v1 | DCT/DST v2 | ROT | ROT (lifting) | Sec. Tx | Adaptive DCT/DST |
| Avg. Tx Ops/Pixel | Mults | 5.5 | 0.0 |  |  | 6.4 | 6.4 | 5.7 | 5.5 |
|  | Adds | 10.3 | 0.0 |  |  | 11.1 | 12.3 | 10.4 | 10.8 |
|  | Shifts | 6.3 | 0.0 |  |  | 6.7 | 7.6 | 6.3 | 6.3 |
| Worst-Case Tx Ops/Pixel | Mults | 5.5 | 0.0 |  |  | 6.63 | 6.6 | 6.0 | 5.5 |
|  | Adds | 10.3 | 0.0 |  |  | 11.25 | 12.8 | 10.8 | 10.8 |
|  | Shifts | 6.3 | 0.0 |  |  | 6.75 | 7.9 | 6.3 | 6.3 |
| Additional Avg. Tx Ops/pixel | Mults |  |  |  |  | 0.9 | 0.9 | 0.2 | 0.0 |
|  | Adds |  |  |  |  | 0.8 | 2.0 | 0.2 | 0.5 |
|  | Shifts |  |  |  |  | 0.4 | 1.3 | 0.0 | 0.0 |
| Additional Worst-Case Tx Ops/pixel | Mults |  |  |  |  | 1.13 | 1.1 | 0.5 | 0.0 |
|  | Adds |  |  |  |  | 1.00 | 2.5 | 0.5 | 0.5 |
|  | Shifts |  |  |  |  | 0.50 | 1.6 | 0.1 | 0.0 |
| Additional Memory for transform | bytes |  |  |  |  | 0 | 0 | 32 | 1136 |
| Additional R-D loops? (yes/no) | encoder |  |  |  |  | Yes | Yes | No | Yes |

Table 7. Complexity summary for 32x32 transforms

| 32x32 |  | HM | 1 (E098) | 2 (E125) |  | 4 (E380) |  | 5 (E074) | 6 (E107) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  | DCT/DST | DCT/DST v1 | DCT/DST v2 | ROT | ROT (lifting) | Sec. Tx | Adaptive DCT/DST |
| Avg. Tx Ops/Pixel | Mults | 7.3 | 0.0 |  |  | 7.5 | 7.5 | 7.3 | 7.3 |
|  | Adds | 13.1 | 0.0 |  |  | 13.3 | 13.6 | 13.2 | 13.6 |
|  | Shifts | 7.1 | 0.0 |  |  | 7.2 | 7.5 | 7.1 | 7.1 |
| Worst-Case Tx Ops/Pixel | Mults | 7.3 | 0.0 |  |  | 7.5 | 7.5 | 7.4 | 8.3 |
|  | Adds | 13.1 | 0.0 |  |  | 13.4 | 13.8 | 13.3 | 13.1 |
|  | Shifts | 7.1 | 0.0 |  |  | 7.3 | 7.5 | 7.1 | 7.1 |
| Additional Avg. Tx Ops/pixel | Mults |  |  |  |  | 0.2 | 0.2 | 0.0 | 0.0 |
|  | Adds |  |  |  |  | 0.2 | 0.5 | 0.0 | 0.5 |
|  | Shifts |  |  |  |  | 0.1 | 0.3 | 0.0 | 0.0 |
| Additional Worst-Case Tx Ops/pixel | Mults |  |  |  |  | 0.3 | 0.3 | 0.1 | 1.0 |
|  | Adds |  |  |  |  | 0.3 | 0.6 | 0.1 | 0.0 |
|  | Shifts |  |  |  |  | 0.1 | 0.4 | 0.0 | 0.0 |
| Additional Memory for transform | bytes |  |  |  |  | 0 | 0 | 32 | 4320 |
| Additional R-D loops? (yes/no) | encoder |  |  |  |  | Yes | Yes | No | Yes |

# Additional activities

A combination of Tools 2, 4 and 5 (Mode-dependent DCT/DST of E125, rotational transform of JCTVC-E380, and mode-dependent secondary 4x4 transform of JCTVC-E074) are currently being tested. Here, if the UDI mode is not the same as the Most Probable Mode (MPM), then the 4x4 and 8x8 blocks use the DCT/DST of JCTVC-E125, and the 16x16 and 32x32 blocks use the HM transform followed by the secondary 4x4 transform of JCTVC-E074. If the UDI mode equals the MPM, then R-D optimization is performed over four secondary rotational transforms (ROT) as in JCTVC-E380. If any of the four transforms produce a cost better than using the HM transform alone, then the secondary ROT with the best cost is applied. Results from the proponents, which are currently undergoing cross-verification, are included in Table 8,

Table 8. BD-Rate results for combination of E380, E125, and E074 (cross-checking in progress)

# References

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “Description of Core Experiment 7 (CE7): Alternative Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D607, 4th JCT-VC Meeting, Daegu, Korea, Jan., 2011.

- HM 2.0 software, available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware