| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E028_r2 WG11 Number: m19725 |
| --- | --- |

| Title: | CE8: Non-deblocking Loop Filtering - Summary Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 In Suk Chong2 Matthias Narroschke3 |  | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com Matthias.Narroschke@eu.panasonic.com |
| Source: | CE coordinators (Toshiba1, Qualcomm2, Panasonic3) |  |  |

_____________________________

# Introduction

This contribution is a summary of core experiment 8 (CE8) on Non-deblocking loop filtering. There are five Subtests in CE8, Number of inputs (Subtest 1), Filter adaptation (Subtest 2), Enhancement schemes of loop filtering process (Subtest 3), Adaptation for Low Complexity design (Subtest 4) and Complexity reduction and/or quality improvement (Subtest 5) for adaptive loop filtering included HM 2. The number of proposals for each subtest is two, three, two, one and two, respectively, and those are evaluated based on the common conditions in JCTVC-D600. All results are verified by cross-checkers.

# Document list and cross-check assignment

- Summary document (this document)

  - JCTVC-E028 [CE coordinators] CE8: Non-deblocking Loop Filtering - Summary Report

- Proponent's and verification/study documents

Subtest 1: Number of inputs

  - JCTVC-E193 [Toshiba/MediaTek/Qualcomm] CE8.1: Deblocking parameter adjustment for 1-input ALF

    - Cross-check: JCTVC-E334 [Ericsson]

  - JCTVC-E140 [Sharp] CE8.1: DF-combined adaptive loop filter

    - Cross-check: JCTVC-E173 [Toshiba]

Subtest 2: Filter adaptation

  - JCTVC-E323 [Qualcomm/MediaTek/Toshiba] CE8 Subtest 2: Block based adaptive loop filter

    - Cross-check: JCTVC-E111 [Samsung], JCTVC-E137 [Sharp]

  - JCTVC-E046 [MediaTek] CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection

    - Cross-check: JCTVC-E247 [Panasonic]

  - JCTVC-E141 [Sharp] CE 8.2: Region-based adaptive loop filter using two-dimensional feature

    - Cross-check: JCTVC-E325 [Qualcomm]

Note: Samsung’s CE8.2 contribution was moved to CE8.3.

Subtest 3: Enhancement schemes of loop filtering process

  - JCTVC-E047 [MediaTek/Qualcomm/Toshiba] CE8 Subtest 3: Adaptation between ALF and AO

    - Cross-check: JCTVC-E109 [Samsung]

  - JCTVC-E108 [Samsung] CE8.3: Loop filter with directional similarity mapping (DSM)

    - Cross-check: JCTVC-E175 [Toshiba]

    - Cross-check: JCTVC-E265 [MediaTek], JCTVC-E138 [Sharp]

    - Cross-check: JCTVC-E327 [Qualcomm], JCTVC-E234 [TI]

Subtest 4: Adaptation for low complexity design

  - JCTVC-E320 [Sony] Parametric Adaptive Loop Filter

    - Cross-check: JCTVC-E065 [Mitsubishi], JCTVC-E331 [Qualcomm]

Subtest 5: Filter size and shape

  - JCTVC-E342 [eBrisk Video] CE8.5: Adaptive Loop Filtering Using Multiple Filter Shapes

    - Cross-check: JCTVC-E233 [TI]

  - JCTVC-E060 [TI] CE8.5: Luma ALF decode with reduced vertical filter size

    - Cross-check: JCTVC-E340 [eBrisk Video]

Note: Samsung’s CE8.5 contribution was moved to CE8.3.

# Experimental results

Based on the common test conditions in JCTVC-D600, CE8 specified test conditions as described in JCTVC-D608.  Results of the CE will be evaluated on the basis of BD-measures and/or complexity.

## Subtest 1: Number of inputs

|  | Toshiba/MediaTek/Qualcomm 1-input (JCTVC-E193) |  |  | Sharp parallel 2-inputs (JCTVC-E140) |  |  | Sharp non-parallel 2-inputs (JCTVC-E140) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD | IO | RA | LD |
| BD-rate | -0.7 | -0.5 | -0.5 | -1.3 | -0.7 | -0.5 | -1.1 | -0.7 | -0.7 |
| Enc. Time | 100 | 100 | 100 | 103 | 100 | 100 | 103 | 100 | 99 |
| Dec. time | 100 | 100 | 100 | 104 | 105 | 104 | 104 | 104 | 104 |
| Vert. taps | 7 |  |  | 7 |  |  | 7 |  |  |

Note: These results are for High Efficiency cases.

The difference of JCTVC-E193 from the anchor is parameters  and tc offsets for deblocking filter ( = -2, tc = -4).

The difference of parallel ALF of JCTVC-E140 from the anchor is applying the same shape of filter to pre-deblocked pixels and obtaining the weighted sum with the deblocked pixel.

The differences of non-parallel ALF of JCTVC-E140 from the anchor are applying the same shape of filter to deblocked pixels and obtaining the weighted sum with the pre-deblocked pixel, and parameters  for deblocking filter ( = -6).

### Comments from cross-checkers

To MediaTek, Qualcomm and Toshiba (JCTVC-E193): Ericsson confirmed the results provided by the proponent.  The subjective quality is similar to the anchor on the majority of the sequences except two cases, Vidyo3 and Vidyo4 low-delay, QP37, which are worse than the anchor.

To Sharp (JCTVC-E140): Toshiba confirmed the results provided by the proponent.  Toshiba reviewed the provided software by the proponent, and confirmed that the modification followed what was claimed in their contribution.

## Subtest 2: Filter adaptation

Note: Samsung’s results to CE8.2 are moved to subtest 3 due to the inclusion of non-linear processing part.

|  | Qualcomm /MediaTek /Toshiba 4x4 block (JCTVC-E323) |  |  | MediaTek (JCTVC-E046) |  |  | Sharp 4x4 block (JCTVC-E141) |  |  | Sharp 2x block (JCTVC-E141, additioanl) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD | IO | RA | LD | IO | RA | LD |
| BD-rate | -0.2 | -0.5 | -0.4 | 0.0 | -0.2 | -0.4 | -0.2 | -0.5 | -0.6 | -0.4 | -0.8 | -0.9 |
| Enc. Time | 102 | 102 | 102 | 102 | 101 | 101 | 83 | 94 | 94 | 94 | 97 | 97 |
| Dec. time | 93 | 92 | 94 | 99 | 96 | 97 | 96 (88) | 98 (90) | 99 (90) | 103 (93) | 104 (95) | 106 (96) |
| Vert. taps | 7 | 7 | 7 | 7 |  |  | 7 |  |  | 7 |  |  |

Note: These results are for High Efficiency cases.

Sharp reported two decoding times which measured on Linux and Windows, where without brackets was on Linux, in brackets was on Windows. The Linux result (larger one) was similar to the cross-checker’s result which also measured on Linux.

The difference of JCTVC-E323 from the anchor is replacing the pixel based classification in the anchor with a 4x4 block based pixel classification (sum of Laplacian based). And harmonizing Laplacian based technique with directional information. There is no changes in syntax and encoding techinques.

The difference of JCTVC-E046 from the anchor is adding Single-Filter mode that can be switched from the pixel based classification in the anchor.  The encoding algorithm is almost the same as the anchor, except one extra encoding pass for Single-Filter mode, compared to original multi-pass encoding algorithm in HM2.0.  This extra encoding pass can be processed in parallel with the original multi-pass encoding algorithm in HM2.0 in principle.

The differences of JCTVC-E141 from the anchor are

- replacing the pixel based classification with a 4x4 or 2x2 block based pixel classification (two-dimensional classification with six-level activities and six-level directions)

- replacing the region classification syntax with two-dimensional hierarchical one

- replacing the filter prediction and entropy coding with a new one which doesn’t use golomn coding

- replacing the filtering and correlation acquirement implementation with a re-write one for compatibility of multi-input ALF.

The encoding algorithm is similar to the anchor, except the design of region classification in an encoding pass for two-dimensional hierarchical representation.

Note: Decoding time varies between Linux and Windows. Decoding time of Linux is similar to that of the cross-checker.

### Comments from cross-checkers

To Qualcomm CE8.2 block adaptation (JCT-VC-E323): Sharp reviewed the provided software by the proponent, and confirmed that the coding results perfectly match those of proponents and the software follows the CE8's description.

Samsung checked Qualcomm’s JCTVC-E323:

The coding results are exactly matched.  Run-time also follows closely to the results reported by Qualcomm: Increases in enc time <5%; with decreases in decoding time, partially due to black-based filter adaptation (BA) as explained below.

As for code-study, since this is a new proposal, details are provided as follows:

- Block-based filter adaptation (BA):

  - For each pixel, the neighborhood to compute sum-modified Laplacian (SML) is changed from ±3×±3 in HM2.0 to ±1×±1.

  - For each 4×4 block, the average of the 16 SML values from all 16 pixels is computed as the feature to classifying all pixels in the 4×4 block into the same class.

In conclusion, the saving in computation is using smaller neighborhood to compute SML, but not in using fewer SML values (still 16 SML values for 16 pixels).

- To further perform filter adaptation with directional features (DIR):

  - Directional features (vertical and horizontal) are calculated using [-1, 2, -1] weights.

  - To accommodate both directional feature and SML feature, initially the classification might produce up to 3×16 = 48 filters (set by NO_VAR_BINS). Then when computing filter coefficients and determining number of filters, the mergeFiltersGreedy funtion will return up to 16 filters (set by NO_FILTERS).

In conclusion, the reported increases in encoding time may be attributed to the computation of initially up to 48 filters, and the merging process to produce the resulting up to 16 filters.

To MediaTek (JCTVC-E046): Panasonic confirmed the results provided by the proponent except for -encoding and decoding time which may be caused by the difference of operating environments..

To Sharp CE8.2 block adaptation (JCTVC-E141): Qualcomm reviewed the provided software by the proponent, and confirmed that the coding results perfectly match those of proponents. and the software follows the CE8's description. Encoding time is in similar order, and decoding time is similar to Sharp’s number with linux cluster. Note that Qualcomm used a single windows PC to check decoding time.

## Subtest 3: Enhancement schemes of loop filtering process

|  | MediaTek/Qualcomm/Toshiba (JCTVC-E047) |  |  | Samsung CE8.3 (JCTVC-E108)* |  |  | Samsung Moved from CE8.2 (JCTVC-E108)** |  |  | Samsung Moved from CE8.5 (JCTVC-E108)*** |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD | IO | RA | LD | IO | RA | LD |
| BD-rate | -0.2 | -0.7 | -1.0 | -0.2 | -0.2 | 0.3 | -0.2 | -0.1 | 0.4 | 0.2 | 0.4 | 1.0 |
| Enc. Time | 100 | 100 | 100 | 217 (163) | 159 (125) | 144 (117) | 209 (158) | 156 (122) | 139 (115) | 126 (126) | 120 (112) | 109 (105) |
| Dec. time | 89 | 89 | 90 | 171 (146) | 137 (130) | 133 (124) | 184 (178) | 145 (153) | 140 (146) | 138 (139) | 125 (131) | 112 (124) |
| Vert. taps | 7 |  |  | 7 |  |  | 7 |  |  | 5 |  |  |

Note: These results are for High Efficiency cases.

In the table above, Samsung provides additional enc/dec time data from the cross-checkers: JCTVC-E256 for CE8.3, JCTVC-E175 for originally CE8.2, and JCTVC-E234 for originally CE8.5.

The difference of JCTVC-E047 from the anchor is adding adaptive offset mode that can be switched from adaptive loop filter, and the pixel classification method is unified for both adaptive loop filter and adaptive offset. The encoding algorithm is almost the same as the anchor, except one extra encoding pass for adaptive offset is implemented, compared to original multi-pass encoding algorithm in HM2.0. This extra encoding pass can be processed in parallel with the original multi-pass encoding algorithm in HM2.0 in principle.

The differences of JCTVC-E108 from the anchor are:

- * Replacing the pixel based classification with a 4x4 block based pixel classification. The classification uses four directional gradients H, V, D, U with [1, -1] weights. Each 4x4 block will be classified into 1 of the 4 directional classes to adapt to a directional filer.

- **,*** Replacing the pixel based classification with a 4x4 block based pixel classification. The classification uses four directional gradients H, V, D, U with [-1, 2, -1] weights. Each 4x4 block will be classified into 1 of the 4 directional classes to adapt to a directional filer.

- *,**,*** The filter for each class (f_h, f_v, h_d, h_u) is a combination of a non-linear directional filter part and a liner spatial filter part. The symmetry in the directional filter part follows the direction of the classification. Neighboring pixels used in the directional filters will be modified with a thresholding (thus non-linear) on the difference between them and the center pixel.

- *,** The four filters f_h, f_v, h_d, h_u are 7x7-tap diamond only, no other sizes and shapes. For f_h and f_v, the number of coefficient is 25; for h_d and h_u, the number of coefficient is 23.

- *** The four filters f_h, f_v, h_d, h_u are 5x5-tap diamond only, no other sizes and shapes. For f_h and f_v, the number of coefficient is 14; for h_d and h_u, the number of coefficient is 13.

### Comments from cross-checkers

To MediaTek, Qualcomm and Totshiba (JCTVC-E047): Samsung checked MQT’s JCTVC-E047.

The coding results are exactly matched. As for run-time, the reduction in decoding time is confirmed. This is due to the fact that there will be slices that use AO instead of ALF.

- In the unified pixel classification method, for each pixel, the window to compute sum-modified Laplacian is changed from 7×7 in HM2.0 to 1×1. If there were no such change, the computation in pixel classification is expected to increase with the addition of computing edge pattern for each pixel.

- In HM 2.0, ALF is not used for non-referenced B frames. However in JCTVC-E047, AO method is enabled for non-referenced B fames, while ALF is still disabled. The will affect the results for random access cases.

To Samsung CE8.3 (JCT-VC-E108): Sharp reviewed the provided software by the proponent, and confirmed that the coding results perfectly match those of proponents and the software follows the CE8's description.

To Samsung CE8.2 7x7 filter (JCTVC-E108): Toshiba reviewed the provided software by the proponent, and found that the modification included the non-linear filtering part that was not expected in CE8.2.  After the discussion with the proponent, it was agreed on this result be compared with CE8.3 results.  Encoding and decoding time have some variance ranging from 0% to 50%.  However, even if the smaller value is adopted, increase of time seems large.

## Subtest 4: Adaptation for Low Complexity design

|  | Sony (JCTVC-E320) |  |  |
| --- | --- | --- | --- |
|  | IO | RA | LD |
| BD-rate | -1.9 | -3.4 | -4.1 |
| Enc. Time | 131 | 106 | 104 |
| Dec. time | 153 | 122 | 121 |
| Vert. taps | 9 (luma), 7 (chroma) |  |  |
| # of coef. sets | 32 (luma, chroma) |  |  |

Note: These results are for Low Complexity cases.

The difference of JCTVC-E320 from the anchor is to use sets of pre-designed filter coefficients.  One of them is selected during encoding and an index to specify which set is used is signaled.  There are 32 fixed filters for luma with a base shape of 9x9 rectangle and 32 filters for chroma with a base shape of 7x7 rectangle. Number of nonzero elements for luma and chroma filters are 23 and 17, respectively. A 14-bit precision is used to describe the filter coefficients.

### Comments from cross-checkers

To Sony (JCTVC-E320): Mitsubishi confirmed the results provided by the proponent.  An additional information for High Efficiency cases is reported in JCTVC-E065.

## Subtest 5: Filter size and shape

|  | eBrisk Video (JCTVC-E342) |  |  | TI (JCTVC-E060) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| BD-rate | 0.3 | 0.1 | 0.2 | 0.2 | 0.2 | 0.3 |
| Enc. Time | 99 | 101 | 100 | 98 | 96 | 96 |
| Dec. time | 87 | 94 | 93 | 95 | 94 | 96 |
| Vert. taps | 7 | 7 | 7 | 5 |  |  |
| # of coefs. | 10 | 10 | 10 | 18 (max, inc. offset) |  |  |
| Luma only | Yes | Yes | Yes | Yes |  |  |

Note: These results are for High Efficiency cases.

The difference of JCTVC-E060 from the anchor is using three different shapes with Nx5-set 1 filter.  In JCTVC-E060, additional results of Nx5-set 3 filter are provided.

The difference of JCTVC-E342 from the anchor is the use of two different shapes with 10 coefficients.  More specifically, we select between a set of 5×7 diamond-like-shaped filters and a set of 13×7 cross-shaped filters. The selection is performed during the first step of the algorithm. After one of the two sets of filters is selected, the encoding process is the same as in HM 2.0.

### Comments from cross-checkers

To eBrisk Video (JCTVC-E342): TI confirmed the results provided by the proponent except for -encoding and decoding time which may be caused by the difference of operating environments.

To TI (JCTVC-E060): eBrisk Video confirmed the results provided by the proponent.

# References

[1]	“Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January 2011.

[2]	“Description of CE8: Non-deblocking loop filtering”, JCTVC-D608, Daegu, January 2011.