| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E029 WG11 Number: m19746 |
| --- | --- |

| Title: | CE9: Summary report of core experiment 9 on MV Coding and Skip/Merge operations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | J. Jung 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com |
|  | B. Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin |  | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
|  | P. Chen Qualcomm 5775 Morehouse Drive San Diego, CA, USA |  | +1 858 845 1347 peisongc@qualcomm.com |
|  | W.-J. Han Samsung Electronics Suwon, Korea |  | +82 10 3329 6393 wjhan.han@samsung.com |
| Source: | CE coordinators |  |  |

___________________________

# Introduction

This document summarizes the activities in the Core Experiment CE9 on motion vector coding and Skip/Merge operations. It also includes adaptive motion vector resolution experiments. A total of 15 companies or universities have registered to the CE. A few emails were exchanged mainly to coordinate the cross-check activity and exchange the modified versions of the HM2.0 software. Also it can be noticed that some experiments were withdrawn by the proponents due to lack of time running the experiment, or insufficient results. In some cases of very close experiments, only the best performing method has been maintained in the CE. Finally, 29 experiments are proposed for the MV coding part, and 2 for the adaptive MV resolution part.

# Documents List

MV Coding

|  | Tester 1 | Tester 2 (cross-check) |
| --- | --- | --- |
| A | JCTVC-E230 – Panasonic | JCTVC-E250 - HHI |
| B | JCTVC-E204- HHI | JCTVC-E246 - Orange Labs |
| C | JCTVC-E204- HHI | JCTVC-E228 - Panasonic |
| D | JCTVC-E204 - HHI | JCTVC-E103 - I2R |
| E | JCTVC-E116 - TI | JCTVC-E155 - NTT DOCOMO |
| F | JCTVC-E116 – TI | JCTVC-E155 - NTT DOCOMO |
| G | JCTVC-E155 - NTT DOCOMO | JCTVC-E218 - Canon |
| H | JCTVC-E155 - NTT DOCOMO | JCTVC-E218 - Canon |
| I | JCTVC-E230 – Panasonic | JCTVC-E058 - Sony |
| J | JCTVC-E230 – Panasonic | JCTVC-E058 - Sony |
| L | JCTVC-E102 - I2R | JCTVC-E419 - SKKU |
| M01 | JCTVC-E048 - MediaTek | JCTVC-E273 - Samsung |
| M07 | JCTVC-E048 - MediaTek | JCTVC-E273 - Samsung |
| M10 | JCTVC-E048 - MediaTek | JCTVC-E273 - Samsung |
| M11 | JCTVC-E048 - MediaTek | JCTVC-E158 - LG |
| M12 | JCTVC-E048 - MediaTek | JCTVC-E158 - LG |
| M22 | JCTVC-E048 - MediaTek | JCTVC-E229 - Panasonic |
| M23 | JCTVC-E048 - MediaTek | JCTVC-E229 - Panasonic |
| M25 | JCTVC-E048 - MediaTek | JCTVC-E229 - Panasonic |
| M26 | JCTVC-E048 - MediaTek | JCTVC-E226 - Panasonic |
| M27 | JCTVC-E048 - MediaTek | JCTVC-E246 - Orange Labs |
| M28 | JCTVC-E048 - MediaTek | JCTVC-E246 - Orange Labs |
| M29 | JCTVC-E048 - MediaTek | JCTVC-E158 - LG |
| M30 | JCTVC-E048 - MediaTek | JCTVC-E158 - LG |
| M31 | JCTVC-E048 - MediaTek | JCTVC-E158 - LG |
| N | JCTVC-E350 - LG & Qualcomm | JCTVC-E267 - MediaTek |
| O | JCTVC-E350 - Qualcomm & Qualcomm | JCTVC-E285 - HHI |
| S | JCTVC-E230 – Panasonic | JCTVC-E058 - Sony |
| T | JCTVC-E204 – HHI | JCTVC-E267 - MediaTek |

Table 1:

Adaptive MV resolution

|  | JCTVC-Exxx - | JCTVC-Exxx - |
| --- | --- | --- |
|  | JCTVC-Exxx - | JCTVC-Exxx - |

Table 2:

# Overview of the experiments and results

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | Candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| A | HM2.0 | HM2.0 | HM2.0 | JCTVC-D274(2) | HM2.0 |
| B | HM2.0 | HM2.0 | MERGE | derived | derived |
| C | HM2.0 | AMVP | HM2.0 | derived | derived |
| D | MERGE | HM2.0 | MERGE | derived | derived |
| E | JCTVC-D055-b | HM2.0 | JCTVC-D055-b | HM2.0 | HM2.0 |
| F | JCTVC-D055-c | HM2.0 | JCTVC-D055-c | HM2.0 | HM2.0 |
| G | JCTVC-D055-b | HM2.0 | MERGE | derived | derived |
| H | JCTVC-D055-c | HM2.0 | MERGE | derived | derived |
| I | JCTVC-D274 (3) | HM2.0 | JCTVC-D274 (3) | HM2.0 | HM2.0 |
| J | JCTVC-D274 (3) | HM2.0 | JCTVC-D274 (3) | JCTVC-D274 (2) | HM2.0 |
| L | HM2.0 | HM2.0 | JCTVC-D051 (MERGE reduced candidate set) | derived | derived |
| M01 | JCTVC-D125 S2.1 (1 T MVP) | HM2.0 | JCTVC-D125 S 2.1 (1 T MVP) | HM2.0 | HM2.0 |
| M07 | JCTVC-D125 S2.3 | HM2.0 | JCTVC-D125 S S2.3 | HM2.0 | HM2.0 |
| M10 | JCTVC-D125 S2.4 | HM2.0 | JCTVC-D125 S2.4 | HM2.0 | HM2.0 |
| M11 | HM2.0 | JCTVC-D125 S2.4 | HM2.0 | HM2.0 | HM2.0 |
| M12 | JCTVC-D125 S2.4 | JCTVC-D125 S2.4 | JCTVC-D125 S2.4 | HM2.0 | HM2.0 |
| M22 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | HM2.0 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | HM2.0 | HM2.0 |
| M23 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | JCTVC-D125 S2.4 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | HM2.0 | HM2.0 |
| M25 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | HM2.0 | HM2.0 |
| M26 | JCTVC-D125 S2.3 S2.4 | JCTVC-D125 S2.1 (1 T MVP) S2.4 | JCTVC-D125 S2.3 S2.4 | HM2.0 | HM2.0 |
| M27 | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4 | JCTVC-D125 Section 2.4 | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4 | HM2.0 | HM2.0 |
| M28 | JCTVC-D125 S2.1(4 T MVPs) S2.3 S2.4 | JCTVC-D125 S2.4 | JCTVC-D125 S2.1(4 T MVPs) S2.3 S2.4 | HM2.0 | HM2.0 |
| M29 | JCTVC-D125 S2.1 (1 T MVP*) S2.3 JCTVC-D274 (3’) | JCTVC-D274 (3’) | JCTVC-D125 S2.1 (1 T MVP*) S2.3 JCTVC-D274 (3’) | JCTVC-D274 (2) | HM2.0 |
| M30 | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4’ | JCTVC-D125 S2.4’ | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4’ | HM2.0 | HM2.0 |
| M31 | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4’ | JCTVC-D125 2.4’ | JCTVC-D125 S2.1 (1 T MVP*) S2.3 S2.4’ | JCTVC-D274 (2) | HM2.0 |
| N | JCTVC-D095 | HM2.0 | JCTVC-D095 | HM2.0 | HM2.0 |
| O | JCTVC-D095 | HM2.0 | MERGE | derived | derived |
| S | JCTVC-D274 (3’) | JCTVC-D274 (3’) | JCTVC-D274 (3’) | HM2.0 | HM2.0 |
| T | JCTVC-D125 S2.1 (1 T MVP*) S2.3 JCTVC-D274 (3') | JCTVC-D125 S2.1(1 T MVP*) JCTVC-D274 (3') | JCTVC-D125 S2.1(1 T MVP*) JCTVC-D274 (3') | derived | derived |

Table 3:

Figure 1:

Some experiments are not mentioned in Figure 1 (E, F, G, H, L). Complexity reduction (memory access and arithmetic operations) without bitrate losses are reported.

# Detailed experiments

## JCTVC-D051 (I2R)

JCTVC-D051 presented a coding scheme that aimed to simplify the Merge/Skip/Direct mechanisms within the HM1 design by reducing the degrees of freedom of the Skip mode and removing the Direct mode. One characteristic of the proposed design is the possibility of coding a merged CU with a single bin. In this experiment, in view of the adoption of various changes to the Merge/Skip/Direct/Inter modes, it is planned to investigate a simpler design of the remaining Merge/Skip modes using the same approach as in D051.

## JCTVC-D055 (TI)

The contribution JCTVC-D055 advocates simplification of upper/left spatial motion vector predictor calculation by using up to two neighboring Pus instead of up to 16 Pus as currently defined in the test model, no coding loss has been reported by this simplification. It is recommended to evaluate the impact of this simplification on the coding efficiency and complexity. In this experiment, the following two simplifications of upper/left spatial motion vector predictor calculation will be evaluated. Only number of PU to be searched will be changed in this evaluation.

Anchor: Current default setting (HM2.0)

In the HM2, as shown in Figure 3.1.3a, the spatial MVP candidates (left, upper) of the current prediction unit (PU) are calculated from the motion vectors of its neighboring PUs, and has the following steps:

Figure 3.1.3a

- MVP candidate from left neighbors:

  - Scanning the left neighbors (PU) from bottom to top (A0 … Am,Am+1; maximum 17 Pus), find the

first PU that meets the following conditions:

    - It is available

    - It is inter-coded

    - It has a MV for the same direction (L0 or L1)

    - It uses the same reference index

  - Otherwise, left predictor is set to (0,0) and marked unavailable.

- MVP candidate from upper neighbors :

  - Do the same procedure with one following additional condition (as that of left predictor) for the upper

neighbors, scanning from right to left (B-1,B0 … Bn,Bn+1, maximum 18 Pus)

    - It is not same to left predictor

Note that m and n can be 0 if the current PU has the minimum vector block size allowed by the HEVC standard.

JCTVC-D055- b: Reduction of search partition in left/top PMV candidate detection (JCTVC-D055)

As shown in Figure 3.1.3b, for the search of left predictor, instead of scanning all the left neighboring Pus from bottom to top, only up to three neighboring Pus, Am+1, Am,A0 are used. Similarly, For the search of top predictor, only up to four neighboring Pus,Bn+1,Bn,B0,B-1 ,are used. Other steps to select MVP candidate are unchanged.

Figure 3.1.3b

JCTVC-D055-c: Further reduction of search partition in left/top PMV candidate detection

As shown in Figure 3.1.3c, left/upper MVP candidate calculation is further simplified. For the search of left predictor, instead of scanning all the left neighboring Pus from bottom to top, only up to two neighboring Pus, Am+1,Am, are used. Similarly, For the search of top predictor, only up to three neighboring Pus, Bn+1,Bn ,B-1　are used. Other steps to select MVP candidate are unchanged.

Figure 3.1.3c

## JCTVC-D095 [LG and Qualcomm] Improvements on median motion vectors of AMVP

One changes on spatial neighboring MVs for median vectors in AMVP was proposed.

In AMVP, MVs with the same reference index and same reference list are regarded as spatial neighboring MV. But in the proposed method, MVs with a different reference index or a different reference list are also considered. The first available MV is chosen with pre-determined order, and it is scaled and inverted according to temporal distances and prediction directions. It is then used to calculate median MV predictors.

The proposed method can be also applied to other spatial MV candidates as well.

One experiment in this CE is proposed: test predefined priority and MV scaling for deriving each spatial MVP candidate.

Based on this proposal, tests are also conducted to verify the performance of current skip mode, merge mode, and AMVP mode in HM-2.0, which suggests some directions for unifying the skip, merge, and inter mode in motion vector prediction.

## JCTVC-D125 [MediaTek] Improved AMVP

Four changes to AMVP were proposed:

- First, in general cases, six spatial motion vector prediction (MVP) candidates and four temporal MVP candidates are used in the MVP candidate set for motion vector competition (MVC); in low complexity configurations, when the picture order count (POC) difference between the current picture and the co-located picture is equal to 1 and the co-located MV exists, the MVP candidates are reduced to one spatial MVP candidate and four temporal MVP candidates.

- Second, the order of the co-located temporal MVP is modified according to the POC difference.

- Next, when each spatial MVP candidate is derived, a predefined priority is used to select one motion vector (MV) from list 0 and list 1 MVs of the spatial neighboring block, and the chosen MV is scaled to the same reference picture of the current prediction unit (PU) as the spatial MVP candidate for MVC.

- Last, when each temporal MVP candidate is derived, a predefined priority is used to select one MV from list 0 and list 1 MVs of two co-located blocks, and the chosen MV is scaled to the same reference picture of the current PU as the temporal MVP candidate for MVC. A simplification of the temporal predictor derivation in current HM 2.0 is also proposed by removing the process of checking shortest reference distance. If both vectors cross the current frame, or both don’t cross, the co-located vector in same list will be taken first.

The four improvements can also be applied for the new Skip and Merge that are adopted at the Daegu meeting.

The following tests are proposed.

- Section 2.1: Test different numbers of MVP candidates. Variants:

  - 1 T MVP: one single temporal MV predictor (H)

  - 1 T MVP*: one single temporal MV predictor (H) – Tctr used when H not available

  - 4 T MVPs: 4 temporal MV predictors in competition (H, F, G, Tctr)

- Section 2.3: Test predefined priority and MV scaling for deriving each spatial MVP candidate

- Section 2.4: Test predefined priority and MV scaling for deriving each temporal MVP candidate. Variants:

  - 2.4: The temporal derivation algorithm as suggested in D125.

  - 2.4’: Simplification of the temporal predictor derivation in HM2.0

## JCTVC-D274 (Panasonic)

The contribution JCTVC-D274 by Panasonic reports modified derivation process of reference index for skip mode and temporal motion vector predictor. It includes:

- JCTVC-D274 (2): modified reference index decision for skip mode. Reference indices for skip mode follow the majority of reference indices of neighboring prediction units A, B and C (Figure 4.5.1). If prediction unit C is not available, prediction units D or E is used.

Figure 4.5.1: Neighboring prediction units to decide reference indices for skip mode

- JCTVC-D274 (3): modified temporal motion vector predictor. Two temporal motion vector predictors (mvL0Col and mvL1Col), which derived from motion vector mvL0 and mvL1 of co-located partition respectively, are added to motion vector predictor candidates when co-located partition is forward directional bi-prediction (Figure 4.5.2). The flowchart of modified derivation process of temporal motion vector predictors is shown in Figure 4.5.3. Non-highlighted blocks are same as original HM2.0 process, and highlighted blocks are newly added in this modification.

Figure 4.5.2: two temporal motion vector predictors in modified method

Figure 4.5.3: flowchart of derivation process for temporal motion vector predictor

- JCTVC-D274 (3’): instead of adding two derived temporal motion vector predictors in (3), one temporal motion vector predictor is selected according to referred direction and added into mvpList. It was recommended to evaluate the impact of this modification on the coding efficiency and complexity. One temporal motion vector predictor (mvL0Col or mvL1Col) is selected according to referred direction and added to mvpListX. The flowchart of modified derivation process of temporal motion vector predictors is shown in Figure 4.5.4. Non-highlighted blocks are same as original HM2.0 process, and highlighted blocks are newly added in this modification.

Figure 4.5.4: flowchart of modified derivation process for temporal motion vector predictor

## Merge Skip (HHI)

In HM2.0, the skip mode uses the AMVP method to derive motion information that is slice dependent. For P slices, a motion vector predictor (MVP) index is signaled for list0 prediction. For B slices, a MVP index is signaled for list0 prediction and a MVP index is signaled for list1 prediction. The reference index is always set to 0, the motion vector(s) is (are) equal to the MVP(s) and no residual is transmitted.

If the skip mode uses the merge method derive motion information, a merge index is signaled. The motion information is fully derived from the neighboring block indexed by the merge index and no residual is transmitted either.

# Adaptive MV Resolution

xxx

# Conclusion

Several experiments have been performed with different configurations of the software. Trying to summarize, the following observations can be made:

- A set of 4 configurations performs better than the other: M31 (-1.7%), M29 (-1.7%), M26 (-1.6%), T (-1.6%), in terms of coding efficiency.

- Complexity reduction (memory access and arithmetic operations) without bitrate losses are reported in experiments E, F, G, H.

Suggestion:

Similarly to what has been done in Daegu, creation of a BoG to check inclusion of ‘simplifications’ (and possibly other contributions) on top of the best performing method(s).