| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E031 WG11 Number: m19568 |
| --- | --- |

| Title: | CE11: Summary report on coefficient scanning and coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vivienne Sze (coordinator) Texas Instruments Krit Panusopone Motorola Mobility Jianle Chen Samsung Tung Nguyen Fraunhofer HHI Muhammed Coban Qualcomm | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-858-404-3618 krit@motorola.com +82-31-279-8826 jianle.chen@samsung.com +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de +1-858-658-3973 mcoban@qualcomm.com |
| Source: | Texas Instruments, Motorola Mobility, Samsung, Fraunhofer HHI, Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 11, Coefficient scanning and coding.  Eleven companies have been registered in CE11 and tools from five proposals have been evaluated on the condition defined in document by the Software Ad Hoc Group. A sixth proposal was withdrawn.  Several of the proposed tools had several variations, totaling a set of ten results.  In addition to measuring the coding efficiency, the bins statistics for distribution across syntax elements, method of coding bins and number of contexts are reported.  Methods of scan pattern selection and signaling are also to be reported by proponents.

# Introduction

Various methods have been proposed for coding the transform coefficients of the residual to reduce complexity and/or improve coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of

- Alternative binarization and context modeling/selection for syntax elements related to transform coefficients. (i.e. significiant_coeff_flag, last_signifiicant_coeff_flag, and abs_coeff_level_minus1)

- transform coefficient scanning order methods

The objectives of this CE include:

- Investigation of the impact of alternative binarization and context selection schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order method when combining with context selection

Complexity measures are described in Section 5.2

# Participants List

| Company | Participants |
| --- | --- |
| TI | Vivienne Sze, sze@ti.com Madhukar Budagavi, madhukar@ti.com |
| HHI | Tung Nguyen, tung.nguyen@hhi.fraunhofer.de Heiko Schwarz, heiko.schwarz@hhi.fraunhofer.de |
| Panasonic | Hisao Sasai, sasai.hisao@jp.panasonic.com |
| Qualcomm | Muhammed Coban, mcoban@qualcomm.com Joel Sole, joels@qualcomm.com Yunfei Zheng, zhengy@qualcomm.com |
| Samsung | Jianle Chen, jianle.chen@samsung.com Vadim Seregin, V.seregin@samsung.com |
| Huawei | Jin Song, andy.songjin@huawei.com Haitao Yang, haitao.yang@huawei.com Haoping Yu, haopingyu@huawei.com |
| RIM | Dake He, dhe@rim.com |
| Sony | Cheung Auyeung, Cheung.Auyeung@am.sony.com |
| DOCOMO USA Labs | Frank Bossen, bossen@docomolabs-usa.com |
| Sharp | Andrew Segall, asegall@sharplabs.com Yukinobu Yasugi, yasugi.yukinobu@sharp.co.jp Tomoyuki Yamamoto, yamamoto.tomoyuki@sharp.co.jp |
| Motorola Mobility | Krit Panusopone, krit@motorola.com Jian Lou, lou@motorola.com |

# Tools under Test

The following tools will be considered.

| Proponent | Document | Tool description | Configurations/ Versions | Cross-checker |
| --- | --- | --- | --- | --- |
| HHI | JCTVC-D336 | Coding of transform coefficient levels with Golomb-Rice codes | Ver.1 (sent 2/23/11) | RIM, TI |
|  |  |  | Ver.2 (sent 3/7/11) | TI, |
| Panasonic | JCTVC-D185 | Simplified Context modeling for Transform Coefficient Coding | N1 | Qualcomm |
|  |  |  | N2 | Qualcomm |
|  |  |  | N1N2 | Qualcomm |
| Qualcomm | JCTVC-D262 | Context size reduction for the significance map |  | Panasonic, Sony |
| Huawei | JCTVC-D311 | Adaptive coefficients scanning for inter-frame coding | Proposal 1 | HHI |
|  |  |  | Proposal 2 | HHI |
|  |  |  | Proposal 3 (sent 3/11/11) |  |
| Qualcomm | JCTVC-D374 | Adaptive Coefficient Scanning for LCEC | Ver.1 (sent 2/28/11) | Motorola, Sharp |
|  |  |  | Ver.2 (sent 3/6/11) | Motorola, Sharp |

# List of Contributions

| Proposal Document # | Cross-Verification Document # | Author(s) | Title |
| --- | --- | --- | --- |
| JCTVC-D336 (JCTVC-E253) | JCTVC-E121 | J. Zan, D. He (RIM) | CE11: Cross-verification of HHI's (JCTVC-D336) Reduced-Complexity Entropy Coding of Transform Coefficient Levels |
|  | JCTVC-E326 | V. Sze (TI) | CE11: Cross-verification of HHI's (JCTVC-D336) Coding of transform coefficient levels with Golomb-Rice codes |
| JCTVC-D185 (JCTVC-E227) | JCTVC-E403 | J. Sole (Qualcomm) | CE11: Cross-check of Panasonic’s context size reduction for the significance map (JCTVC-E227) by Qualcomm |
| JCTVC-D262 (JCTVC-E338) | JCTVC-E164 | Hisao Sasai, Takahiro Nishi (Panasonic), | CE11: Cross-check report for Qualcomm's proposal JCTVC-D262 on Parallel Context Processing for the significance map |
|  | JCTVC-E300 | Cheung Auyeung (Sony) | CE11: Cross check of Qualcomm JCTVC-D262 on parallel context processing for the significance map in high coding efficiency |
| JCTVC-D311 (JCTVC-E296) | JCTVC-E150* | J. Xu (Microsoft) | CE11.B: Cross-check report for Huawei's adaptive coefficient scanning |
|  | JCTVC-E255 | T. Nguyen (HHI) | CE11: Cross-check JCTVC-D311 - Adaptive coefficients scanning for inter-frame coding |
| JCTVC-D374 (JCTVC-E392) | JCTVC-E100 | C. Yeo (I2R) | CE11: Cross-check of Qualcomm’s proposal on adaptive scans |
|  | JCTVC-E132 | Y.Yasugi, T. Yamamoto(SHARP) | CE11: Cross-check report of Qualcomm’s Adaptive Coefficient Scanning for LCEC (JCTVC-D374) by SHARP |
|  | JCTVC-E357 | J. Lou, K. Panusopone, L. Wang (Motorola Mobility) | CE11: Cross-check report from Motorola Mobility for Qualcomm's adaptive coefficient scanning for LCEC (JCTVC-E392) |

* no document available at time of upload

# Definition of Performance Measurement Criteria

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

## Coding Performance Measurements

Objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate [1] for all tools in section 3. Proposals in sub experiment CE. B relating to coefficient scanning methods should be tested with both high efficiency and low complexity configurations.

## Complexity Measurements

For the complexity measurement, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the tool implemented.

However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft. The impact on optimized implementations in hardware and software are taken into account.

Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered.

As the number of bin impact overall throughput, the following needs to be reported

- the total number of bins and bin reduction/increase due to proposed technique

- a breakdown of the number of bins coded in regular and bypass mode for CABAC

- a breakdown of the number of bins for sig flag, last flag and absolute coefficient level

The size of the context memory (i.e. number of contexts used) should be reported.

In addition, for sub experiment CE.B

Scheme of selecting the scan pattern during encoding needs to be described.

Scheme of updating the scan pattern during parsing needs to be described.

# Summary of Results

## Sub Experiment CE.A: Context Modeling/Selection and Binarization for Transform Coefficient related syntax elements

### Coding Efficiency

High Efficiency Configurations

| Proposal Document # | Intra | Random Access | Low Delay |
| --- | --- | --- | --- |
| JCTVC-D336 (ver. 1) | 0.02 | 0.03 | -0.02 |
| JCTVC-D336 (ver. 2) | -0.05 | -0.01 | -0.05 |
| JCTVC-D185 (N1) | -0.04 | -0.08 | -0.22 |
| JCTVC-D185 (N2) | 0.08 | 0.08 | 0.02 |
| JCTVC-D185 (N1N2) | 0.03 | 0.00 | -0.14 |
| JCTVC-D262 | -0.03 | 0.01 | -0.10 |

Each tool was compared to the HM-2.0 (anchor).

### Bin Statistics

#### Syntax Element

| Proposal Document # | Intra |  |  | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | % sig | % last | % coeff | % sig | % last | % coeff | % sig | % last | % coeff |
| ANCHOR | 29% | 14% | 24% | 26% | 10% | 16% | 27% | 9% | 12% |
| JCTVC-D336 (ver. 1) | 29% | 14% | 23% | 26% | 10% | 15% | 27% | 9% | 12% |
| JCTVC-D336 (ver. 2) | 29% | 14% | 23% | 26% | 10% | 15% | 27% | 9% | 12% |
| JCTVC-D185 (N1) | 29% | 14% | 24% | 26% | 10% | 16% | 27% | 9% | 12% |
| JCTVC-D185 (N2) | 29% | 14% | 24% | 26% | 10% | 16% | 27% | 9% | 12% |
| JCTVC-D185 (N1N2) | 29% | 14% | 24% | 26% | 10% | 16% | 27% | 9% | 12% |
| JCTVC-D262 | 29% | 11% | 25% | 26% | 9% | 16% | 26% | 10% | 11% |

#### Method of Coding

| Proposal Document # | Intra |  | Random Access |  | Low Delay |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | % reg | % bypass | % reg | % bypass | % reg | % bypass |
| ANCHOR | 86% | 14% | 86% | 13% | 88% | 11% |
| JCTVC-D336 (ver. 1) | 81% | 19% | 84% | 15% | 87% | 12% |
| JCTVC-D336 (ver. 2) | 81% | 19% | 84% | 15% | 87% | 12% |
| JCTVC-D185 (N1) | 86% | 14% | 86% | 13% | 88% | 11% |
| JCTVC-D185 (N2) | 86% | 14% | 86% | 13% | 88% | 11% |
| JCTVC-D185 (N1N2) | 86% | 14% | 86% | 13% | 88% | 11% |
| JCTVC-D262 | 85% | 15% | 86% | 13% | 88% | 11% |

#### Bin Count Increase (%)

| Proposal Document # | Intra | Random Access | Low Delay |
| --- | --- | --- | --- |
| JCTVC-D336 (ver. 1) | -1 | 0 | 0 |
| JCTVC-D336 (ver. 2) | -1 | 0 | 0 |
| JCTVC-D185 (N1) | 0 | 0 | 0 |
| JCTVC-D185 (N2) | 0 | 0 | 0 |
| JCTVC-D185 (N1N2) | 0 | 0 | 0 |
| JCTVC-D262 | -5 | -2 | 0 |

### Number of Contexts

JCTVC-D336 (nothing reported)

JCTVC-D185 (N1N2)

| increase number of context | increase sig % | increase last % | increase coeff % | increase total % |
| --- | --- | --- | --- | --- |
| Intra only | -22.4% | -73.1% | 0.0% | -27.3% |
| Random access | -22.4% | -73.1% | 0.0% | -24.4% |
| Low Delay | -22.4% | -73.1% | 0.0% | -24.4% |

JCTVC-D185 (N1)

| increase number of context | increase sig % | increase last % | increase coeff % | increase total % |
| --- | --- | --- | --- | --- |
| Intra only | -22.4% | 0.0% | 0.0% | -6.6% |
| Random access | -22.4% | 0.0% | 0.0% | -5.9% |
| Low Delay | -22.4% | 0.0% | 0.0% | -5.9% |

JCTVC-D185 (N2)

| increase number of context | increase sig % | increase last % | increase coeff % | increase total % |
| --- | --- | --- | --- | --- |
| Intra only | 0.0% | -73.1% | 0.0% | -20.8% |
| Random access | 0.0% | -73.1% | 0.0% | -18.6% |
| Low Delay | 0.0% | -73.1% | 0.0% | -18.6% |

JCTVC-D262

| increase number of context | increase sig % | increase last % | increase coeff % | increase total % |
| --- | --- | --- | --- | --- |
| Intra only | 58.9% | 15.4% | 0.0% | 21.6% |
| Random access | 58.9% | 15.4% | 0.0% | 19.3% |
| Low Delay | 58.9% | 15.4% | 0.0% | 19.3% |

### Encoding/Decoding Time

| Proposal Document # | Intra |  | Random Access |  | Low Delay |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoding Time | Decoding Time | Encoding Time | Decoding Time | Encoding Time | Decoding Time |
| JCTVC-D336 (ver. 1) | 104% | 103% | 103% | 106% | 103% | 109% |
| JCTVC-D336 (ver. 2) | 99% | 98% | 100% | 97% | 99% | 100% |
| JCTVC-D185 (N1) | 100% | 100% | 101% | 99% | 101% | 98% |
| JCTVC-D185 (N2) | 100% | 99% | 100% | 99% | 100% | 99% |
| JCTVC-D185 (N1N2) | 100% | 99% | 100% | 98% | 100% | 97% |
| JCTVC-D262 | 100% | 101% | 100% | 99% | 98% | 102% |

## Sub Experiment CE.B: Coefficient Scanning Methods

### Coding Efficiency

| Proposal Document # | Configuration | Intra | Random Access | Low Delay |
| --- | --- | --- | --- | --- |
| JCTVC-D311 (proposal 1) | High Efficiency | N/A | -0.3 | -1.0 |
| JCTVC-D311 (proposal 2) | High Efficiency | N/A | -0.3 | -0.8 |
| JCTVC-D374 (ver. 1) | Low Complexity | -1.0 | -0.4 | 0.0 |
| JCTVC-D374 (ver. 2) | Low Complexity | -1.0 | -0.4 | 0.0 |

Each tool was compared to the HM-2.0 (anchor).

### Bin Statistics

#### Syntax Element

| Proposal Document # | Intra |  |  | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | % sig | % last | % coeff | % sig | % last | % coeff | % sig | % last | % coeff |
| ANCHOR (HE) | 29% | 14% | 24% | 26% | 10% | 16% | 27% | 9% | 12% |
| JCTVC-D311 (proposal 1) | N/A | N/A | N/A | 24% | 10% | 10% | 25% | 9% | 9% |
| JCTVC-D311 (proposal 2) | N/A | N/A | N/A | 23% | 10% | 10% | 23% | 9% | 9% |
| ANCHOR (LC) |  |  |  |  |  |  |  |  |  |
| JCTVC-D374 (ver. 1) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| JCTVC-D374 (ver. 2) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

#### Method of Coding

| Proposal Document # | Intra |  | Random Access |  | Low Delay |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | % reg | % bypass | % reg | % bypass | % reg | % bypass |
| ANCHOR (HE) | 86% | 14% | 86% | 13% | 88% | 11% |
| JCTVC-D311 (proposal 1) | N/A | N/A | 86% | 13% | 88% | 12% |
| JCTVC-D311 (proposal 2) | N/A | N/A | 86% | 13% | 87% | 12% |
| ANCHOR (LC) |  |  |  |  |  |  |
| JCTVC-D374 (ver. 1) | N/A | N/A | N/A | N/A | N/A | N/A |
| JCTVC-D374 (ver. 2) | N/A | N/A | N/A | N/A | N/A | N/A |

#### Bin Count Increase (%)

| Proposal Document # | Intra | Random Access | Low Delay |
| --- | --- | --- | --- |
| JCTVC-D311 (proposal 1) | N/A | 0% | -2% |
| JCTVC-D311 (proposal 2) | N/A | -1% | -3% |
| JCTVC-D374 (ver. 1) | N/A | N/A | N/A |
| JCTVC-D374 (ver. 2) | N/A | N/A | N/A |

### Number of Contexts

Nothing Reported

### Encoding/Decoding Time

| Proposal Document # | Intra |  | Random Access |  | Low Delay |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoding Time | Decoding Time | Encoding Time | Decoding Time | Encoding Time | Decoding Time |
| JCTVC-D311 (proposal 1) | N/A | N/A | 102% | 103% | 102% | 102% |
| JCTVC-D311 (proposal 2) | N/A | N/A | 106% | 101% | 106% | 101% |
| JCTVC-D374 (ver. 1) | 102% | 102% | 101% | 99% | 98% | 99% |
| JCTVC-D374 (ver. 2) | 101% | 101% | 101% | 99% | 96% | 101% |

# Summary of Tools

## Sub Experiment CE.A: Context Modeling/Selection and Binarization for Transform Coefficient related syntax elements

### JCTVC-D336/JCTVC-E253: "Coding of transform coefficient levels with Golomb-Rice codes ", HHI

JCTVC-D336 proposes a reduced-complexity method for the coding of absolute transform coefficient levels by increasing the number of bins coded in bypass mode of CABAC/PIPE. The main underlying idea of this proposal is to allow the mixing of structured VLCs and PIPE/CABAC coded bits. Compared to the current method implemented in HM 1.0, the same coding efficiency is achieved while the computational complexity is reduced, especially for the high bitrate case. Also, the upper limit on the number of bins to be parsed by PIPE/CABAC can be reduced by at least a factor of 3 compared to the current method.

### JCTVC-D185/JCTVC-E227: "Context size reduction for the significance map", Panasonic

JCTVC-D185 proposes that a simplified context modeling for significant map and last flag. With the simplification of the context model, some context is shared with several transform block sizes and neighborhood of coefficient position. This core experiment will verify the coding efficiency of the simplification of context model and complexity impact.

### JCTVC-D262/JCTVC-E338: "Parallel Context Processing for the significance map in high coding efficiency", Qualcomm

JCTVC-D262 proposes a technique for the parallelization of context processing. The method separates syntax elements for parallel processing and simplifies contexts. Significance coefficient flag and last significant coefficient flag are separated by encoding the position of the last significant coefficient before the position of the other significant coefficients within a block.

## Sub Experiment CE.B: Coefficient Scanning Methods

### JCTVC-D311/ JCTVC-E296: "Adaptive coefficients scanning for inter-frame coding", Huawei

JCTVC-D311 proposes an adaptive scanning method for inter blocks. Three scanning orders are used, and for every Transform Unit (TU) the best scanning order is chosen based on the texture direction of reference block. The encoder and decoder follow the same rules to decide the best scanning order, so explicit signaling is avoided.

### JCTVC-D374/JCTVC-E392: “Adaptive Coefficient Scanning for LCEC”, Qualcomm

In adaptive scans three scans are used: horizontal, vertical and zigzag. The scan order for first few coefficients (up to 64) is adaptively adjusted based on previously coded coefficients (as described in the document JCTVC-C250). For large blocks with a size bigger than 16x16, they share the same scan with 16x16 block.

# Summary of Modifications for CE11 Proposals

- D336/E253 (HE)

  - Modify binarization of coefficient levels

  - Change number of context-coded binary symbols

- D185/E227 (HE)

  - Change number of contexts for significant_coeff_flag and last_significant_coeff_flag

- D262/E338 (HE)

  - Modify method of signaling last significant coefficient information: Transmit location (X,Y) of last significant coefficient (instead of last_significant_coeff_flag) before all significant_coeff_flag

  - Modify context selection of first 10 significant_coeff_flag

- D311/E296 (HE)

  - For Inter CU, select one of three transform coefficient scanning method (zig-zag, vertical, horizontal) based on pixel texture for each block

  - New zig-zag scan proposed

- D374/E392 (LC)

  - For Intra CU in LCEC, change transform coefficient scanning for each coefficient position based on statistics (counter adaptation)

# References

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.