| 7 Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E032_r2 WG11 Number: m19649 |
| --- | --- |

| Title: | CE12: Summary report of core experiment on deblocking filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin1 Byeungwoo Jeon2 Matthias Narroschke3 | Email: | andrey.norkin@ericsson.com bjeon@skku.edu matthias.narroschke@eu.panasonic.com |
| Source: | CE coordinators (Ericsson1, SKKU2, Panasonic3) |  |  |

_____________________________

# Introduction

This contribution is a summary report on Core Experiment 12 (deblocking filtering). There are two subsets in CE12. Subset 1 comprises proposals aimed at improving subjective and objective performance of deblocking filtering. Subset 2 comprises approaches that address parallelization aspects of deblocking filtering. Subset 1 contains four proposals and Subset 2 contains two. All the proposals are based on HM2.0 and experiments are performed according to the common test conditions provided in JCTVC-D600. All results are verified by cross-checkers.

# Document list

- Summary document (this document)

- JCTVC-E032 [CE coordinators] CE12: Summary report of core experiment on deblocking filtering.

- Proponents and cross-verification documents

Subset 1: Deblocking filters improving objective or/and subjective performance

- JCTVC-E079 [MediaTek] CE12.1: Improved Deblocking Filter.

  - Cross-check: JCTVC-E071 [Sony] CE12 Subset1: Cross-verification of MediaTek's proposal JCTVC-E079

  - Cross-check: JCTVC-E256 [Ericsson] CE12 subset 1: Cross verification of MediaTek’s deblocking by Ericsson

- JCTVC-E144 [USTC, Microsoft] CE12 Subset 1: Deblocking for large size blocks.

  - Cross-check: JCTVC-E311 [MediaTek] CE12 Subset1: Cross-verification of Microsoft's proposal JCTVC-E144

- JCTVC-E417 [SK Telecom and Sungkyunkwan University] CE12 Subset1: SKT/SKKU Deblocking Filter (Version 2 of the proposal was a late contribution after the meeting had started).

  - Cross-check of v1: JCTVC-E151 [Microsoft] CE12: Cross-check report for SKKU's deblocking

  - Cross-check of v2: JCTVC-E467 [I2R] CE12 Subset1: Cross-check of SKT/SKKU’s proposal on Deblocking Filter (JCTVC-E417) – Late contribution (non-CE cross-checker).

Note: The change made in version 2 is summarized in the revised version of JCTVC-E417.

- JCTVC-E276 [Ericsson] CE12.1: Ericsson deblocking filter

  - Cross-check: JCTVC-E418 [SKT/SKKU] CE12 Subset1: Cross-verification Results of Ericsson Deblocking Filter (JCTVC-E276) by SKT/SKKU

  - Cross-check: JCTVC-E171 [Toshiba] CE12: Verification results of Ericsson's Proposal JCTVC-E276

  - Cross-check: JCTVC-E270 [Samsung] CE12: Cross-verification of the deblocking filter from Ericsson (JCTVD-E276) by Samsung

  - Cross-check: JCTVC-E310 [MediaTek] CE12 Subset1: Cross-verification of Ericsson's proposal JCTVC-E276

Subset 2: Parallel deblocking filters

    - JCTVC- E181 [Sony] CE12.2: Parallel deblocking filter.

  - Cross-check: JCTVC-E257 [Panasonic] CE12: Cross-check results of the parallel deblocking filter of Sony (JCTVC-E181)

  - Cross-check: JCTVC-E314 [Qualcomm] CE12 Subset 2: Cross-Verification of Sony’s Parallel Deblocking Contribution (JCTVC-E181) by Qualcomm

    - JCTVC- E224 [Panasonic] CE12.2: Parallel filter decision for de-blocking.

  - Cross-check: JCTVC-E070 [Sony] CE12 Subset2: Cross-verification of Panasonic's proposal JCTVC-E224

  - Cross-check: JCTVC-E312 [Qualcomm] CE12 Subset 2: Cross-Verification of Panasonic’s Parallel Deblocking Contribution (JCTVC-E224) by Qualcomm

# Cross-check assignment

Table 1 Subset 1 – cross-checkers

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| MediaTek | Ericsson, Sony |
| SKT/SKKU | Microsoft, I2R |
| Microsoft | MediaTek |
| Ericsson | SKT/SKKU, Toshiba, Samsung, MediaTek |

Table 2 Subset 2 – cross-checkers

| Proponent(s) | Cross/checker(s) |
| --- | --- |
| Sony | Panasonic, Qualcomm |
| Panasonic | Sony, Qualcomm |

# Deblocking filter modifications

## Subset 1: Deblocking filters improving objective or/and subjective performance

Table 3 Parts that have been modified compared to HM2.0

| Proponent | MediaTek(luma and chroma) | MediaTek(chroma) | Microsoft | SKKU/SKT | Ericsson |
| --- | --- | --- | --- | --- | --- |
| Luma filter (weak) | Yes | No | No | Yes | Yes |
| Luma filter (strong) | No | No | Yes | No | No |
| Decision weak/strong filter | Yes | No | No for mode A (a criterion for Mode B added) | No | No |
| Chroma filter | Yes | Yes | Yes | No | No |
| Luma boundary deblocking decision | Yes | No | No | No | No |
| Chroma boundary deblocking decisions | Yes | Yes | No | Yes | Yes |
| Bs derivation | Yes | Yes | No | Yes | Chroma modified |
| Tc_values | Yes | Yes | No | Yes | No |
| Beta values | Yes | Yes | No | No | No |
| Optimization of orig. code parts | Yes (Luma and chroma are processed in the same loop) | Yes (Luma and chroma are processed in the same loop) | No | No | No |
| Remarks |  |  |  |  | Number of pixels modified by luma filter is one, two, three or zero |

## Subset 2: Parallel deblocking filters

Table 4 Parts that have been modified compared to HM2.0

| Proponent | Sony item 2 | Sony item 3 | Panasonic |
| --- | --- | --- | --- |
| Filtering parallelization (CU base) | Yes, First all filtering for vertical edges in parallel then all filtering for horizontal edges in parallel. Filtering for horizontal edge at 3 pixels at right CU border is applied in the next CU | Yes, First all filtering for vertical edges in parallel then all filtering for horizontal edges in parallel. Filtering for horizontal edge at 3 pixels at right CU border is applied in the next CU | Yes, First all filtering for vertical edges in parallel then all filtering for horizontal edges in parallel |
| Filtering parallelization (frame base) | Yes, First all filtering for vertical edges in parallel then all filtering for horizontal edges in parallel | Yes, First all filtering for vertical edges in parallel then all filtering for horizontal edges in parallel |  |
| Decision parallelization (CU base) | Yes, ON/OFF decision can be done in parallel with modification of H/V filtering order. | Yes, in addition item 2, strong/weak decision can be done in parallel with modification of H/V filtering order | Yes, All decisions for vertical and horizontal edges in parallel |
| Decision parallelization (frame base) | Yes, First all decisions for vertical edges in parallel then all decisions for horizontal edges in parallel | Yes, First all decisions for vertical edges in parallel then all decisions for horizontal edges in parallel |  |
| Modification of luma decisions | Yes (Order is changed) | Yes (Order is changed, individual decisions for each line/column) | Yes (For a current CU to be deblocked, all decisions are performed based on the unfiltered signal of this CU) |
| Chroma decisions | No | No | No |
| Optimization of orig. code parts | No | No | No |
| Remarks | Software does not exploit the benefit of increased parallel processing capabilities | Software does not exploit the benefit of increased parallel processing capabilities | Software does not exploit the benefit of increased parallel processing capabilities |

# Experimental results

Based on common test conditions [1], CE12 defines all six test cases mandatory both Subset 1 and Subset 2 proposals. The following evaluation criteria are used:

- BD-rate relative to the anchor(s)

- Complexity (decoding time measurement)

- Subjective video quality

In the tables below, the following abbreviations are used:

IO – intra only

LD – low delay

RA – random access

## Subset 1: Deblocking filters improving objective or/and subjective performance

### Coding efficiency

Table 5 BD-Rate (Y) for Subset 1

| Proponent | MediaTek (luma and chroma) |  |  | MediaTek (chroma) |  |  | Microsoft |  |  | SKT/SKKU (v1) |  |  | SKT/SKKU (v2) Late contribution (Non-CE cross-checker) |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complex. | -1.5 | -1.2 | -1.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -1.1 | -0.6 | -0.5 | -1.1 | -1.0 | -0.9 | -0.9 | -1.2 | -0.9 |
| High Effic. | -1.3 | -0.9 | -0.9 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | -1.2 | -1.3 | -0.9 | -1.2 | -1.4 | -1.0 | -0.7 | -1.5 | -0.9 |

Table 6 BD-Rate (U) for Subset 1

| Proponent | MediaTek (luma and chroma) |  |  | MediaTek (chroma) |  |  | Microsoft |  |  | SKT/SKKU (v1) |  |  | SKT/SKKU (v2) Late contribution (Non-CE cross-checker) |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complex. | -3.0 | -1.8 | -1.8 | -2.9 | -1.1 | -2.2 | 0.0 | 0.0 | 0.0 | -0.4 | 0.9 | 2.2 | -0.5 | -0.3 | -0.7 | 0.0 | -1.0 | 0.2 |
| High Effic. | -1.4 | -0.7 | -1.8 | -1.5 | -0.6 | -1.8 | -0.1 | 0.0 | 0.0 | -0.5 | 0.8 | 1.5 | -0.6 | -0.4 | -0.6 | 0.0 | -1.4 | -0.2 |

Table 7BD-Rate (V) for Subset 1

| Proponent | MediaTek (luma and chroma) |  |  | MediaTek (chroma) |  |  | Microsoft |  |  | SKT/SKKU (v1) |  |  | SKT/SKKU (v2) Late contribution (Non-CE cross-checker) |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complex. | -3.4 | -1.9 | -1.9 | -3.2 | -1.0 | -2.2 | 0.0 | 0.0 | 0.0 | -0.4 | 1.7 | 2.2 | -0.4 | -0.1 | -0.6 | 0.0 | -0.7 | 0.2 |
| High Effic. | -1.6 | -0.9 | -0.7 | -1.8 | -0.8 | -2.0 | -0.1 | 0.0 | -0.1 | -0.5 | 1.0 | 1.4 | -0.6 | -0.3 | -0.6 | 0.0 | -1.4 | -0.2 |

### Complexity – relative decoding time against the anchor -

Table 8 Summary of relative decoding time (%) for Subset 1 (against the anchor)

| Proponent | MediaTek (luma and chroma) |  |  | MediaTek (chroma) |  |  | Microsoft |  |  | SKT/SKKU (v1) |  |  | SKT/SKKU (v2) Late contribution (Non-CE cross-checker) |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complex. | 100 | 100 | 101 | 100 | 99 | 100 | 102 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 99 | 100 | 101 | 101 |
| High Effic. | 99 | 99 | 100 | 99 | 100 | 100 | 101 | 100 | 100 | 100 | 100 | 98 | 101 | 100 | 99 | 100 | 98 | 100 |

### Comments from cross-checkers

MediaTek’s proposals E079

- Sony cross-checked MediaTek’s proposals and got a perfect match of the BD-rates to both parts of MediaTek’s proposal. The decoding times matched approximately. Cross-checker thinks that the difference in simulation time is due to simulation environment. There was a problem with compiling the proposal by a g++ compiler that was solved by removing the "-Werror" compiler option. This reportedly could be a problem of an HM2.0 version. The subjective quality has been evaluated on a subset of sequences. The comment from cross-checker was that the difference between MediaTek’s proposal and anchor is visible, for example, in woman’s face in Kimono1 sequence, but it is difficult to decide which one (the anchor or the proposal) is better.

- Ericsson cross-checked MediaTek’s proposal and got a perfect match of the BD-rates to both parts of MediaTek’s proposal. The decoding time matched approximately. Visual inspection by the cross-checker showed that the proposal with both luma and chroma changes looked slightly better than the anchor in a few more cases than it looked worse than the anchor. The chroma only modifications were reported to have a subjective quality similar to the anchor on average. It should be noted that the subjective differences are very small and requires very careful viewing with many repetitions.

Microsoft’s proposal E144

- MediaTek cross-checked Microsoft’s proposal and got a perfect match for BD-rates. The running time was reported to be very similar to HM2.0. Visual quality was reported to be similar to the anchor on average. In some cases, less block artifacts can be observed.

Ericsson’s proposal E276

- MediaTek cross-checked Ericsson’s proposal and got an exact match of BD-rates. The decoding time was reported to be similar to that provided by Ericsson. Subjective quality was also inspected and reported to be similar to the anchor on average.

- Toshiba confirmed Ericsson’s results. BD-rate matched perfectly. Decoding time was close to the proponent’s. The subjective picture quality of Ericsson’s proposal did not have significant differences from the anchor in the cross-checkers environment (CG-301W). The cross-checker cannot say which one was better.

- Samsung confirmed the results of Ericsson’s proposal. The simulation results are matched with BD-rate and decoder running time provided by proponent. Regarding subjective quality it was observed with provided deblocking filter that there is no evident visual quality difference for motion pictures, but if comparing frame by frame it is possible to find regions with better and worse visual quality comparing to the anchor.

- SKKU confirmed Ericsson’s result. BD-rates match and decoding time is similar.

SKKU’s proposal E417

- For version 1, Microsoft confirmed the objective results from SKKU’s. The encoding/decoding times compared with the anchors’ are also similar to those reported by SKK. No mismatch between encoder and decoder is found. The visual quality of SKKU’s proposal was compared with that of the anchor using a DELL 24inch monitor. For all test sequences, no significant improvement and degradation by the proposal were observed in the playback video. By comparing individual frames for anchor and proposal results, some accurate edge reconstruction was observed in some frames of BQMall (All Intra, HE,QP =37).

- For version 2, I2R has cross-checked proposal JCTVC-E417 and verified the results provided by SKKU.

## Subset 2: Parallel deblocking filters

### Coding efficiency

Table 9 BD-Rate (Y) for Subset 2

| Proponent | Panasonic |  |  | Sony item2 |  |  | Sony item3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.3 | -0.2 |
| High Efficiency | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.4 | -0.2 |

### Complexity – relative decoding time against the anchor

Table 10 Summary of relative decoding time (%) for Subset 2

| Proponent | Panasonic |  |  | Sony item2 |  |  | Sony item3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 100 | 100 | 101 | 100 | 102 | 101 | 102 | 102 | 102 |
| High Efficiency | 100 | 101 | 99 | 101 | 104 | 100 | 101 | 103 | 101 |

Table 11 Summary of relative decoding time (%) for Subset 2 measured on one PC reported by Quallcomm

| Proponent | Panasonic |  |  | Sony item2 |  |  | Sony item3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 102 | 103 | 102 | 102 | 102 | 101 | 103 | 103 | 102 |
| High Efficiency | 101 | 101 | 101 | 100 | 101 | 101 | 101 | 102 | 101 |

### Comments from cross-checkers

Sony’s proposals E181

- Panasonic confirmed Sony’s results. BD-rates matched perfectly, the decoding time matched approximately. The subjective quality of Sony’s proposals item 2 and item 3 is reported to be similar to one of the HM2.0.

- Qualcomm confirmed Sony’s results. RD results matched perfectly and the decoding time is very similar. There was a difference in encoding time, reportedly caused by usage of a heterogeneous computing cluster. The subjective quality based on informal viewing on a PC monitor of a subset of sequences is reported to be similar to the HM2.0 anchors.

Panasonic’s proposal E224

- Sony has confirmed cross-check results for Panasonic’s proposal. BD-rate results match perfectly. Simulation time was slightly different but consistent. Sony confirms Panasonic’s results. A subjective quality has been evaluated on a subset of sequences. It is reported that the proposal has no impact on subjective quality.

- Qualcomm confirmed results for Panasonic’s proposal. RD results matched perfectly, and the decoding times are very similar. There was a difference in encoding time, reportedly caused by usage of a heterogeneous computing cluster. The subjective quality based on informal viewing on a PC monitor of a subset of sequence is reported to be similar to one of the HM2.0.

# Subjective quality

A subjective quality test (informal viewing by experts) will be held during the Geneva meeting.

# Conclusions

In the CE eight proposals are evaluated based on test conditions (Sony and MediaTek have two proposals each). The results are summarized based on BD-rates, decoding time and modifications to the current HM2.0 deblocking. The informal subjective viewing by experts will be held in Geneva in order to assess how the proposals affect the subjective quality.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January 2011.

[2]	A. Norkin, B. Jeon, M. Narroschke et al., “Description of CE12: deblocking filtering,” JCTVC-D612, Daegu, January 2011.

Non-CE cross-checker.