| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E033 |
| --- | --- |

| Title: | CE13: Summary Report of Core Experiment on Sample Adaptive Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Authors: | Yu-Wen Huang | Email: | yuwen.huang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is a summary of Core Experiment 13 (CE13) on sample adaptive offset. In CE13, six different cases were tested. Each test was conducted by the proponent and a crosschecker. Common test configurations in JCTVC-D600 were used in all experiments. It is reported that sample adaptive offset achieves 0.5%, 1.3%, 2.2%, 0.7%, 1.8%, and 3.0% bit rate reductions for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD respectively with roughly unchanged encoding time and 1-3% decoding time increase. The decoding process of the sample adaptive offset is LCU-independent, which means the number of line buffers required on the decoder side is zero.

- Introduction

In CE13, sample adaptive offset has been studied with six cases, as shown in Table 1. The crosschecker assignment is different from the CE13 description in JCTVC-D613 [1] due to insufficient computing resource. During email discussions, all CE13 participants agreed to reduce the number of crosscheckers from two to one for each test case. Therefore, each test case was conducted by the proponent and one crosschecker.

Table 1  Test cases and cross-verification assignment

| Case | Software | Condition | Crosschecker |
| --- | --- | --- | --- |
| 1 | MediaTek based on HM2.0 | Band offset only | Qualcomm |
| 2 | MediaTek based on HM2.0 | Edge offset only | Qualcomm |
| 3 | MediaTek based on HM2.0 | Adaptation between band offset and edge offset | Sony |
| 4 | MediaTek based on HM2.0 | Band offset only, HE configurations only but ALF off | Sony |
| 5 | MediaTek based on HM2.0 | Edge offset only, HE configurations only but ALF off | Samsung |
| 6 | MediaTek based on HM2.0 | Adaptation between band offset and edge offset, HE configurations only but ALF off | Samsung |

# Document list

- Summary document

  - JCTVC-E033 [MediaTek] CE13: Summary report of core experiment on sample adaptive offset

- The proponent’s study document and corresponding crosscheck reports

  - JCTVC-E049[MediaTek] CE13: Sample adaptive offset with LCU-independent decoding

    - JCTVC-E332 [Qualcomm] CE13: Cross-verification of MediaTek’s adaptive offset

    - JCTVC-E385 [Sony] CE13: Cross checking of case 3 and case 4 of JCTVC-E049 on adaptive offset proposed by MediaTek

    - JCTVC-E389 [Samsung] CE13: Cross-verification for MediaTek's test (JCTVC-E049) by Samsung

# Simulation results

Experiments are described in JCTVC-D613, and the common test configurations in JCTVC-D600 [2] are used as the anchor.

## Band offset only

Table 2 reportedly shows the results of band offset only. The BD-rate reduction is 0.2-0.6%. The encoding time and decoding time are roughly unchanged.

Table 2  Results of band offset only

| Anchor | JCTVC-D600 |  |  |  |  |  | JCTVC-D600 ALF-off |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD | HE-AI | HE-RA | HE-LD |
| BD-Rate (%) | -0.2 | -0.3 | -0.5 | -0.2 | -0.3 | -0.5 | -0.2 | -0.3 | -0.6 |
| Enc. Time (%) | 99 | 99 | 99 | 99 | 100 | 100 | 100 | 100 | 101 |
| Dec. Time (%) | 100 | 100 | 99 | 101 | 102 | 101 | 101 | 100 | 101 |

## Edge offset only

Table 3 reportedly shows the results of band offset only. The BD-rate reduction is 0.4-1.3%. The encoding time is roughly unchanged. The decoding time is increased by 1-5%.

Table 3  Results of edge offset only

| Anchor | JCTVC-D600 |  |  |  |  |  | JCTVC-D600 ALF-off |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD | HE-AI | HE-RA | HE-LD |
| BD-Rate (%) | -0.4 | -0.9 | -1.3 | -0.4 | -1.0 | -1.2 | -0.6 | -1.3 | -0.9 |
| Enc. Time (%) | 100 | 100 | 99 | 99 | 100 | 100 | 101 | 100 | 100 |
| Dec. Time (%) | 101 | 104 | 103 | 103 | 102 | 105 | 103 | 102 | 103 |

## Adaptation between band offset and edge offset

Table 4 reportedly shows the results of adaptation between band offset and edge offset. The BD-rate reduction is 0.5-3.0%. The encoding time is roughly unchanged. The decoding time is increased by 1-3%.

Table 4  Results of adaptation between band offset and edge offset

| Anchor | JCTVC-D600 |  |  |  |  |  | JCTVC-D600 ALF-off |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration | HE-AI | HE-RA | HE-LD | LC-AI | LC-RA | LC-LD | HE-AI | HE-RA | HE-LD |
| BD-Rate (%) | -0.5 | -1.3 | -2.2 | -0.7 | -1.8 | -3.0 | -0.8 | -1.8 | -2.5 |
| Enc. Time (%) | 100 | 100 | 100 | 99 | 100 | 99 | 100 | 100 | 100 |
| Dec. Time (%) | 101 | 103 | 103 | 102 | 101 | 101 | 103 | 102 | 102 |

# Complexity issues

## Number of encoding passes

In JCTVC-E049, only one additional encoding pass is required. The sample adaptive offset (SAO) stage is located after the deblocking filter (DF) stage. During the DF pass, statistical data can be collected for SAO. Next, with a fast distortion estimation technique, all decisions of SAO can be made without accessing any pixel from picture memory buffers for adding offsets and calculating real distortion values. Finally, one extra pass is used to add offsets on deblocked pixels and output the results of SAO.

## Number of line buffers on the decoder side

In JCTVC-E049, the decoding process of SAO is LCU-independent. That is, the SAO decoding process of an LCU does not require any other LCU. This is done by skipping LCU boundary pixels for edge offset. In this way, the number of line buffers on the decoder side is zero.

# Conclusions

In CE13, one proposal was evaluated. It is reported that 0.5-3.0% bit rate reductions, roughly unchanged encoding time, 1-3% increase in decoding time, 1-pass encoding, and zero line buffers on the decoder side are achieved. Based on the observed results, it is suggested to review the results of the proposals, JCTVC-E049, with the JCT-VC committee and to make recommendation for adopting the proposed technologies in HM.

# References

- Y.-W. Huang, “Description of Core Experiment 13: Sample Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-D613, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.