| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E034 |
| --- | --- |

| Title: | CE14: Summary Report of Core Experiment 14 on Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shawmin Lei1 Marta Karczewicz 2 | e-mail : | shawmin.lei@mediatek.com martak@qualcomm.com |
| Source: | CE coordinators (MediaTek1, Qualcomm2) |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 14 (CE14) on intra mode coding. There are two Subtests described in CE14 description document, JCTVC-D614. Subtest 1 is the Most Probable Mode Signaling for Luma and Subtest 2 is Intra Mode Derivation with Multiple Predictor Sets (MPS) technology. There are three test reports from three proponents for Subtest 1, respectively. Each report was confirmed by at least two cross-checkers. Two of the three reports are exactly matched and bit-stream exchanges can be done. These two reports reported 0.5% BD-rate gain for both the high-efficiency and low-complexity intra only configurations. These two reports also reported essential the same encoding time and decoding time as those of anchor JCTVC-D600. The third report reported less BD-rate gain. The Subtest 2 was withdrawn by its proponent because of lack of gain.

# Introduction

This is a summary of Core Experiment 14 (CE14) on Intra Mode Coding. There are two Subtests in CE14, The Most Probable Mode Signaling for Luma (Subtest 1) and Intra Mode Derivation with Multiple Predictor Sets (MPS) technology (Subtest 2). The core experiment was conducted according to the CE description [2]. The software used in the CE is based on HM2.0 software. The performance is evaluated based on the common test conditions specified in JCTVC-D600 [1]. However, the emphasis is mainly on two all-intra (AI) configurations since the CE is on intra mode coding.

There are three independent implementations for the Subtest 1 by three proponents, S. Lei (MediaTek), TK Tan (NTT DOCOMO, Inc.), and W. Ding (Beijing University of Technology). The results from these three implementations are comparable and have all been confirmed by at least two cross-checkers.

The Subtest 2 has been implemented onto HM2.0 software. However, the test results have shown almost no gain. Thus, the proponent of Subtest 2 has decided withdrawing its proposal.

# Document list and cross-check assignment

- Summary document (this document)

  - JCTVC-E034 [S. Lei, M. Karczewicz  (CE coordinators)] CE14: Summary Report of Core Experiment 14 on Intra Mode Coding

- Proponent's and verification/study documents

Subtest 1: The Most Probable Mode Signaling for Luma

  - JCTVC-E088 [M. Guo, X. Guo, S. Lei (MediaTek)] CE14 Subtest 1: The Most Probable Mode Signaling for Luma

    - Cross-check: JCTVC-E105 [H. L. Tan, Y. H. Tan, C. Yeo (I2R)] CE14 Subtest 1: Cross-check report from Institute for Infocomm Research

    - Cross-check: JCTVC-E131 [TK Tan (DOCOMO)] CE14.1: Results for DOCOMO’s proposal and cross verification of MediaTek’s implementation for the most probable mode signalling for luma

  - JCTVC-E131 [TK Tan (DOCOMO)] CE14.1: Results for DOCOMO’s proposal and cross verification of MediaTek’s implementation for the most probable mode signalling for luma

    - Cross-check: JCTVC-E105 [H. L. Tan, Y. H. Tan, C. Yeo (I2R)] CE14 Subtest 1: Cross-check report from Institute for Infocomm Research

    - Cross-check: JCTVC-E268 [J. Chen (Samsung)] CE14.1: Cross-verification of the most probable mode signalling for luma (JCTVC-E131)

    - Cross-check: JCTVC-E313 [M. Guo, X. Guo (MediaTek)] CE14 Subtest 1: Cross-verification of NTTDOCOMO's proposal JCTVC-E131

  - JCTVC-E136 [W. Ding, X. Che, Y. Shi, B. Yin (BJUT)] CE14: Test Results for Intra Mode Coding by Multiple Mode Candidates

    - Cross-check: JCTVC-E105 [H. L. Tan, Y. H. Tan, C. Yeo (I2R)] CE14 Subtest 1: Cross-check report from Institute for Infocomm Research

    - Cross-check: JCTVC-E210 [A. Tanizawa, T. Shiodera (Toshiba)] CE14 subtest 1: Cross check report of BUT's proposal (JCTVC-E136) from Toshiba

# Experimental results

## Subtest 1: MediaTek Implementation (JCTVC-E088)

| MediaTek | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class B | -0.5 | -0.4 | -0.4 | -0.6 | -0.4 | -0.5 |
| Class C | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class E | -0.8 | -0.5 | -0.5 | -1.0 | -0.6 | -0.6 |
| All | -0.5 | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Subtest 1: NTT DOCOMO Implementation (JCTVC-E131)

| NTT DOCOMO | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class B | -0.5 | -0.4 | -0.4 | -0.6 | -0.4 | -0.5 |
| Class C | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class E | -0.8 | -0.5 | -0.5 | -1.0 | -0.6 | -0.6 |
| All | -0.5 | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

In JCTVC-E131, the four inter configurations were also tested. There was no significant impact for the random access and low delay configurations with overall average luma BD-rate improvements of 0.1% to 0.2% recorded. The encoding time and decoding time were essentially the same as those of the anchor.

## Subtest 1: Beijing University of Technology Implementation (JCTVC-E136)

| Beijing University of Technology | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class B | -0.4 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class E | -0.6 | -0.5 | -0.4 | -0.4 | -0.3 | -0.4 |
| All | -0.3 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions and Comments

In both JCTVC-E088 and JCTVC-E131, an overall average luma BD-rate of 0.5% gain was reported for both the high efficiency and low complexity intra only coding conditions. The decoding time was reported to be almost the same as that of anchor as well in both contributions. The results have been confirmed by two cross-checkers for each proponent.

The results reported in JCTVC-E136 were somewhat worse. After looking into JCTVC-E136’s code, difference found in RDO can explain the 0.2% of BD-rate difference for Intra-Only High-Efficiency configurations. The difference found in RDO and using no VLC in JCTVC-E136 can further explain the 0.4% of BD-rate difference for Intra-Only Low-Complexity configurations.

Based on overall results, it is suggested to review the results of the proposals in the JCTVC meeting in Geneva, and to make recommendation for adopting the proposed technology in CE14 Subset 1 into HM.

# References

[1]	“Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January 2011.

[2]	“CE14: Intra Mode Coding”, JCTVC-D614, Daegu, January 2011.