| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E042 |
| --- | --- |

| Title: | Unified End-Of-Slice Detection for LCEC and CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Chih-Wei Hsu Yu-Wen Huang Shawmin Lei | Emails: | cw.hsu@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution described a unified procedure of end-of-slice detection for both LCEC and CABAC. Similar to the concept used in JCTVC-D383 proposed by DOCOMO, the encoder sends only one end-of-slice bin after the last largest coding unit (LCU) in the slice when CABAC is used. However, unlike the idea of removing one RBSP stop bit in JCTVC-D383, in this proposal it is suggested to use two RBSP stop bits for CABAC as in the current HM. In this way, on the decoder side the look-ahead procedure of finding RBSP trailing bits for end-of-slice detection in LCEC can be fully reused for CABAC. The end-of-slice bin is decoded only when the RBSP trailing bits are recognized. With this modification, the end-of-slice detection for LCEC and CABAC is unified, which is the key point of this proposal and can reduce the design complexity of supporting both entropy coding modes. No coding efficiency loss is observed in all test cases.

- Introduction

In the current HEVC, a slice layer raw byte sequence payload (RBSP) contains slice header, slice data, and RBSP slice trailing bits, as shown in Figure 1, where the slice header is located at the beginning, the slice data is in the middle carrying information of coding units, and RBSP slice trailing bits are located at the end. The RBSP slice trailing bits first include an RBSP stop bit, which is always equal to 1 (as colored in red). After the RBSP stop bit, if byte alignment is achieved, the slice layer RBSP does not contain any more bit; otherwise, zeros are sent until byte alignment is reached. For the decoder to parse the slice data, two different methods are applied to detect the end of the slice depending on which entropy coder (LCEC or CABAC) is used. For LCEC case, the detection of the RBSP stop bit is always performed right after the last syntax element of each LCU is parsed. Then the end of the slice is inferred when the RBSP stop bit is found. As for CABAC case, in the current HM, one end-of-slice bin is transmitted for each LCU to indicate whether the slice is terminated, as shown in Figure 2.

Figure 1  Slice layer RBSP

Figure 2  End-of-slice flags for CABAC in the current HM

It is not necessary for CABAC to detect the RBSP stop bit as LCEC does. However, in JCTVC-D383 [1], it was pointed that multiple end-of-slice bins in a slice are redundant, and a method that only requires one end-of-slice bin was proposed. On the encoder side, instead of sending an end-of-slice bin after each LCU, only one end-of-slice bin is transmitted after the last LCU of the slice. In addition, unlike the current HM sending two RBSP stop bits, only one RBSP stop bit is sent and treated as slice data. On the decoder side, in order to decide whether to decode the end-of-slice bin, the number of remaining bits excluding the RBSP stop bit has to be checked. If the number of remaining bits is larger than zero, end-of-slice bin equal to zero is inferred. If the number of remaining bits is zero, end-of-slice bin is derived by checking the CABAC range. In this contribution, a similar concept that can reduce the number of end-of-slice bins is proposed. However, to reuse the RBSP stop bit detection procedure for LCEC, an extra CABAC stop bit in addition to the original RBSP stop bit is inserted right after the coding of end-of-slice bin for CABAC. That is, two stop bits are still used, as is the case in the current HM. With this modification, the end-of-slice detection for LCEC and CABAC can be unified.

The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section , and conclusions will be given in Section 4.

# Proposed method

The end-of-slice detection for LCEC requires looking ahead the bitstream to see if the next bit pattern matches the RBSP trailing bit(s). If the pattern is matched, the function, more_rbsp_data( ), returns false; otherwise, more_rbsp_data( ) returns true. For the CABAC case, the end-of-slice detection in the current HM is to decode the end-of-slice bin for each LCU. Therefore, more_rbsp_data( ) is not used. In the last JCT-VC meeting, JCTVC-D383 was proposed to reduce the number of end-of-slice bins to only one for CABAC; however, it was not adopted due to the calculation for the number of remaining bits. This is another bitstream look-ahead that has to be supported by the decoder besides more_rbsp_data( ), the end-of-slice detection for LCEC. Adding this complexity was not supported. In order to reduce the additional complexity of JCTVC-D383, we propose to use two stop bits for CABAC. As shown in Figure 3, one is the RBSP stop bit, and the other is the CABAC stop bit. The RBSP stop bit belongs to the RBSP trailing bits, and the CABAC stop bit belongs to the slice data. On the encoder side, only one end-of-slice bin is transmitted after the last LCU of the slice. In addition, the CABAC stop bit is inserted before the RBSP stop bit. On the decoder side, if the more_rbsp_data( ) returns true (the RBSP stop bit is not found), the end-of-slice bin equal to 0 will be inferred. If the more_rbsp_data( ) returns false (the RBSP stop bit is found), the end-of-slice bin will be derived as follows:

ruiBin = ( m_pcTComBitstream->isMoreRBSPData())

? 0

: (m_uiValue >= m_uiRange – 2);

where the isMoreRBSPData( ) is exactly the same as the more_rbsp_data( ) described before, the ruiBin denotes the decoded end-of-slice bin, and the m_uiValue and the m_uiRange denote the offset (low) value and the range value, respectively, in the CABAC decoding engine. With our proposed method, the function, more_rbsp_data( ), can be shared and reused for both LCEC and CABAC. Thus, the end-of-slice detection for LCEC and CABAC is unified.

Figure 3  Proposed bitstream for CABAC

# Simulation results

Experiments were conducted according to the common test configurations defined in JCTVC-D600 [2], and the software under test was based on HM2.0.

## Unified end-of-slice detection for LCU-aligned slices

In this experiment, the proposed unified solution to detect the end of a slice is tested in comparison with the current end-of-slice detection method in HM. The slice settings are LCU-aligned slices with 1500 target bytes. The simulation results are shown in Table 1 and Table 2. The bit rate increases of our proposed method are 4.9%, 5.5%, 4.4%, 4.3%, 2.6%, and 2.5% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively, while those of the current method in HM are 4.9%, 5.5%, 4.4%, 4.3%, 2.6%, and 2.5% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The coding efficiency of the proposed method is slightly better than the current HM for HE configurations (in which CABAC is used) and is the same as the current HM for LC configurations (in which LCEC is used).

## Unified end-of-slice detection for leaf-CU-aligned slices

In this experiment, the proposed unified solution to detect the end of a slice is tested in comparison with the 2-level end-of-slice signaling method in JCTVC-E043 [3]. The slice settings are leaf-CU-aligned slices with 1500 target bytes. The simulation results are shown in Table 3 and Table 4. The bit rate increases of our proposed method are 5.3%, 5.8%, 4.5%, 4.4%, 2.6%, and 2.5% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively, while those of the 2-level end-of-slice signaling are 5.3%, 5.8%, 4.6%, 4.4%, 2.7%, and 2.5% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The coding efficiency of the proposed method is slightly better than the 2-level end-of-slice signaling for HE configurations (in which CABAC is used) and is the same as the 2-level end-of-slice signaling for LC configurations (in which LCEC is used).

# Conclusions

In this contribution, a unified procedure to detect the end of a slice for both LCEC and CABAC was proposed. The function, more_rbsp_data( ), can be shared and reused for both LCEC and CABAC. In addition, only one end-of-slice bin for the last LCU is required for CABAC, and two stop bits are used as in the current HM. It is reported that no bit rate saving in HE cases for LCU-aligned slices and 0.1% bit rate saving in HE cases for leaf-CU-aligned slices can be achieved. The key point of this proposal is not improving coding efficiency but unifying the end-of-slice detection for LCEC and CABAC.

Table 1  Results of the proposed method for LCU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.4 |
| Class C | 5.3 | 4.6 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.4 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.7 | 6.5 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.7 | 9.0 | 5.4 | 5.9 | 6.7 |
| Class B | 4.7 | 5.8 | 5.5 | 4.8 | 5.0 | 4.7 |
| Class C | 4.2 | 4.9 | 4.9 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.6 | 4.3 | 4.3 | 4.5 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.5 | 3.6 | 3.7 | 3.5 |
| Class C | 2.8 | 5.0 | 5.0 | 2.8 | 2.6 | 2.3 |
| Class D | 1.7 | 2.5 | 2.4 | 1.8 | 1.6 | 2.0 |
| Class E | 1.7 | 2.4 | 1.5 | 1.2 | 3.1 | 3.0 |
| All | 2.6 | 4.6 | 3.9 | 2.5 | 2.8 | 2.7 |

Table 2  Results of the current HM method for LCU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.4 |
| Class C | 5.3 | 4.6 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.4 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.7 | 6.5 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.7 | 9.0 | 5.4 | 5.9 | 6.7 |
| Class B | 4.8 | 5.8 | 5.5 | 4.8 | 5.0 | 4.7 |
| Class C | 4.2 | 4.9 | 4.9 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.6 | 4.3 | 4.3 | 4.5 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.5 | 3.6 | 3.7 | 3.5 |
| Class C | 2.8 | 5.0 | 5.0 | 2.8 | 2.6 | 2.3 |
| Class D | 1.7 | 2.5 | 2.4 | 1.8 | 1.6 | 2.0 |
| Class E | 1.7 | 2.5 | 1.5 | 1.2 | 3.1 | 3.0 |
| All | 2.6 | 4.6 | 3.9 | 2.5 | 2.8 | 2.7 |

Table 3  Results of the proposed method for leaf-CU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.2 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.2 | 4.5 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.1 | 5.1 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 3.9 | 4.3 | 3.8 | 4.1 |
| Class E | 7.7 | 8.0 | 6.8 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 4.9 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.9 | 9.6 | 8.5 | 5.4 | 6.0 | 6.9 |
| Class B | 4.9 | 6.1 | 5.4 | 5.0 | 5.3 | 5.1 |
| Class C | 4.3 | 5.3 | 5.2 | 4.4 | 4.4 | 4.2 |
| Class D | 2.7 | 2.9 | 3.4 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.5 | 6.0 | 5.6 | 4.4 | 4.6 | 4.7 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.4 | 3.7 | 3.8 | 3.6 |
| Class C | 2.9 | 4.9 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.7 | 3.3 | 2.6 | 1.8 | 1.8 | 2.0 |
| Class E | 1.7 | 1.8 | 2.1 | 1.3 | 3.8 | 3.1 |
| All | 2.6 | 4.7 | 4.0 | 2.5 | 3.0 | 2.8 |

Table 4  Results of 2-level end-of-slice flags in JCTVC-E043 for leaf-CU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.3 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.3 | 4.6 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.2 | 5.2 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 4.0 | 4.3 | 3.8 | 4.1 |
| Class E | 7.8 | 8.1 | 6.9 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 5.0 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.0 | 9.7 | 8.6 | 5.4 | 6.0 | 6.9 |
| Class B | 5.0 | 6.2 | 5.5 | 5.0 | 5.3 | 5.1 |
| Class C | 4.4 | 5.4 | 5.3 | 4.4 | 4.4 | 4.2 |
| Class D | 2.9 | 3.0 | 3.5 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 6.1 | 5.7 | 4.4 | 4.6 | 4.7 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.7 | 7.3 | 5.5 | 3.7 | 3.8 | 3.6 |
| Class C | 3.0 | 5.0 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.8 | 3.4 | 2.7 | 1.8 | 1.8 | 2.0 |
| Class E | 1.9 | 1.9 | 2.3 | 1.3 | 3.8 | 3.1 |
| All | 2.7 | 4.8 | 4.1 | 2.5 | 3.0 |  |

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Simplification of End-of-slice Coding for Arithmetic Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-D383, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

- C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, and S. Lei, “CE4 Subset1: Leaf-CU-Aligned Slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-E043, Mar. 2011.