| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E044 |
| --- | --- |

| Title: | CE4 Subset2: Slice Boundary Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Chih-Wei Hsu Chia-Yang Tsai Yu-Wen Huang Shawmin Lei | Emails: | cw.hsu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the results of CE4 Subset2: slice boundary filter (SBF). For LCU-aligned 1500-byte slices, the bit rate saving of the SBF-on case is reportedly 0.1-0.3% over the SBF-off case when slice-independent deblocking filter (DF) and adaptive loop filter (ALF) are enabled. For two slices per picture, the bit rate saving of the SBF-on case is reportedly 0-0.1% over the SBF-off case when slice-independent DF and ALF are enabled. It is reported that the subjective quality differences between the SBF-on and the SBF-off cases can be easily observed.

- Introduction

In the current HEVC, one picture can be partitioned into multiple LCU-aligned slices. In the 4th JCT-VC meeting, JCTVC-D128 [1] proposed slice-independent DF and ALF where DF and ALF processes are not performed across slice boundaries in order to achieve parallel decoding of all slices. This concept was adopted and is to be integrated in HM2.1. In JCTVC-D128, a slice boundary filter (SBF) was also proposed. The SBF is useful only when slice-independent DF and ALF are used. The SBF can be regarded as the DF process applied on slice boundaries after slice-independent DF and ALF processes are done within slices. Subsequently, Core Experiment 4 (CE4) [2] was established for further study of slice-related issues, and the SBF is included in Subset2. This contribution will report the results for CE4 Subset2, the use of SBF when the slice-independent DF and ALF are applied. The rest of this document is organized as follows. The algorithm will be described in Section Error: Reference source not found, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

The SBF is very similar to the DF but only processes slice boundaries and is aimed to remove possible slice boundary artifacts caused by the slice-independent DF and ALF. The detailed description of the SBF can be found in JCTVC-D128. The only syntax change is to add one flag in the sequence parameter set to enable or disable the SBF when the slice-independent DF and ALF are used.

# Simulation results

The common test configurations in JCTVC-D600 [3] are used as anchors. The slice-independent DF and ALF, as well as the SBF, are added on top of the MediaTek’s software that supports both LCU-aligned and leaf-CU-aligned slices in CE4 Subset1, which is based on HM2.0.

## Slice boundary filter for 1500-byte slices

In this experiment, cases with SBF-on and SBF-off are tested. The slice-related settings are “LCU-aligned slices”, “1500 bytes for each slice”, and “slice-independent DF and ALF”. The simulation results are shown in Table 1 and Table 2. The bit rate increases of disabling SBF are 5.6%, 5.8%, 4.7%, 4.5%, 2.7%, and 2.7% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The corresponding bit rate increases of enabling SBF are 5.3%, 5.5%, 4.6%, 4.3%, 2.6%, and 2.5%. The bit rate saving of SBF-on over SBF-off is 0.1-0.3%.  and  provide subjective views for disabling and enabling the SBF, respectively.

Table 1  Results of disabling the SBF for 1500-bytes slices

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.9 | 5.2 | 6.0 | 5.2 | 5.9 | 6.1 |
| Class B | 5.6 | 5.7 | 6.4 | 6.0 | 6.7 | 6.9 |
| Class C | 5.7 | 5.9 | 6.3 | 6.0 | 6.8 | 6.9 |
| Class D | 3.9 | 4.5 | 4.8 | 4.1 | 4.4 | 4.8 |
| Class E | 8.6 | 10.7 | 9.1 | 8.3 | 12.7 | 11.6 |
| All | 5.6 | 6.2 | 6.4 | 5.8 | 7.0 | 7.0 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.5 | 10.7 | 10.0 | 5.7 | 6.5 | 7.3 |
| Class B | 5.2 | 6.9 | 6.4 | 5.2 | 5.8 | 5.3 |
| Class C | 4.4 | 5.6 | 5.8 | 4.4 | 4.7 | 4.7 |
| Class D | 2.7 | 3.3 | 3.6 | 2.6 | 2.5 | 2.8 |
| Class E |  |  |  |  |  |  |
| All | 4.7 | 6.7 | 6.4 | 4.5 | 4.9 | 5.0 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.8 | 7.4 | 5.8 | 3.9 | 4.0 | 3.6 |
| Class C | 3.0 | 5.2 | 5.2 | 2.9 | 2.8 | 2.4 |
| Class D | 1.7 | 2.9 | 2.6 | 1.9 | 1.8 | 2.2 |
| Class E | 1.7 | 2.0 | 2.1 | 1.3 | 3.5 | 3.8 |
| All | 2.7 | 4.7 | 4.2 | 2.7 | 3.1 | 3.0 |

Table 2  Results of enabling the SBF for 1500-bytes slices

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.8 | 4.0 | 4.8 | 4.9 | 4.8 | 5.1 |
| Class B | 5.2 | 4.3 | 4.8 | 5.5 | 5.4 | 5.4 |
| Class C | 5.5 | 4.7 | 4.8 | 5.8 | 5.6 | 5.5 |
| Class D | 3.8 | 3.5 | 3.6 | 4.0 | 3.4 | 3.6 |
| Class E | 7.8 | 8.3 | 7.0 | 7.6 | 10.4 | 9.6 |
| All | 5.3 | 4.7 | 4.9 | 5.5 | 5.7 | 5.6 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.4 | 9.8 | 9.3 | 5.4 | 5.8 | 6.7 |
| Class B | 5.0 | 6.2 | 5.5 | 4.8 | 5.1 | 4.7 |
| Class C | 4.3 | 5.0 | 5.2 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.8 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 6.0 | 5.7 | 4.3 | 4.4 | 4.5 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.8 | 7.1 | 5.5 | 3.6 | 3.7 | 3.4 |
| Class C | 3.0 | 5.0 | 5.0 | 2.8 | 2.6 | 2.2 |
| Class D | 1.7 | 2.4 | 2.4 | 1.8 | 2.0 | 2.2 |
| Class E | 1.7 | 2.4 | 1.6 | 1.4 | 3.1 | 2.9 |
| All | 2.6 | 4.5 | 3.9 | 2.5 | 2.9 | 2.7 |

Figure 1  Subjective view of BlowingBubbles_416x240, POC=7, QP=37, HE-AI when the SBF is disabled for 1500-byte slices

Figure 2  Subjective view of BlowingBubbles_416x240, POC=7, QP=37, HE-AI when the SBF is enabled for 1500-byte slices

## Slice boundary filter for 2 slices per picture

In this experiment, cases with SBF-on and SBF-off are tested. The slice-related settings are “LCU-aligned slices”, “two slices per picture”, and “slice-independent DF and ALF”. The simulation results are shown in Table 3 and Table 4. The bit rate increases of disabling SBF are 0.6%, 0.6%, 1.3%, 1.2%, 1.6%, and 1.4% for HE-AI, LC-AI, HE-RA, LC-RA, HE-LD, and LC-LD, respectively. The corresponding bit rate increases of enabling SBF are 0.5%, 0.6%, 1.3%, 1.1%, 1.6%, and 1.3%. The bit rate saving of SBF-on over SBF-off is 0-0.1%.  and  provide subjective views for disabling and enabling the SBF, respectively.

Table 3  Results of disabling the SBF for two slices per picture

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.4 | 0.2 | 0.3 | 0.3 |
| Class B | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Class C | 0.6 | 0.7 | 0.7 | 0.7 | 0.8 | 0.8 |
| Class D | 1.0 | 1.2 | 1.2 | 1.0 | 1.2 | 1.3 |
| Class E | 1.0 | 1.1 | 0.9 | 0.9 | 1.3 | 1.2 |
| All | 0.6 | 0.6 | 0.7 | 0.6 | 0.7 | 0.8 |

|  |  | HE-RA |  |  | LC-RA |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.1 | 0.9 | 0.7 | 0.5 | 0.9 |
| Class B | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.5 |
| Class C | 1.3 | 1.6 | 1.6 | 1.2 | 1.2 | 1.2 |
| Class D | 2.6 | 2.6 | 2.8 | 2.2 | 2.0 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.5 | 1.4 | 1.2 | 1.1 | 1.1 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.2 | 0.7 | 0.6 | 0.4 | 0.9 |
| Class C | 1.2 | 1.8 | 1.8 | 1.1 | 0.8 | 0.6 |
| Class D | 2.5 | 2.9 | 2.4 | 2.1 | 2.3 | 2.1 |
| Class E | 2.5 | 3.2 | 2.5 | 2.1 | 2.2 | 2.1 |
| All | 1.6 | 2.1 | 1.7 | 1.4 | 1.3 | 1.3 |

Table 4  Results of enabling the SBF for two slices per picture

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.4 | 0.2 | 0.3 | 0.3 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.3 |
| Class C | 0.6 | 0.5 | 0.5 | 0.7 | 0.7 | 0.6 |
| Class D | 1.0 | 0.9 | 0.9 | 1.0 | 0.9 | 1.0 |
| Class E | 0.9 | 0.8 | 0.7 | 0.8 | 1.0 | 1.0 |
| All | 0.5 | 0.5 | 0.5 | 0.6 | 0.6 | 0.6 |

|  |  | HE-RA |  |  | LC-RA |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.0 | 0.8 | 0.6 | 0.6 | 0.9 |
| Class B | 0.7 | 0.7 | 0.6 | 0.6 | 0.6 | 0.4 |
| Class C | 1.3 | 1.5 | 1.5 | 1.2 | 1.1 | 1.1 |
| Class D | 2.6 | 2.4 | 2.6 | 2.2 | 1.9 | 1.9 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.4 | 1.3 | 1.1 | 1.0 | 1.1 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.5 | 0.7 | 0.6 | 0.4 | 0.7 |
| Class C | 1.2 | 1.7 | 1.7 | 1.0 | 0.8 | 0.6 |
| Class D | 2.4 | 2.7 | 2.7 | 2.1 | 2.1 | 1.9 |
| Class E | 2.6 | 3.0 | 2.6 | 2.1 | 2.2 | 2.2 |
| All | 1.6 | 2.1 | 1.8 | 1.3 | 1.3 | 1.2 |

Figure 3  Subjective view of BlowingBubbles_416x240, POC=7, QP=37, HE-AI when the SBF is disabled for two slices per picture

Figure 4  Subjective view of BlowingBubbles_416x240, POC=7, QP=37, HE-AI when the SBF is enabled for two slices per picture

# Conclusions

In this contribution, CE4 Subset2 experiments were conducted to evaluate the slice boundary filter (SBF) when slice-independent DF and ALF are used. The bit rate saving of the SBF-on case is 0.1-0.3% over the SBF-off case. More importantly, the subjective quality of enabling the SBF is often observed to be better than that of disabling the SBF.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei, “Slice Boundary Processing and Picture Layer RBSP,” Document of Joint Collaborative Team on Video Coding, JCTVC-D128, Jan. 2011.

- Y.-W. Huang and I.-K. Kim, “Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity” Document of Joint Collaborative Team on Video Coding, JCTVC-D604, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.