| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E045 |
| --- | --- |

| Title: | CE4 Subset3: Slice Common Information Sharing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Chia-Yang Tsai Chih-Wei Hsu Yu-Wen Huang Shawmin Lei | Emails: | chiayang.tsai@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the results of CE4 Subset3: slice common information sharing. In the current HM, the ALF parameters are adapted at picture level. However, the entire ALF information is always sent in the first slice header even when a picture contains multiple slices, which is harmful for parallel slice decoding. In JCTVC-D128, for the sake of decoding multiple slices in parallel, it was proposed to add an option of moving ALF coefficients and slice common information to a picture-layer raw byte sequence payload (RBSP), e.g. the picture parameter set (PPS), and leaving ALF CU on/off flags in separate slice headers. Compared with the current syntax in HM, the slice common information sharing technique reportedly show 0.1% bit rate increase and 0.1% bit rate decrease for HE and LC configurations, respectively, with the JCTVC-D600 test configurations and 1500-byte slices.

- Introduction

In the current HEVC, one picture can be partitioned into multiple LCU-aligned slices with ALF parameters that can be adapted at picture level. Nevertheless, the entire ALF information is always sent in the first slice header even when a picture contains multiple slices, which is very unfriendly for parallel slice decoding. For example, if the first slice header is received at last, ALF processes for all other slices have to wait. In the 4th JCT-VC meeting, JCTVC-D128 [1] proposed slice-independent DF and ALF where DF and ALF processes are not performed across slice boundaries. This concept was adopted and is to be integrated in HM2.1. In JCTVC-D128, a slice common information sharing technique was also proposed. When the slice-independent DF and ALF and the slice common information sharing technique are all enabled at the same time, fully parallel decoding of multiple slices can be achieved. Subsequently, Core Experiment 4 (CE4) [2] was established for further study of slice-related issues, and the slice common information sharing is the focus of Subset3. This contribution will report the CE4 Subset3 results for the slice common information sharing technique. In the rest of this contribution, we may use “the sharing technique” to indicate “the common slice information sharing technique”. The rest of this document is organized as follows. The algorithm will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## Slice common information sharing

In HM-2.0-dev-slices, one picture can be partitioned into multiple LCU-aligned slices, and the ALF parameters are adapted at picture level. As Figure 1 shows, the encoded ALF information is allocated in the first slice header of the picture. This syntax design may cause huge decoding delay and problems for parallel slice decoding if out-of-order slices are received on the decoder side. Besides, the common information in different slice headers is redundant.

Figure 1  Bitstream structure of HM-2.0-dev-slices

The sharing technique is implemented on top of HM-2.0 by MediaTek, and it can support LCU-aligned slices. The new bitstream structure is illustrated in Figure 2. In order to remove redundancy, the common information is moved from multiple slice headers to the PPS. For better support of parallel slice decoding, the ALF coefficients are moved from the first slice header to the PPS, and each slice header contains its own ALF CU on/off flags. The sharing technique may suffer a little bit increase caused by distributing ALF CU on/off flags in all slice headers due to the byte alignment stuffing bits for ALF information in the slice header.

Figure 2  Bitstream structure of the common slice information sharing technique

## Syntax description

The proposed syntax is shown in Table 1, and the modified part is highlighted with yellow background. One flag, pps_use_more_pps_info_flag, is sent in the PPS. When a picture has only one slice, it is suggested to set pps_use_more_pps_info_flag as zero. The bitstream structure will fall back to what is used by the common test configurations in JCTVC-D600 [3] where no common information or ALF coefficients are sent in the PPS and ALF parameters are sent in the slice header. When a picture has multiple slices, it is suggested to set pps_use_more_info_flag as one. The bitstream structure in Figure 2 will be used.

Table 1  Proposed syntax modification

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 1 | ue(v) |
| seq_parameter_set_id | 1 | ue(v) |
| ... |  |  |
| pps_use_more_pps_info_flag | 1 | u(1) |
| if( pps_use_more_pps_info_flag ) { |  |  |
| pps_info( ) |  |  |
| pps_alf_param( ) |  |  |
| } |  |  |
| } |  |  |

| pps_info( ) { | C | Descriptor |
| --- | --- | --- |
| ... // non-ALF syntax elements that can be shared by multiple slices |  |  |
| } |  |  |

| pps_alf_param ( ) { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 1 | u(1) |
| if( adaptive_loop_filter_flag ) { |  |  |
| … // filter size, coefficients, …; filter on/off flags NOT included |  |  |
| } |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 2 | ue(v) |
| if( !pps_use_more_pps_info_flag ) { |  |  |
| slice_info( ) |  |  |
| slice_alf_param( ) |  |  |
| } |  |  |
| alf_cu_control_param( ) |  |  |
| } |  |  |

| slice_info( ) { | C | Descriptor |
| --- | --- | --- |
| ... // non-ALF syntax elements |  |  |
| } |  |  |

| slice_alf_param ( ) { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if( adaptive_loop_filter_flag ) { |  |  |
| … // filter size, coefficients, …; filter on/off flags NOT included |  |  |
| } |  |  |
| } |  |  |

| alf_cu_control_param ( ) { | C | Descriptor |
| --- | --- | --- |
| if( adaptive_loop_filter_flag ) |  |  |
| alf_cu_control_flag | 2 | u(1)¦ae(v) |
| if( alf_cu_control_flag) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v)¦ae(v) |
| num_of_alf_cu_control_flags | 2 | ue(v)¦ae(v) |
| for( i = 0; i < num_of_alf_cu_control_flags; i++ ) |  |  |
| alf_onoff_flag | 2 | u(1)¦ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

# Simulation results

The JCTVC-D600 test configurations are used as anchors. The sharing technique is implemented on top of HM-2.0 by MediaTek, and it can support LCU-aligned slices. In the conducted experiments, LCU-aligned 1500-byte slices are used. As shown in Table 2, two cases are tested according to the CE4 Subset3 description.

Table 2  Experiments to be conducted in CE4 Subset3

| Case | Condition |
| --- | --- |
| 1 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice common information sharing off |
| 2 | LCU=64x64, LCU-aligned, 1500 bytes per slice, slice common information sharing on |

In Case 1, the sharing technique is disabled. The bitstream structure in the HM-2.0-dev-slices is used, as shown in Figure 1. Table 3 shows the simulation results. When compared with the JCTVC-D600 anchor, bit rate increases are 4.9%, 4.4%, 2.6%, 5.5%, 4.3%, and 2.5% for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively.

Table 3  Results of CE4 Subset3 Case 1: Slice common information sharing disabled

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.4 |
| Class C | 5.3 | 4.6 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.4 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.7 | 6.5 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.7 | 9.0 | 5.4 | 5.9 | 6.7 |
| Class B | 4.8 | 5.8 | 5.5 | 4.8 | 5.0 | 4.7 |
| Class C | 4.2 | 4.9 | 4.9 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.6 | 4.3 | 4.3 | 4.5 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.5 | 3.6 | 3.7 | 3.5 |
| Class C | 2.8 | 5.0 | 5.0 | 2.8 | 2.6 | 2.3 |
| Class D | 1.7 | 2.5 | 2.4 | 1.8 | 1.6 | 2.0 |
| Class E | 1.7 | 2.5 | 1.5 | 1.2 | 3.1 | 3.0 |
| All | 2.6 | 4.6 | 3.9 | 2.5 | 2.8 | 2.7 |

In Case 2, the sharing technique is enabled. The bitstream structure is the same as shown in Figure 2. Table 4 shows the simulation results. When compared with the JCTVC-D600 anchor, bit rate increases are 4.8%, 4.6%, 2.9%, 5.3%, 4.2%, and 2.6% for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively.

The coding efficiency of enabling the common slice information sharing is essentially the same as that of disabling it. For HE, disabling the sharing technique is 0.1% better, but for LC, enabling the sharing technique is 0.1% better. Because ALF is disabled in LC configurations, the overhead of distributing ALF CU on/off flags in all slice headers does not exist for the sharing technique. Since the two cases have similar compression performance, the sharing technique is suggested to be enabled when multiple slices in a picture are used due to its friendliness for parallel slice decoding.

Table 4  Results of CE4 Subset3 Case 2: Slice common information sharing enabled

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.9 | 3.4 | 4.2 | 4.8 | 4.7 | 5.0 |
| Class B | 4.8 | 3.9 | 4.4 | 5.4 | 5.3 | 5.3 |
| Class C | 5.2 | 4.5 | 4.5 | 5.7 | 5.5 | 5.4 |
| Class D | 3.6 | 3.3 | 3.4 | 3.9 | 3.3 | 3.5 |
| Class E | 7.3 | 7.5 | 6.4 | 7.5 | 10.3 | 9.5 |
| All | 4.8 | 4.3 | 4.5 | 5.3 | 5.6 | 5.5 |

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.9 | 9.6 | 8.7 | 5.1 | 5.6 | 6.5 |
| Class B | 4.9 | 5.9 | 5.5 | 4.7 | 4.8 | 4.5 |
| Class C | 4.3 | 5.0 | 5.1 | 4.2 | 4.2 | 4.0 |
| Class D | 3.1 | 3.3 | 3.5 | 2.9 | 2.5 | 2.8 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 5.9 | 5.7 | 4.2 | 4.3 | 4.5 |

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.7 | 7.1 | 5.5 | 3.4 | 3.5 | 3.2 |
| Class C | 3.1 | 5.1 | 5.1 | 2.7 | 2.5 | 2.2 |
| Class D | 2.3 | 2.8 | 2.6 | 2.1 | 1.9 | 2.3 |
| Class E | 2.3 | 2.8 | 2.4 | 1.6 | 3.5 | 3.4 |
| All | 2.9 | 4.7 | 4.1 | 2.6 | 2.9 | 2.8 |

# Conclusions

In CE4 Subset3, MediaTek tested a technique of slice common information sharing that sends slice common information and ALF coefficients in the PPS and distributes ALF CU on/off flags in separate slice headers. In comparison with disabling the sharing technique as is used in the HM-2.0-dev-slices, enabling the sharing technique reportedly showed 0.1% coding loss in HE configurations and 0.1% coding gain in LC configurations. The sharing technique can solve the problem of parallel slice decoding encountered by the current HM when multi-slice pictures are used. It is suggested to adopt the sharing technique into HM but disable it by default when there is no slice partitioning for pictures.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, C.-Y. Chen, C.-M. Fu, and S. Lei, “Slice Boundary Processing and Picture Layer RBSP,” Document of Joint Collaborative Team on Video Coding, JCTVC-D128, Jan. 2011.

- Y.-W. Huang and I.-K. Kim, “Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity” Document of Joint Collaborative Team on Video Coding, JCTVC-D604, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.