| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E046 |
| --- | --- |

| Title: | CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Ching-Yeh Chen Chih-Ming Fu Chia-Yang Tsai Yu-Wen Huang Shawmin Lei | Emails: | chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the results of CE8 Subtest 2: adaptation between pixel-based and region-based filter selection, which was one of the coding tools jointly proposed by MediaTek, Qualcomm, and Toshiba in JCTVC-D119. In HM2.0, pixel-based filter selection is used for adaptive loop filtering (ALF). All pixels of a picture are classified into 16 groups, and after group merging one filter is applied for each group. In this proposal, a region-based filter selection method is added. When a picture chooses the region-based filter selection, the picture is divided into 16 roughly-equal-size regions where each region is aligned with largest coding unit (LCU) boundaries. Similar to the pixel-based method, regions can be merged, and after region merging one filter is applied for each region. The only syntax change is to add one flag in the slice header to switch between the pixel-based ALF and the region-based ALF. When compared with the JCTVC-D600 anchor, the proposed method reportedly achieved 0%, 0.2%, and 0.4% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively, with 1-2% encoding time increase and 1-4% decoding time decrease.

- Introduction

In JCTVC-D119 [1], MediaTek, Qualcomm, and Toshiba proposed an ALF solution, which contains several tools including filter unit (FU) concepts and adaptation among single-filter (SF), pixel-adaptation (PA), and adaptive-offset (AO) methods for each FU. Coming out of the 4th JCT-VC meeting, Core Experiment 8 (CE8) [2] was established, and each tool will be evaluated separately. In CE8 Subtest 2, the focus is the adaptation between the PA method and the SF method. In the rest of this document, we may use pixel-based filter selection and region-based filter selection to indicate PA and SF, respectively. The rest of this document is organized as follows. The proposed region-based filter selection will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

In JCTVC-D119, the SF method was to filter all pixels in one FU by a filter, and the size of FU is flexible and decided by the encoder. In order to keep the encoding algorithm the same as the anchor as much as possible, we implemented the adaptation between PA and SF on top of HM 2.0 by adding a region-based filter selection and allowing adaptation between the pixel-based method and the region-based method. In the region-based method, a picture is always divided into 16 roughly-equal-size regions where each region is aligned with LCU boundaries, as shown in Figure 1. Each region is assigned with an index, and two regions with successive indices can be merged. After region merging, one filter is applied to one region. The supported filter sizes of the region-based method are the same as the anchor. As for the encoding flow, only one extra encoding pass is added for the region-based ALF before the original multi-pass pixel-based ALF encoding algorithm in the anchor. Please note that the extra encoding pass can be processed in parallel with the original group merging process of the pixel-based method. After the original group merging process, the current picture can choose to use the pixel-based ALF or the region-based ALF for the following refinement process including filter redesign with CU on/off control and filter tap decision. Compared to the pixel-based ALF, the region-based ALF can save computations of pixel adaptation, so the average decoder complexity can be reduced when the two methods are switched. Moreover, the region-based ALF also significantly reduces the number of filter switches in a picture, which means less switching power is consumed. The proposed syntax is shown in Table 1, and the modified part is highlighted with yellow background. The only syntax change is to add one flag in the slice header is used to select between the pixel-based ALF and region-based ALF.

Figure 1  16 regions in a 416x240 picture where each square denotes a 64x64 LCU

Table 1  Proposed syntax modification

| alf_param( ) { | C | Descriptor |
| --- | --- | --- |
| alf_flag | 2 | u(1)¦ae(v) |
| if( alf_flag ) { |  |  |
| region_adaptation_flag | 2 | u(1)¦ae(v) |
| … // original ALF parameters for luma |  |  |
| alf_chroma_idc | 2 | u(2)¦ae(v) |
| … // original ALF parameters for chroma and luma CU on/off control |  |  |
| } |  |  |
| } |  |  |

Unlike the FUs proposed in JCTVC-D119 where the FU size can be changed picture by picture, the regions for adapting filters in this proposal are fixed as long as the picture size is determined. During discussion of FUs at the last meeting in Daegu, it was commented that the FU concept may have problems when it is applied at slice level. This may be true; however, the current ALF design in HEVC is always applied at picture level even when slices are supported. Now the ALF parameters of a picture are sent in the first slice header of the picture, and CE4 Subset 3 [4] is evaluating the possibility of sending ALF parameters in the picture parameter set when multiple slices are used in a picture. When the ALF is applied at picture-level, the FUs and the regions defined in this proposal would not cause problems.

# Simulation results

As requested in JCTVC-D608 [2], the high efficiency (HE) test configurations in JCTVC-D600 [3] were tested. Experimental results are shown in Table 2, Table 3, and Table 4. The proposed adaptation between the pixel-based ALF and the region-based ALF can achieve 0%, 0.2%, and 0.4% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is increased by 1-2%. The decoding time is decreased by 1-4%. Generally speaking, the proposed method can achieve better compression performance with less decoding time than the anchor.

Table 2  Experimental data for HE-AI

| Proposal versus JCTVC-D600 |  | High Efficiency All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | 0.0 | 0.0 | 0.0 |
|  | PeopleOnStreet | 0.0 | 0.0 | 0.0 |
|  | Nebuta | 0.0 | 0.0 | 0.0 |
|  | SteamLocomotive | 0.1 | 0.0 | 0.0 |
| Class B 1080p | Kimono | 0.2 | 0.0 | 0.0 |
|  | ParkScene | 0.0 | 0.0 | 0.0 |
|  | Cactus | 0.0 | 0.0 | 0.0 |
|  | BasketballDrive | 0.0 | 0.1 | 0.1 |
|  | BQTerrace | 0.0 | 0.0 | 0.0 |
| Class C WVGA | BasketballDrill | -0.3 | 0.3 | 0.3 |
|  | BQMall | 0.0 | 0.0 | 0.0 |
|  | PartyScene | 0.0 | 0.0 | 0.0 |
|  | RaceHorses | 0.0 | 0.0 | 0.0 |
| Class D WQVGA | BasketballPass | 0.0 | 0.0 | 0.0 |
|  | BQSquare | 0.0 | 0.0 | 0.0 |
|  | BlowingBubbles | 0.0 | 0.0 | 0.0 |
|  | RaceHorses | 0.0 | 0.0 | 0.0 |
| Class E 720p | Vidyo1 | 0.0 | 0.0 | 0.0 |
|  | Vidyo3 | 0.0 | 0.0 | 0.0 |
|  | Vidyo4 | 0.0 | 0.0 | 0.0 |
| Summary | Class A | 0.0 | 0.0 | 0.0 |
|  | Class B | 0.1 | 0.0 | 0.0 |
|  | Class C | -0.1 | 0.1 | 0.1 |
|  | Class D | 0.0 | 0.0 | 0.0 |
|  | Class E | 0.0 | 0.0 | 0.0 |
|  | All | 0.0 | 0.0 | 0.0 |
|  | Encoding Time % | 102 |  |  |
|  | Decoding Time % | 99 |  |  |

Table 3  Experimental data for HE-RA

| Proposal versus JCTVC-D600 |  | High Efficiency Random Access |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.1 | 0.0 | 0.0 |
|  | PeopleOnStreet | 0.1 | 0.0 | -0.1 |
|  | Nebuta | -0.1 | -0.1 | 0.0 |
|  | SteamLocomotive | 0.0 | 1.0 | 0.6 |
| Class B 1080p | Kimono | 0.0 | 0.0 | -0.1 |
|  | ParkScene | -0.1 | 0.1 | 0.1 |
|  | Cactus | -0.4 | 0.1 | 0.0 |
|  | BasketballDrive | -0.4 | 0.4 | 0.2 |
|  | BQTerrace | -0.4 | 0.0 | 0.2 |
| Class C WVGA | BasketballDrill | -0.8 | 0.3 | 0.4 |
|  | BQMall | -0.2 | 0.2 | 0.3 |
|  | PartyScene | -0.1 | 0.0 | 0.2 |
|  | RaceHorses | 0.0 | 0.0 | -0.1 |
| Class D WQVGA | BasketballPass | 0.0 | 0.2 | 0.1 |
|  | BQSquare | 0.0 | -0.3 | 0.0 |
|  | BlowingBubbles | -0.1 | -0.1 | 0.0 |
|  | RaceHorses | 0.0 | -0.2 | -0.1 |
| Class E 720p | Vidyo1 |  |  |  |
|  | Vidyo3 |  |  |  |
|  | Vidyo4 |  |  |  |
| Summary | Class A | -0.1 | 0.2 | 0.1 |
|  | Class B | -0.3 | 0.1 | 0.1 |
|  | Class C | -0.3 | 0.1 | 0.2 |
|  | Class D | 0.0 | -0.1 | 0.0 |
|  | Class E |  |  |  |
|  | All | -0.2 | 0.1 | 0.1 |
|  | Encoding Time % | 101 |  |  |
|  | Decoding Time % | 96 |  |  |

Table 4  Experimental data for HE-LD

| Proposal versus JCTVC-D600 |  | High Efficiency Low Delay |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |
|  | PeopleOnStreet |  |  |  |
|  | Nebuta |  |  |  |
|  | SteamLocomotive |  |  |  |
| Class B 1080p | Kimono | 0.2 | 0.0 | 0.2 |
|  | ParkScene | -0.2 | -0.1 | -0.1 |
|  | Cactus | -1.1 | -0.1 | -0.4 |
|  | BasketballDrive | -0.7 | 0.2 | -0.3 |
|  | BQTerrace | -0.6 | 0.0 | -0.1 |
| Class C WVGA | BasketballDrill | -1.2 | 0.2 | 0.3 |
|  | BQMall | -0.4 | 0.1 | -0.7 |
|  | PartyScene | -0.4 | 0.0 | 0.1 |
|  | RaceHorses | -0.1 | 0.4 | 0.3 |
| Class D WQVGA | BasketballPass | -0.1 | -0.4 | -0.1 |
|  | BQSquare | 0.0 | 0.0 | -0.1 |
|  | BlowingBubbles | -0.1 | -0.2 | 0.5 |
|  | RaceHorses | -0.1 | 0.1 | 0.1 |
| Class E 720p | Vidyo1 | -1.1 | -0.5 | -0.1 |
|  | Vidyo3 | -0.7 | -1.4 | 0.7 |
|  | Vidyo4 | -0.4 | 0.2 | -0.5 |
| Summary | Class A |  |  |  |
|  | Class B | -0.5 | 0.0 | -0.1 |
|  | Class C | -0.5 | 0.2 | 0.0 |
|  | Class D | -0.1 | -0.1 | 0.1 |
|  | Class E | -0.7 | -0.5 | 0.0 |
|  | All | -0.4 | -0.1 | 0.0 |
|  | Encoding Time % | 101 |  |  |
|  | Decoding Time % | 97 |  |  |

# Conclusions

In CE8 Subtest 2, MediaTek tested an ALF solution allowing adaptation between the pixel-based filter selection and the region-based filter selection at slice level. The region-based method can not only improve the coding efficiency but also reduce the decoding time. It is reported that bit rate reductions are 0%, 0.2%, and 0.4% for HE-AI, HE-RA, and HE-LD, respectively, when the JCTVC-D600 anchor is compared. The encoding time is increased by 1-2%, and the decoding time is decreased by 1-4%. It is proposed to adopt this tool into HM and enable it for HE configurations.

# Patent rights declaration(s)

MediaTek Inc. Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, and T. Watanabe, “CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, Jan. 2011.

- T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, and F. Kossentini, “Description of CE8: Non-deblocking Loop Filtering” Document of Joint Collaborative Team on Video Coding, JCTVC-D608, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

- Y.-W. Huang and I.-K. Kim, “Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity” Document of Joint Collaborative Team on Video Coding, JCTVC-D604, Jan. 2011.