| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E047 |
| --- | --- |

| Title: | CE8 Subtest 3: Adaptation between ALF and AO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Ching-Yeh Chen1 Chih-Ming Fu1 Chia-Yang Tsai1 Yu-Wen Huang1 Shawmin Lei1 Marta Karczewicz2 In Suk Chong2 Tomoo Yamakage3 Takeshi Chujoh3 Takashi Watanabe3 | Emails: | chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com tomoo.yamakage@toshiba.co.jp takeshi.chujoh@toshiba.co.jp takashi39.watanabe@toshiba.co.jp |
| Source: | MediaTek Inc.1, Qualcomm2, and TOSHIBA Corporation3 |  |  |

_____________________________

# Abstract

This contribution reported the results of CE8 Subtest 3: adaptation between adaptive loop filter (ALF) and adaptive offset (AO), which is a joint proposal from MediaTek, Qualcomm, and Toshiba. In order to avoid using multiple pixel classification methods in the decoder, the pixel classification is simplified and unified for both ALF and AO. Moreover, adaptation between ALF and AO at slice level is implemented to reduce average decoding time. In comparison with the JCTVC-D600 anchor, the proposed method reportedly achieved 0.2%, 0.7%, and 1.0% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively, with roughly unchanged encoding time and 10-11% decoding time reduction.

- Introduction

In JCTVC-D119 [1], MediaTek, Qualcomm, and Toshiba proposed an ALF solution, which contains several tools including filter unit (FU) concepts and adaptation among single-filter (SF), pixel-adaptation (PA), and adaptive-offset (AO) methods for each FU. Coming out of the 4th JCT-VC meeting, Core Experiment 8 (CE8) [2] was established, and each tool will be evaluated separately. In CE8 Subtest 3, the focus is the adaptation between ALF and AO where the ALF uses the PA method. First, we implemented AO on top of HM 2.0. Next, we simplified and unified the pixel classification for ALF and AO. Finally, we enabled adaptation between ALF and AO at slice level. The encoding algorithm was kept the same as anchor as much as possible for fair comparisons. In addition to the multiple ALF encoding passes in HM2.0, only one AO encoding pass was added, which can be parallelized with the ALF encoding passes. The rest of this document is organized as follows. The proposed pixel classification will be described in Section 2, simulation results will be shown in Section , and conclusions will be given in Section 4.

# Algorithm description

In JCTVC-D119, AO classified all pixels in one FU into several categories and reduced the distortion for each category with an offset. There were eight kinds of AO modes, including band offset, cross edge offset, diagonal cross edge offset, … etc. Multiple pixel classification methods were used in the solution, which complicated both the encoder and the decoder. Hence, we made efforts on developing a simplified and unified pixel classification. In this proposal, edge information and sum of Laplacian method are combined as follows. First, the neighboring pixel information is applied to classify all pixels in one picture into 9 categories, as shown in Figure 1. The directional categorization in JCTVC-D119 is removed for simplification. Instead of using two 2-D 4-pel patterns and four 1-D 2-pel patterns, only one 2-D 8-pel edge pattern is used. Next, category 0 is further divided into 7 sub-categories by sum of Laplacian. In this way, there are up to 15 groups in one picture, and each group has its own filter (ALF) or offset (AO). With the unified classification, AO is conceptually a simplified ALF. For ALF, the group merging is allowed as in the anchor. Filter tap decision, CU on/off control, and other encoder decisions are all the same as those used in the anchor. As for AO, the group merging is disabled, and no CU on/off control is used. In addition to the original multiple encoding passes, only one more encoding pass is added, which can be processed in parallel with the filter design of ALF in principle. The adaptation between ALF and AO mode is done at slice level. By simplifying and unifying the pixel classification with adaptation between ALF and AO, not only the coding efficiency can be improved but also the decoder complexity can be reduced.

CTable[17] = { 1, 1, 2, 2, 3, 3, 4, 0, 0, 0, 5, 5, 6, 6, 7, 7, 8};

groupIdx = 8;

for( i = 0; i < 8; i++ )

groupIdx += ( (imgY[C]>imgY[pi]) - (imgY[C]<imgY[pi]) );

Category[C] = CTable[groupIdx];

Figure 1  Proposed pixel classification with neighboring pixel information

The proposed syntax is shown Table 1. Modifications are highlighted with yellow background. One flag in the slice header is used to select between ALF and AO. There were up to 16 groups in JCTVC-D119, and now the number is reduced to 15. Consequently, the number of group merging flags is reduced from 15 to 14. When ALF is selected by a slice, there is no further change of syntax in comparison with HM2.0. When AO is selected, the rest ALF syntax elements are bypassed, and 15 offsets are transmitted.

Table 1  Proposed syntax modification

| alf_param( ) { | C | Descriptor |
| --- | --- | --- |
| alf_flag | 2 | u(1)¦ae(v) |
| if( alf_flag ) { |  |  |
| offset_only_flag | 2 | u(1)¦ae(v) |
| if( offset_only_flag ) { |  |  |
| for ( i = 0; i < 15 ; i ++ ) |  |  |
| offset | 2 | se(v)¦ae(v) |
| } else { |  |  |
| ... // original ALF parameters for luma | 2 |  |
| } |  |  |
| … // original ALF parameters for chroma and luma CU on/off control |  |  |
| } |  |  |
| } |  |  |

# Simulation results

As requested in JCTVC-D608 [2], the high efficiency (HE) test configurations in JCTVC-D600 [3] were tested. Experimental results are shown in , Table 3, and Table 4. The proposed adaptation between AO and ALF can achieve 0.2%, 0.7%, and 1.0% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is almost unchanged. The decoding time is reduced by 10-11%. Generally speaking, the proposed method can achieve better compression performance with less decoding time than the anchor.

# Conclusions

In CE8 Subtest 3, MediaTek, Qualcomm, and Toshiba tested an ALF solution allowing adaptation between ALF and AO with a simplified and unified pixel classification originated from JCTVC-D119. The AO can not only improve the coding efficiency but also reduce the decoding time. It is reported that bit rate reductions are 0.2%, 0.7%, and 1.0% for HE-AI, HE-RA, and HE-LD, respectively. The encoding time is about the same as the JCTVC-D600 anchor, and the decoding time is reduced by 10-11%. It is proposed to adopt this tool into HM and enable it for HE configurations.

# Patent rights declarations

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, and T. Watanabe, “CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, Jan. 2011.

- T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, and F. Kossentini, “Description of CE8: Non-deblocking Loop Filtering” Document of Joint Collaborative Team on Video Coding, JCTVC-D608, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

Table 2  Experimental results for HE-AI

| Proposal versus JCTVC-D600 |  | High Efficiency All Intra |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -0.3 | 0.1 | 0.0 |
|  | PeopleOnStreet | -0.9 | 0.0 | 0.0 |
|  | Nebuta | 0.0 | 0.1 | 0.1 |
|  | SteamLocomotive | 0.1 | 0.1 | 0.0 |
| Class B 1080p | Kimono | 0.1 | 0.1 | 0.1 |
|  | ParkScene | -1.1 | 0.1 | 0.1 |
|  | Cactus | -0.3 | 0.1 | -0.1 |
|  | BasketballDrive | -0.1 | 0.3 | 0.2 |
|  | BQTerrace | -0.3 | 0.2 | 0.1 |
| Class C WVGA | BasketballDrill | -0.8 | 0.1 | 0.1 |
|  | BQMall | -0.2 | 0.0 | 0.1 |
|  | PartyScene | -0.4 | 0.1 | 0.2 |
|  | RaceHorses | -0.1 | 0.1 | 0.1 |
| Class D WQVGA | BasketballPass | 0.2 | 0.1 | 0.0 |
|  | BQSquare | -0.5 | 0.0 | 0.0 |
|  | BlowingBubbles | -0.2 | 0.1 | 0.1 |
|  | RaceHorses | 0.0 | 0.1 | 0.0 |
| Class E 720p | Vidyo1 | 0.0 | 0.1 | 0.1 |
|  | Vidyo3 | -0.4 | 0.1 | 0.1 |
|  | Vidyo4 | 0.0 | 0.1 | -0.1 |
| Summary | Class A | -0.3 | 0.3 | 0.2 |
|  | Class B | -0.3 | 0.2 | 0.1 |
|  | Class C | -0.4 | 0.1 | 0.1 |
|  | Class D | -0.1 | 0.0 | 0.1 |
|  | Class E | -0.1 | 0.1 | 0.2 |
|  | All | -0.2 | 0.1 | 0.1 |
|  | Encoding Time % | 100 |  |  |
|  | Decoding Time % | 89 |  |  |

Table 3  Experimental results for HE-RA

| Proposal versus JCTVC-D600 |  | High Efficiency Random Access |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic | -1.6 | -0.1 | -0.2 |
|  | PeopleOnStreet | -1.9 | 0.0 | 0.0 |
|  | Nebuta | 0.0 | 0.1 | 0.0 |
|  | SteamLocomotive | 0.5 | -0.2 | 0.1 |
| Class B 1080p | Kimono | 0.0 | -0.1 | 0.0 |
|  | ParkScene | -1.4 | 0.1 | 0.1 |
|  | Cactus | -0.8 | -0.1 | -0.1 |
|  | BasketballDrive | 0.0 | 0.2 | 0.0 |
|  | BQTerrace | -1.8 | 0.1 | 0.1 |
| Class C WVGA | BasketballDrill | -1.2 | -0.3 | -0.3 |
|  | BQMall | -1.0 | 0.1 | 0.0 |
|  | PartyScene | -0.8 | 0.1 | 0.1 |
|  | RaceHorses | -0.3 | 0.0 | 0.1 |
| Class D WQVGA | BasketballPass | 0.1 | 0.6 | 0.2 |
|  | BQSquare | -1.1 | 1.4 | 1.4 |
|  | BlowingBubbles | -0.7 | -0.2 | 0.1 |
|  | RaceHorses | -0.3 | 0.0 | -0.3 |
| Class E 720p | Vidyo1 |  |  |  |
|  | Vidyo3 |  |  |  |
|  | Vidyo4 |  |  |  |
| Summary | Class A | -0.8 | 0.0 | 0.0 |
|  | Class B | -0.8 | 0.0 | 0.0 |
|  | Class C | -0.8 | 0.0 | 0.0 |
|  | Class D | -0.5 | 0.4 | 0.4 |
|  | Class E |  |  |  |
|  | All | -0.7 | 0.1 | 0.1 |
|  | Encoding Time % | 100 |  |  |
|  | Decoding Time % | 89 |  |  |

Table 4  Experimental results for HE-LD

| Proposal versus JCTVC-D600 |  | High Efficiency Low Delay |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-Rate Y % | BD-Rate Cb % | BD-Rate Cr % |
| Class A 2560x1600 | Traffic |  |  |  |
|  | PeopleOnStreet |  |  |  |
|  | Nebuta |  |  |  |
|  | SteamLocomotive |  |  |  |
| Class B 1080p | Kimono | -0.4 | -0.2 | -0.2 |
|  | ParkScene | -1.8 | -0.1 | 0.0 |
|  | Cactus | -0.7 | -0.6 | -0.4 |
|  | BasketballDrive | 0.0 | 0.5 | 0.1 |
|  | BQTerrace | -0.6 | 1.2 | 1.2 |
| Class C WVGA | BasketballDrill | -1.9 | 0.0 | -0.3 |
|  | BQMall | -1.7 | 0.3 | 0.3 |
|  | PartyScene | -1.1 | 0.5 | 0.5 |
|  | RaceHorses | -0.6 | 0.2 | 0.5 |
| Class D WQVGA | BasketballPass | -0.2 | -0.7 | -0.2 |
|  | BQSquare | -1.1 | 3.9 | 2.8 |
|  | BlowingBubbles | -1.1 | 0.0 | 0.2 |
|  | RaceHorses | -1.4 | 0.0 | 0.1 |
| Class E 720p | Vidyo1 | -0.6 | -1.2 | -0.5 |
|  | Vidyo3 | -3.3 | -0.8 | 0.9 |
|  | Vidyo4 | -0.2 | -0.1 | -0.2 |
| Summary | Class A |  |  |  |
|  | Class B | -0.7 | 0.1 | 0.2 |
|  | Class C | -1.3 | 0.2 | 0.2 |
|  | Class D | -0.9 | 0.8 | 0.7 |
|  | Class E | -1.4 | -0.7 | 0.1 |
|  | All | -1.0 | 0.2 | 0.3 |
|  | Encoding Time % | 100 |  |  |
|  | Decoding Time % | 90 |  |  |