| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E048 |
| --- | --- |

| Title: | CE9: Results of Experiments M-Series |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Jian-Liang Lin Yi-Wen Chen Yu-Pao Tsai Yu-Wen Huang Shawmin Lei | Emails: | jl.lin@mediatek.com yiwen.chen@mediatek.com yupao.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the results of CE9 experiments M-series. The important results are summarized as follows. In experiment M01, replacing the temporal candidate by TBR reportedly achieved 0.8%, 1.0%, 0.9%, and 1.3% bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD, respectively. In experiment M07, the modified derivation of spatial candidates reportedly achieved 0.6%, 0.8%, 0.2%, and 0.3% bit rate reductions. In experiment M12, the modified derivation of temporal candidates reportedly achieved 0%, 0%, 0.3% and 0.6% bit rate reductions. In experiment M27, the combination of M01, M07, and M12 reportedly achieved 1.3%, 1.7%, 1.2% and 2.0% bit rate reductions. In experiment M31, the combination of M01, M07, D125(2.4’), and D274(3) reportedly achieved 1.3%, 1.8%, 1.5%, and 2.6% bit rate reductions.

- Introduction

The CE9 [1] experiments M-series were conducted. The experiments M-series include techniques from JCTV-D125 [2] and JCTVC-D274 [3].

# Simulation results

The evaluation of M-series was performed using a software implementation based on HM2.0, and the anchor is JCTVC-D600 [4]. The important results are summarized in the following subsections. The complete results can be found in the attached Excel file.

## M01: temporal candidate at the above left position of the below right PU

The temporal candidate in HM2.0 is replaced by the temporal candidate TBR, as the temporal candidate H proposed in section 2.1 of JCTVC-D125. As shown in Figure 1, the temporal candidate in the above left position of the below right PU is used, and the AMVP candidate set becomes:

- Left predictor

- Top predictor

- TBR

Figure 1  The temporal candidate at the above left position of the below right PU

Table 1 shows the description of M01. The results are shown in Table 2. The bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD are 0.8%, 1.0%, 0.9%, and 1.3%, respectively.

Table 1  Description of M01

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| M01 | JCTVC-D125 Section 2.1 (H) | HM2.0 | JCTVC-D125 Section 2.1 (H) | HM2.0 | HM2.0 |

Table 2  Results of M01

## M07: modified derivation of spatial candidates

The derivation of spatial candidates is modified as proposed in the Section 2.3 of JCTVC-D125. When the motion vector with same reference picture index of the same reference list for a spatially neighboring block does not exist, the spatial candidate can be derived from the other reference list or different reference pictures. The selection is based on a predefined order as shown in the following:

- The MV from the same reference list and the same reference picture

- The MV from the other reference list and the same reference picture

- The scaled MV from the same reference list and a different reference picture

- The scaled MV from the other reference list and a different reference picture

Table 3 shows the description of M07. The results are shown in Table 4. The bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD are 0.6%, 0.8%, 0.2%, and 0.3%, respectively.

Table 3  Description of M07

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| M07 | JCTVC-D125 Section 2.3 | HM2.0 | JCTVC-D125 Section 2.3 | HM2.0 | HM2.0 |

Table 4  Results of M07

## M12: modified derivation of temporal candidates

The derivation of temporal candidates is modified as proposed in the Section 2.4 of JCTVC-D125. Table 5 shows the description of M12. The results are shown in Table 6. The bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD are 0%, 0%, 0.3%, and 0.6%, respectively.

Table 5  Description of M12

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| M12 | JCTVC-D125 Section 2.4 | JCTVC-D125 Section 2.4 | JCTVC-D125 Section 2.4 | HM2.0 | HM2.0 |

Table 6  Results of M12

## M27: combination of M01, M07, and M12

The M01, M07, and M12 are combined. The temporal candidate TBR is applied on AMVP. When TBR is not available, the central temporal candidate in HM2.0 is used. The modified derivation of spatial candidates is applied on AMVP only, and the modified derivation of temporal candidates is applied on both AMVP and MERGE. Table 7 shows the description of M27. The results are shown in Table 8. The bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD are 1.3%, 1.7%, 1.2%, and 2.0%, respectively.

Table 7  Description of M27

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| M27 | JCTVC-D125 Section 2.1 (H’), 2.3, 2.4 | JCTVC-D125 Section 2.4 | JCTVC-D125 Section 2.1 (H’), 2.3,2.4 | HM2.0 | HM2.0 |

Table 8  Results of M27

## M31: combination of M01, M07, and simplified Sec. 2.4 in JCTVC-D125

The M01, M07, a simplified temporal derivation denoted as JCTVC-D125(2.4’), and the reference index decision suggested in D274(3) are combined, as shown in Table 9. According to D274(3), the reference index of SKIP is decided by the majority of reference picture indices of neighboring prediction units instead of always setting to 0.

In the temporal candidate derivation of HM2.0, the co-located motion vector that crosses the current picture is chosen first. If both co-located motion vectors cross the current picture or both do not cross, the one with shortest reference picture distance will be used. If both have the same reference picture distance, the co-located vector in the list 0 will be selected. The proposed JCTVC-D125(2.4’) is related to a temporal derivation algorithm, which is a simplification of the current HM 2.0 by removing the process of checking shortest reference picture distance. In the proposed algorithm, if both motion vectors cross the current picture or both do not cross, the co-located motion vector in the same list will be chosen.

The results are shown in Table 10. The bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD are 1.3%, 1.8%, 1.5%, and 2.5%, respectively.

Table 9  Description of M31

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| M31 | JCTVC-D125 Section 2.1 (simplified version - 1 MVpred), 2.3, 2.4’ (effectively same as JCTVC-D274 (3’)) | JCTVC-D125 2.4’ (effectively same as JCTVC-D274 (3’)) | JCTVC-D125 Section 2.1 (simplified version - 1 MVpred), 2.3, 2.4’ (effectively same as JCTVC-D274 (3’)) | JCTVC-D274 (2) | HM2.0 |

Table 10  Results of M31

The experimental results of the M-series experiments are included in the attached files.

# Conclusions

In CE9, MediaTek conducted the M-series experiments. The results reportedly showed that combing M01, M07, D125(2.4’), and D274(3) as M31 can achieve an average 1.3%, 1.8%, 1.5%, and 2.5% bit rate reductions for HE-RA, LC-RA, HE-LD, and LC-LD, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Jung, B. Bross, P. Chen, and W.-J. Han, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D609, Jan. 2011.

- J.-L. Lin, Y.-P. Tsai, Y.-W. Huang, and S. Lei, “Improved Advanced Motion Vector Prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-D125, Jan. 2011.

- T. Sugio and T. Nishi, “Modified Usage of Predicted Motion Vectors in Forward Directional Bi-Predictive Coding Frame,” Document of Joint Collaborative Team on Video Coding, JCTVC-D274, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.