#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Sample Adaptive Offset

| sao_param() { | C | Descriptor |
| --- | --- | --- |
| sample_adaptive_offset _flag | 2 | u(1) |
| if (sample_adaptive_offset_flag ) { |  |  |
| sao_split_param( 0, 0, 0) |  |  |
| sao_offset_param( 0, 0, 0) |  |  |
| } |  |  |
| } |  |  |

| sao_split_param( xs, ys, sao_curr_depth ) { | C | Descriptor |
| --- | --- | --- |
| if (sao_curr_depth < SaoMaxDepth ) { |  |  |
| sao_split_flag [ sao_curr_depth ][ ys ][ xs ] | 2 | u(1) |
| } else { |  |  |
| sao_split_flag [ sao_curr_depth ][ ys ][ xs ] = 0 |  |  |
| } |  |  |
| If (sao_split_flag [ sao_curr_depth ][ ys ][ xs ]) { |  |  |
| sao_split_param ( xs + 0, ys + 0, sao_curr_depth + 1 ) |  |  |
| sao_split_param ( xs + 1, ys + 0, sao_curr_depth + 1 ) |  |  |
| sao_split_param ( xs + 0, ys + 1, sao_curr_depth + 1 ) |  |  |
| sao_split_param ( xs + 1, ys + 1, sao_curr_depth + 1 ) |  |  |
| } |  |  |
| } |  |  |

| sao_offset_param( xs, ys, sao_curr_depth ) { | C | Descriptor |
| --- | --- | --- |
| if (sao_split_flag [ sao_curr_depth ][ ys ][ xs ]) { |  |  |
| sao_offset_param( xs + 0, ys + 0, sao_curr_depth + 1 ) |  |  |
| sao_offset_param( xs + 1, ys + 0, sao_curr_depth + 1 ) |  |  |
| sao_offset_param( xs + 0, ys + 1, sao_curr_depth + 1 ) |  |  |
| sao_offset_param( xs + 1, ys + 1, sao_curr_depth + 1 ) |  |  |
| } else { |  |  |
| sao_type_idx [ sao_curr_depth ][ ys ][ xs ] | 2 | ue(v) |
| curr_num = sao_offset_num [sao_type_idx [ sao_curr_depth ][ ys ][ xs ]] |  |  |
| if ( sao_type_idx [ sao_curr_depth ][ ys ][ xs ] != 0 ) { |  |  |
| for ( i = 0; i < curr_num; i++ ) { |  |  |
| sao_offset [ sao_curr_depth ][ ys ][ xs ][ i ] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

#### Sample adaptive offset parameter semantics

sample_adaptive_offset_flag specifies whether sample adaptive offset applies or not for the current slice.

sao_split_flag [ sao_curr_depth ] [ ys ][ xs ] specifies whether a region is split into four sub-regions with half horizontal and vertical number of maximum size CU. The array indices xs, ys specify the four children regions of current sao_curr_depth.

When sao_split_flag [ sao_curr_depth ] [ ys ][ xs ] is not present, the following applies:

–	If sao_curr_depth is equal or greater than SaoMaxDepth, the value of sao_split_flag [ sao_curr_depth ] [ ys ][ xs ] is inferred to be equal to 0, where the derivation of SaoMaxDepth is described as follows

DefaultSaoMaxDepth = 4 (or predefined value)

PicWidthInLCu = (pic_width_in_luma_samples + (1 << Log2MaxCUSize )-1) / (1 << Log2MaxCUSize)

PicHeightInLCu = (pic_height_in_luma_samples + (1 << Log2MaxCUSize )-1) / (1 << Log2MaxCUSize)

max_depth_W = log2(PicWidthInLCu)

max_depth_H = log2(PicHeightInLCu)

–  If max_depth_W is larger than max_depth_H

SaoMaxDepthTmp is set to max_depth_H

–  Otherwise,

SaoMaxDepthTmp is set to max_depth_W

–  If SaoMaxDepthTmp is smaller than DefaultSaoMaxDepth

SaoMaxDepth is set to SaoMaxDepthTmp

–  Otherwise (SaoMaxDepthTmp is equal to or larger than DefaultSaoMaxDepth)

SaoMaxDepth is set to DefaultSaoMaxDepth

–	Otherwise (sao_curr_depth is less than SaoMaxDepth),

sao_split_flag [ sao_curr_depth ] [ ys ][ xs ] is inferred to be equal to 0.

sao_type_idx[ sao_curr_depth ][ ys ][ xs ] indicates the offset type to be applied in (ys*2+xs)-th region of sao_curr_depth-th depth of current slice.

sao_offset [ sao_curr_depth ][ ys ][ xs ][ i ] indicates the i-th category offset value to be applied in (ys*2+xs)-th region of sao_curr_depth-th depth of current slice. The sao_offset number of each offset type nSaoLength [sao_type_idx] is specified in Table 7-7.

Table 7-1 – Specification of sao_type_idx

| - sao_type_idx | - sample adaptive offset type to be used | - Number of categoris, nSaoLength [sao_type_idx] |
| --- | --- | --- |
| - 0 | - None | - 0 |
| - 1 | - 1-D 0-degree pattern edge offset | - 4 |
| - 2 | - 1-D 90-degree pattern edge offset | - 4 |
| - 3 | - 1-D 135-degree pattern edge offset | - 4 |
| - 4 | - 1-D 45-degree pattern edge offset | - 4 |
| - 5 | - central bands band offset | - 16 |
| - 6 | - side bands band offset | - 16 |

### Sample adaptive offset process

A sample adaptive offset process shall be conditionally performed after the completion of the deblocking filter process for the decoded picture. In a selected region for the processed video data, each of pixels in the region is classified based on its characteristic measurement. For the class according to the pixels in the region, the intensity offset is applied. Sample adaptive offset includes band offset (BO) and edge offset (EO). BO and EO classify pixels by intensity interval and 1-D pixel patterns respectively. One offset is derived for each category of pixels. A picture can be divided into multiple regions aligned with largest coding unit boundaries, and each region is allowed to switch between BO and EO.

Edge offset (EO) classifies all pixels of a region into multiple categories by comparing with neighboring pixels and compensates the average offset in each category. The classification types of four 1-D patterns are 1-D 0-degree pattern, 1-D 90-degree pattern, 1-D 135-degree pattern, and 1-D 45-degree pattern. Band offset (BO) classifies all pixels of a region into multiple bands where each band contains pixels of the same intensity interval and has a derived offset. The intensity range is equally divided into M intervals between minimum and the maximum intensity values, and each interval has an offset. The central M/2 bands and the rest M/2 bands are denoted as the first group and the second group, respectively, and only one group of offsets in the selected region is processed. The BO process uses the first five significant bits of pixels as the input of BO processing to reduce the complexity. Each offset value has 1-bit bit-depth increase when the internal bit-depth increase is non-zero.

This process is invoked when both sample_adaptive_offset_enabled_flag is equal to 1 and the sample_adaptive_offset_flag is equal to 1.

If sample_adaptive_offset_flag is equal to 1, the decoding process for luma region as specified in this subclause 8.6.2.1 is invoked with the luma region ( 0, 0 ), the luma largest CU location ( 0, 0 ), the variable sao_curr_depth set equal to 0, the variable saoWidthInCu set equal to PicWidthInLCu, and the variable saoHeightInCu set equal to PicHeightInLCu, where the parameter PicWidthInLCu is the number of LCU in picture width and PicHeightInLCu is the number of LCU in picture height (in section sample adaptive offset parameter semantics)

#### Derivation process for sample adaptive offset

Inputs to this process are:

–	a luma location ( xR, yR ) specifying the top-left luma region of the current region of current depth,

–	a luma location ( xC, yC ) specifying the top-left luma largest CU location of the current region relative to the top-left luma sample of the current picture,

–	a variable saoWidthInCu specifying the number of CUs in width of the current region,

–	a variable saoHeightInCu specifying the number of CUs in Height of the current region,

–	a variable sao_curr_depth specifying the hierarchy depth of the current region.

Output of this process is:

–	a modified reconstruction of the current region.

Depending sao_split_flag [ sao_curr_depth ] [ yR ][ xR ], the following applies:

–	If sao_split_flag [ sao_curr_depth ] [ yR ][ xR ] is equal to 1 and sao_curr_depth smaller than SaoMaxDepth, the following ordered steps apply:

- saoWidthInCuDown1 =  saoWidthInCu >> 1

- saoWidthInCuDown2 =  saoWidthInCu - saoWidthInCuDown1

- saoHeightInCuDown1 =  saoHeightInCu >> 1

- saoHeightInCuDown2 =  saoHeightInCu - saoHeightInCuDown1

- xRDown1 = xR

- xRDown2 = xR + 1

- yRDown1 = yR

- yRDown2 = yR + 1

- xCDown1 = xC

- xCDown2 = xC + saoWidthInCuDown1

- yCDown1 = yC

- yCDown2 = yC + saoHeightInCuDown1

- The decoding process for luma region as specified in this subclause is invoked with the luma region ( xRDown1, yRDown2), the luma largest CU location ( xCDown1, yCDown1), the variable sao_curr_depth set equal to sao_curr_depth + 1, the variable saoWidthInCu set equal to saoWidthInCuDown1, and the variable saoHeightInCu set equal to saoHeightInCuDown1.

- The decoding process for luma region as specified in this subclause is invoked with the luma region ( xRDown2, yRDown1), the luma largest CU location ( xCDown2, yCDown1), the variable sao_curr_depth set equal to sao_curr_depth + 1, the variable saoWidthInCu set equal to saoWidthInCuDown2, and the variable saoHeightInCu set equal to saoHeightInCuDown1.

- The decoding process for luma region as specified in this subclause is invoked with the luma region ( xRDown1, yRDown2), the luma largest CU location ( xCDown1, yCDown2), the variable sao_curr_depth set equal to sao_curr_depth + 1, the variable saoWidthInCu set equal to saoWidthInCuDown1, and the variable saoHeightInCu set equal to saoHeightInCuDown2.

- The decoding process for luma region as specified in this subclause is invoked with the luma region ( xRDown2, yRDown2), the luma largest CU location ( xCDown2, yCDown2), the variable sao_curr_depth set equal to sao_curr_depth + 1, the variable saoWidthInCu set equal to saoWidthInCuDown2, and the variable saoHeightInCu set equal to saoHeightInCuDown2.

–	Otherwise (sao_split_flag [sao_curr_depth ] [ yR ][ xR ] is equal to 0), the following ordered steps apply:

- Sample adaptive offset for luma samples in a coding unit with iC = xC..(xC+saoWidthInCu-1) and jC = yC.. (yC+saoHeightInCu-1) are derived as follows:

  - The scaling and transformation process as specified in subclause Error: Reference source not found2 is invoked with the luma location (iC, jC), a sample offset type of current coding unit, saoTypeIdx set to sao_type_idx [sao_curr_depth][yR][xR], and the (nSaoLength[saoTypeIdx]) array of current coding unit, saoValueArray is set to sao_offset [ sao_curr_depth ][ ys ][ xs ][ i ], the variable nCs specifying the size of the current coding unit, the (nCs)x(nCs) array recPictureL specifying the current coding unit.

#### Derivation process for sample adaptive offset for luma samples in a coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit to the top-left luma sample of the current picture,

–	a sample offset type of current coding unit, saoTypeIdx,

–	a (nSaoLength[saoTypeIdx]) array of current coding unit, saoValueArray,

–	a variable nCs specifying the size of the current coding unit,

–	a (nCs)x(nCs) array recPictureL specifying the current coding unit,

Output of this process is

–	a modified version of the (nCS)x(nCS) array of luma residual samples.

Let s’ represent the luma sample array recPictureL of the current picture.

Let recSaoPictureL represent the SAO processed luma sample array of the current picture.

The nSaoLength[saoTypeIdx] represent the SAO parameters length of each SAO type are specified in Error: Reference source not found

–	If saoTypeIdx equals to 0

luma samples are not processed

–	Otherwise, if saoTypeIdx equals to 1, 2, 3, or 4

recSaoPictureL [ xC+i, yC+j ] = s’[ xC+i, yC+j ] + saoValueArray [edge_table[EC]],

with  i = xS[SaoTypeIdx].. xE[SaoTypeIdx], j = yS[SaoTypeIdx].. yE[SaoTypeIdx]

where

EC = 2 + k (Sign( s’ [ xC+i, yC+j ]- s’ [ xC+i+HPos[SaoTypeIdx][k], yC+j+ VPos [SaoTypeIdx][k] ] ) with k=0..1

edge_table[5] = {1, 2, 0, 3, 4}

HPos[SaoTypeIdx][k] and VPos [SaoTypeIdx][k] are specified in Error: Reference source not found and

xS[SaoTypeIdx], xE[SaoTypeIdx], yS[SaoTypeIdx], and yE[SaoTypeIdx] are specified in Error: Reference source not found.

–	Otherwise (SaoTypeIdx equals to 5 or 6),

recSaoPictureL [ xC+i, yC+j ] = s’[ xC+i, yC+j ] + saoValueArray [s’[ xC+i, yC+j ]>>nBoRefBit], with i, j = 0.. nCs -1

where nBoRefBit is the number of pixel bit-depth minus log2(M), M is the number of BO bands which is set to 32.

Table 8-25 – Specification of HPos and VPos according to SaoTypeIdx for sample adaptive offset process of luma samples

| saoTypeIdx | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| HPos [saoTypeIdx][0] | -1 | 0 | -1 | 1 |
| HPos [saoTypeIdx][1] | 1 | 0 | 1 | -1 |
| VPos [saoTypeIdx][0] | 0 | -1 | -1 | 1 |
| VPos [saoTypeIdx][1] | 0 | 1 | 1 | -1 |

Table 8-16 – Specification of xS and xE according to SaoTypeIdx for sample adaptive offset process of luma samples

| saoTypeIdx | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| xS [SaoTypeIdx] | 1 | 0 | 1 | 1 |
| xE [SaoTypeIdx] | nCs-2 | nCs-1 | nCs-2 | nCs-2 |
| yS [SaoTypeIdx] | 0 | 1 | 1 | 1 |
| yE [SaoTypeIdx] | nCs-1 | nCs-2 | nCs-2 | nCs-2 |