| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E049 |
| --- | --- |

| Title: | CE13: Sample Adaptive Offset with LCU-Independent Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Chih-Ming Fu Ching-Yeh Chen Chia-Yang Tsai Yu-Wen Huang Shawmin Lei | Emails: | chihming.fu@mediatek.com chingyeh.chen@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reported the results of CE13 for evaluating MediaTek’s proposal on sample adaptive offset (SAO) including band offset (BO) and edge offset (EO). BO and EO classify pixels by intensity interval and 1-D pixel patterns respectively. One offset is derived for each category of pixels. A picture can be divided into multiple regions aligned with largest coding unit (LCU) boundaries, and each region is allowed to switch between BO and EO. The SAO decoding process of an LCU does not require any data from other LCUs; i.e. the SAO decoding process is LCU-independent and does not need any line buffer. Compared with the JCTVC-D600 anchor, SAO reportedly shows 0.5%, 1.3%, 2.2%, 0.7%, 1.8%, and 3.0% bit rate reductions for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively, with unnoticeable encoding time increase and 1-3% decoding time increase.

- Introduction

SAO was first proposed by Samsung [1], and then MediaTek tried to improve coding efficiency and reduce complexity [2][3][4]. In the Daegu meeting in January 2011, Core Experiment 13 [5] was established to evaluate the performance of SAO on HM2.0. The rest of this contribution is organized as follows. The SAO algorithm will be described in Section 2, simulation results will be discussed in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## Overview

In this proposal, SAO is located after deblocking and divides a picture into LCU-aligned regions. Each region can be enhanced by either BO or EO with a localized parameter set. The statistical information for deriving offsets is collected, and the decision of all SAO parameters can be made right after the information collection by using fast distortion estimation. On the encoder side, there is only one additional pass of frame buffer access to add offsets to deblocked pixels. Moreover, on the decoder side, the SAO decoding process is designed to be LCU-independent, i.e., each LCU can be decoded independently without any line buffer, and no additional pass of frame buffer access is needed at the decoder, either.

## Band offset

BO classifies all pixels of a region into multiple bands where each band contains pixels of the same intensity interval. The intensity range is equally divided into 32 intervals between 0 and the maximum intensity value (e.g. 255 for 8-bit pixels), and each interval has an offset. Next, the 32 bands are divided into two groups, and only the offsets in one of the two groups are chosen to be transmitted. One group consists of the central 16 bands, while the other group consists of the rest 16 bands. Thus, when band offset is used, there are two options, based on the different chosen group.

## Edge offset

EO classifies all pixels of a region into multiple categories by comparing each pixel with its neighboring pixels. The proposed EO patterns are shown in Figure 1. When EO is used, one of the four 1-D patterns can be chosen for pixel classification. Since each 1-D pattern only uses two neighboring pixels, the number of operations is low. The classification rules of 1-D patterns are shown in Figure 2.

|  | C |  |  |  | C |  |  |  | C |  |  |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(a) 1-D 0-degree (b) 1-D 90-degree (c) 1-D 135-degree (d) 1-D 45-degree

Figure 1  EO pixel classification patterns

| Category | Condition |
| --- | --- |
| 1 | c < 2 neighboring pixels |
| 2 | c < 1 neighbor && c == 1 neighbor |
| 3 | c > 1 neighbor && c == 1 neighbor |
| 4 | c > 2 neighbors |
| 0 | None of the above |

Figure 2  EO categories

In order to achieve LCU-independent decoding, pixels of the top and bottom rows in each LCU are not processed when the 90-degree, 135-degree, and 45-degree patterns are selected; pixels of the leftmost and rightmost columns in each LCU are not processed when the 0-degree, 135-degree, and 45-degree patterns are selected.

## Syntax description

SAO syntax design is shown in Table 1. General SAO parameters are transmitted in the sequence parameter set (SPS) while rest SAO parameters are sent in the slice header. Modifications are highlighted with yellow background.

Table 1  Syntax elements for SAO

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| sao_used_flag | 1 | u(1) |
| … |  |  |
| } |  |  |

sao_used_flag equal to 0 denotes that SAO is disabled for the current sequence; equal to 1 denotes that SAO is enabled for current sequence.

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| sao_param( ) | 2 |  |
| } |  |  |

| sao_param( ) { | C | Descriptor |
| --- | --- | --- |
| sao_flag | 2 | u(1)¦ae(v) |
| if( sao_flag ) { |  |  |
| sao_split_param( 0, 0, 0 ) |  |  |
| sao_offset_param( 0, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |

sao_flag equal to 0 denotes that SAO is disabled for the entire picture; equal to 1 denotes that SAO is enabled for at least a portion of the current picture.

| sao_split_param( x, y, Depth ) { | C | Descriptor |
| --- | --- | --- |
| if( Depth < MaxSplitLevel ) |  |  |
| split_flag[ Depth ][ y ][ x ] = sao_split_flag | 2 | u(1)¦ae(v) |
| else |  |  |
| split_flag[ Depth ][ y ][ x ] = 0 |  |  |
| if( split_flag[ Depth ][ y ][ x ] ) { |  |  |
| pqao_split_param( x + 0, y + 0, Depth + 1 ) |  |  |
| pqao_split_param( x + 1, y + 0, Depth + 1 ) |  |  |
| pqao_split_param( x + 0, y + 1, Depth + 1 ) |  |  |
| pqao_split_param( x + 1, y + 1, Depth + 1 ) |  |  |
| } |  |  |
| } |  |  |

sao_split_flag equal to 0 denotes that current region is a leaf; equal to 1 denotes that the current region is further spitted into four children region.

| sao_offset_param ( x, y, Depth ) { | C | Descriptor |
| --- | --- | --- |
| if( split_flag[ Depth ][ y ][ x ] ) { |  |  |
| sao_offset_param ( x + 0, y + 0, Depth + 1 ) |  |  |
| sao_offset_param ( x + 1, y + 0, Depth + 1 ) |  |  |
| sao_offset_param ( x + 0, y + 1, Depth + 1 ) |  |  |
| sao_offset_param ( x + 1, y + 1, Depth + 1 ) |  |  |
| } else { |  |  |
| type_idx[ Depth ][ y ][ x ] = sao_type_idx | 2 | ue(v)¦ae(v) |
| if( pqao_type_idx != 0 ) { |  |  |
| for( i = 0; i < PqaoOffsetNum[ sao_type_idx ]; i++ ) |  |  |
| offset[ Depth ][ y ][ x ][ i ] = sao_offset | 2 | se(v)¦ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

sao_type_idx indicates the offset type to be applied for the current region as follows.

# Simulation results

Simulation results of all six configurations in JCTVC-D600 [6] and three additional HE configurations with ALF-off are shown in Table 2. Compared with the JCTVC-D600 anchor, SAO achieves 0.5%, 1.3%, 2.2%, 0.7%, 1.8%, and 3.0% bit rate reductions for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively, with unnoticeable encoding time increase and 1-3% decoding time increase. When the ALF is turned off as the anchor, SAO can achieve 0.8%, 1.8%, and 2.5% bit rate reductions for HE-AI, HE-RA, and HE-LD, respectively, with unnoticeable encoding time increase and 2-3% decoding time increase.  shows the experimental data of slice-level BO without localization. The coding gain of BO-only is about 0.2-0.6% while the encoding time and decoding time are almost unchanged. Table 4 shows the experimental data of slice-level EO without localization. The coding gain of EO-only is about 0.4-1.3% while the encoding time and decoding time are increased by 1-5%.

# Conclusions

MediaTek developed a technique, sample adaptive offset, namely SAO, to reduce the distortion between decoded pixels and original pixels by simply adding an offset to each pixel. Pixel classification methods are used to create nonzero offsets that are beneficial for improving coding efficiency. In order to eliminate any additional line buffer and frame buffer access pass on the decoder side, the decoding process is designed to be LCU-independent. It is reported that SAO can achieve 0.5%, 1.3%, 2.2%, 0.7%, 1.8%, and 3.0% bit rate reductions for HE-AI, HE-RA, HE-LD, LC-AI, LC-RA, and LC-LD, respectively. The encoding time is almost unchanged, and the decoding time is increased by 1-3%.

Table 2  Experimental results of SAO (localized BO+EO)

Table 3  Experimental results of slice-level BO

Table 4  Experimental results of slice-level EO

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. McCann, W.-J. Han, I.-K. Kim, J.-H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y.-M. Hong, M.-S. Cheon, and N. Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” Document of Joint Collaborative Team on Video Coding, JCTVC-A124, Apr. 2010.

- Y.-W. Huang, C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y. Gao, J. An, K. Zhang, and S. Lei, “In-Loop Adaptive Restoration,” Document of Joint Collaborative Team on Video Coding, JCTVC-B077, July, 2010.

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “TE10 Subtest 3: Quadtree-based Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-C147, Oct. 2010.

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “TE8 Subtest3: Picture Quadtree-based Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-D122, Jan. 2011.

- Y.-W. Huang, “Description of Core Experiment 13: Sample Adaptive Offset,” Document of Joint Collaborative Team on Video Coding, JCTVC-D613, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.