| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E050 |
| --- | --- |

| Title: | AhG Parsing Robustness: Constrained Usage of Temporal MV and MVP Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Jian-Liang Lin Yi-Wen Chen Chi-Ling Wu Yu-Pao Tsai Yu-Wen Huang Shawmin Lei | Emails: | jl.lin@mediatek.com yiwen.chen@mediatek.com chiling.wu@mediatek.com yupao.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, new syntax elements are proposed in the sequence parameter set (SPS) and the slice header to constrain the use of temporal motion vector (MV) candidates and temporal motion vector predictor (MVP) candidates. The parsing error can thus be controlled within a few pictures instead of uncontrolled propagation to many pictures. The experiment results reported that disabling all temporal candidates causes 2.2% bit rate increase. However, disabling temporal candidates in all reference pictures reportedly reduces the bit rate increase to 1.1% for random access while the parsing error can thus be constrained within non-reference pictures and will not affect other pictures. The results also reported that the proposed control flag in the slice header provides multiple trade-off points between parsing error resilience and coding efficiency. The bit rate increase further reduces to 0.2% and 0.4% while the parsing error propagation is constrained within 16 and 8 pictures respectively.

- Introduction

For Inter and Skip, a temporal MVP is included in the candidate set of MVPs. For Merge, a temporal MV is included in the candidate set of MVs. However, using temporal candidates can cause parsing errors and thus severe error propagation. When an MV of a previous picture cannot be decoded correctly, a mismatch between the candidate set on the encoder side and that on the decoder side may occur, resulting in parsing error of the index of the best candidate. Then, the rest of the current picture cannot be parsed or decoded properly. What is even worse, this parsing error can affect subsequent inter pictures that also allow temporal candidates. One small decoding error of an MV may cause failure of parsing many subsequent pictures.

# Proposed syntax description

In JCTVC-D126 [1], we proposed to control the parsing error by deciding whether temporal candidates are allowed to use. Table 1 shows our syntax design of parsing error control. Modifications are highlighted with yellow background. A first flag, temporal_candidate_not_used, is transmitted in the SPS to disallow temporal candidate for the entire sequence. When the first flag is enabled, no parsing error propagation due to temporal candidates will occur, but the coding efficiency can be significantly degraded.

A second flag, temporal_candidate_not_used_in_ref_pic, will be transmitted in the SPS to disallow temporal candidates for reference pictures when the first flag is disabled. The second flag can provide another trade-off point between the parsing error resilience and the coding efficiency. When the second flag is enabled, only a non-reference picture which still uses temporal candidates may cause parsing error, which will not affect other pictures. Since temporal candidates are conditionally used, the coding efficiency will be better in comparison with entirely disallowing temporal candidates while the parsing error can be constrained within a non-reference picture.

A third flag, temporal_candidate_not_used_in_slice, will be transmitted in the slice header to disallow the temporal candidates for the current slice when the first flag is disabled and any one of the following two conditions is true. One condition is when the second flag is disabled, and the other condition is when the current picture is not a reference picture. The third flag can provide full flexibility of trade-off points between the parsing error resilience and the coding efficiency. When both the first flag and the second flag in the SPS are disabled and the third flag is enabled, the picture order count (POC) of each entry in both reference lists can be transmitted by reusing the reference picture list reordering syntax in the slice header to make sure the scaling derivation for the pictures referring the motion vectors in current slice as temporal candidates will be correct. Therefore, no parsing error propagation will be started from the current slice, and the parsing error caused from any prior slice will be terminated by the current slice.

Table 1  Syntax design of the proposed parsing error control

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| temporal_candidate_not_used | 1 | u(1) |
| if( !temporal_candidate_not_used ) |  |  |
| temporal_candidate_not_used_in_ref_pic | 1 | u(1) |
| … |  |  |
| } |  |  |

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| ... |  |  |
| if( !temporal_candidate_not_used && ( !temporal_candidate_not_used_in_ref_pic ¦¦ !nal_ref_idc ) ) |  |  |
| temporal_candidate_not_used_in_slice | 2 | u(1) |
| ... |  |  |
| } |  |  |

temporal_candidate_not_used equal to 1 denotes that temporal candidates are not used for the entire sequence; equal to 0 denotes that temporal candidates may be used in the sequence.

temporal_candidate_not_used_in_ref_pic equal to 1 denotes that temporal candidates are not used in reference pictures but used in non-reference pictures; equal to 0 denotes that temporal candidates may be used in reference pictures and non-reference pictures. When the temporal_candidate_not_used is equal to 1, temporal_candidate_not_used_in_ref_pic is set as 1.

temporal_candidate_not_used_in_slice equal to 1 denotes that temporal candidates are not used in the current slice; equal to 0 denotes that temporal candidates are used in the current slice. The temporal_candidate_not_used_in_slice will be set as 1 if the temporal_candidate_not_used is equal to 1 or the temporal_predictor_not_used_in_ref_pic is equal to 1 and the current picture is a reference picture.

# Simulation results

Several experiments are conducted to evaluate the coding efficiency and error resilience by disabling temporal candidates. The evaluation was performed using a software implementation based on HM2.0, and the JCTVC-D600 [2] common test configurations are used as anchors.

First, we disable temporal candidates for all pictures. The results are shown in Table 2. The bit rate increases are 2.3%, 2.2%, 1.7%, and 2.4% for HE-RA, LC-RA, HE-LD, LC-LD, respectively.

Table 2  Results of disabling temporal candidates for all pictures

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.1 | 2.6 | 2.3 | 3.4 | 2.8 | 3.0 |
| Class B | 2.2 | 1.3 | 1.3 | 1.8 | 0.7 | 0.7 |
| Class C | 1.9 | 1.6 | 1.7 | 1.9 | 1.5 | 1.6 |
| Class D | 2.1 | 1.5 | 1.6 | 2.0 | 1.1 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 1.7 | 1.7 | 2.2 | 1.5 | 1.6 |
| Enc Time[%] | 96% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.5 | 1.4 | 3.0 | 1.5 | 1.7 |
| Class C | 1.8 | 2.0 | 1.9 | 2.7 | 1.9 | 1.8 |
| Class D | 1.4 | 0.9 | 0.8 | 1.9 | 1.0 | 0.8 |
| Class E | 1.3 | 1.2 | 1.8 | 2.0 | 0.4 | 1.5 |
| All | 1.7 | 1.4 | 1.4 | 2.4 | 1.3 | 1.4 |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  |

Second, we disable temporal candidates for all reference pictures in RA configurations. The results are shown in Table 3. The bit rate increases are 1.1% and 1.0% for HE-RA and LC-RA, respectively, while the parsing error can thus be constrained within a non-reference picture and will not affect other pictures.

Table 3  Results of disabling temporal candidates for all reference pictures

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.6 | 1.6 | 1.3 | 1.7 | 1.8 | 2.0 |
| Class B | 1.0 | 0.6 | 0.6 | 0.8 | 0.4 | 0.4 |
| Class C | 0.8 | 0.7 | 0.7 | 0.7 | 0.6 | 0.6 |
| Class D | 1.2 | 0.8 | 0.8 | 1.0 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 1.1 | 0.9 | 0.8 | 1.0 | 0.8 | 0.9 |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Three more experiments are conducted. We periodically disable temporal candidates in every 4, 8 and 16 pictures, and the results are shown in Table 4, Table 5, and Table 6, respectively. It is observed that the bit rate increases are about 0.6%, 0.4%, and 0.2% when temporal candidates are disabled for every 4, 8, and 16 pictures, respectively. Multiple trade-off points between parsing error control and coding efficiency can thus be achieved by the temporal_predictor_not_used_in_slice flag.

# Conclusions

MediaTek proposed two flags in the SPS and one flag in the slice header to allow flexible trade-off points between parsing error resilience and coding efficiency. The experimental results reportedly showed that bit rate increases are 0.2% and 0.4% while the parsing error propagation is constrained within 16 and 8 pictures, respectively. It is suggested to adopt the proposed syntax design into HM.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 4  Results of disabling temporal candidates for every 4 pictures

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.9 | 0.6 | 0.8 | 1.0 | 1.1 |
| Class B | 0.5 | 0.3 | 0.2 | 0.4 | 0.3 | 0.3 |
| Class C | 0.4 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Class D | 0.9 | 0.5 | 0.5 | 0.7 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.5 | 0.4 | 0.6 | 0.4 | 0.5 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 0.4 | 0.2 | 0.7 | 0.2 | 0.5 |
| Class C | 0.6 | 0.5 | 0.6 | 0.7 | 0.3 | 0.2 |
| Class D | 0.7 | 0.4 | 0.1 | 0.7 | 0.2 | 0.2 |
| Class E | 0.3 | -0.1 | 0.4 | 0.2 | -0.7 | 0.1 |
| All | 0.6 | 0.3 | 0.3 | 0.6 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 5  Results of disabling temporal candidates for every 8 pictures

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.7 | 0.6 | 0.7 | 0.7 | 0.8 |
| Class B | 0.3 | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 |
| Class C | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.7 | 0.3 | 0.4 | 0.5 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.3 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.2 | 0.2 | 0.3 | 0.0 | 0.1 |
| Class C | 0.4 | 0.4 | 0.2 | 0.3 | 0.0 | -0.1 |
| Class D | 0.3 | 0.0 | 0.2 | 0.3 | 0.1 | 0.2 |
| Class E | 0.1 | 0.2 | 0.6 | 0.0 | -0.2 | -0.1 |
| All | 0.3 | 0.2 | 0.3 | 0.3 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 6  Results of disabling temporal candidates for every 16 pictures

|  | HE-RA |  |  | LC-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.7 | 0.3 | 0.5 | 0.5 | 0.5 |
| Class B | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.3 | 0.1 | 0.1 | 0.3 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.2 | 0.1 | 0.2 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | HE-LD |  |  | LC-LD |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.3 |
| Class C | 0.2 | 0.4 | 0.3 | 0.2 | 0.1 | -0.2 |
| Class D | 0.1 | -0.1 | 0.0 | 0.2 | 0.0 | 0.1 |
| Class E | -0.1 | -0.4 | 0.0 | -0.2 | -0.2 | 0.3 |
| All | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- J.-L. Lin, Y.-W. Huang, C.-M. Fu, C.-Y. Chen, Y.-P. Tsai, and S. Lei, “Parsing Error Control for Advanced Motion Vector Prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-D126, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.