| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E051 |
| --- | --- |

| Title: | AhG Quantization: Sub-LCU Delta QP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Tzu-Der Chuang Ching-Yeh Chen Yu-Lin Chang Yu-Wen Huang Shawmin Lei | Emails: | peter.chuang@mediatek.com chingyeh.chen@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution described MediaTek’s proposal on combination of delta quantization parameter (dQP) syntax designs from JCTVC-D038 and JCTVC-D258. In HEVC Working Draft 1, a dQP was transmitted at the end of each 64x64 largest coding unit (LCU). In JCTVC-D038, TI proposed to send dQP with a finer granularity for better rate control. Subsequently, the requirement of sub-LCU dQP has been supported by six companies including Canon, ETRI, MediaTek, Panasonic, Sony, and TI. In JCTVC-D258, Sony proposed to send dQP right before the first nonzero quantized transform coefficient for better pipelined processing, which was adopted at the last JCT-VC meeting. In this contribution, a combined design of a finer dQP granularity and receiving dQP before quantized transform coefficients is proposed. The proposed design is integrated on top of HM-2.0-dev (rev609) software with matching results from its decoder and encoder.

- Introduction

For video streaming and video broadcasting application, rate control and perceptual optimization are important functionalities that prevent bit rate overflow or underflow while maintaining video quality. Moreover, recently, HD-resolution video applications become more and more popular, and for such applications subjective quality control is very important. To provide the functionalities of rate control and perceptual optimization, HEVC also uses dQP. However, different from the dQP that is adjustable for every 16x16 macroblock in H.264/AVC, the current HEVC software encodes a dQP for every 64x64 LCU. If QP can be determined only at 64x64-block level, the unit for rate control would be too coarse and rate control would be difficult. Having sub-LCU QP adaptation can solve this problem. Moreover, several JCT-VC participants are evaluating sub-LCU slice partition. If sub-LCU slice granularity is adopted, it would conflict with LCU-level dQP. This conflict can be naturally solved by a sub-LCU dQP design. In this contribution, a sub-LCU dQP design is proposed to provide finer granularity of rate and visual quality control. The rest of this contribution is organized as follows. The algorithm will be described in Section 2, simulation results will be discussed in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## LCU-based dQP in HM2.1

Figure 1 shows the dQP signaling in HM-2.0-dev (rev 609). The dQP is transmitted in the first non-SKIP leaf-CU [2]. The granularity of dQP signaling is at LCU level.

Figure 1  Example of the dQP signaling in HM-2.0-dev (rev 609)

## Proposed sub-LCU dQP

An adaptive sub-LCU dQP signaling which combines the methods in JCTVC-D038 [1] and JCTVC-D258 [2] is proposed. Instead of using only one dQP for each LCU, a QP minimum CU size (QpMinCuSize) is sent as the minimum CU size that can signal a dQP. The QpMinCuSize is first defined in the sequence parameter set (SPS) and can be changed in the slice header. For each leaf CU that is larger than or equal to the QpMinCuSize and has at least one nonzero coefficient, a dQP is transmitted. For each non-leaf CU that is equal to the QpMinCuSize and is split into children and has at least one nonzero coefficient, a dQP is also transmitted and all leaf CUs in this non-leaf CU share the same dQP.

Figure 2 shows an example of the proposed dQP signaling. In this example, the LCU size is 64x64 and the QpMinCuSize is 32x32. In LCU0, CU0 is the only leaf CU. CU0 is larger than QpMinCuSize and has at least one nonzero coefficient, a dQP is sent right after the coded_block_flag (CBF). In LCU1, CU1 and CU10 are 32x32, and CU2-CU9 are 16x16. Therefore, CU1 has its own QP, CU2-CU5 share one QP, CU6-CU9 share one QP, and CU10 has its own QP. CU1 sends a dQP in the bitstream because it has at least one nonzero coefficient. CU2-CU5 have no dQP in the bitstream because they do not have any nonzero coefficient. CU6-CU9 send one dQP right after the CBF of CU7 in the bitstream because CU7 has at least one nonzero coefficient. CU10 has no dQP in the bitstream because it does not have any nonzero coefficient.

Figure 2  Example of the proposed dQP signaling

The proposed syntax is shown Table 1. Modifications are highlighted with yellow background. As can be seen, QpMinCuSize can be determined in the SPS first and modified by each slice in the slice header individually. A dQP is transmitted for a leaf CU if it is larger than or equal to the QpMinCuSize and has at least one nonzero coefficient. A dQP is also transmitted for a non-leaf CU if it is equal to the QpMinCuSize and is split into children and has at least one nonzero coefficient.

In our implementation, a dQP is the current QP value subtracted by the slice QP value. The prediction method of the QP value is an orthogonal issue of the proposed dQP syntax and can be combined into our design easily without any conflict. For example, in JCTVC-D038, TI proposed a method to achieve better prediction of QP values.

# Simulation results

The proposed approach is implemented on HM-2.0-dev (rev 609). All six test configurations were simulated according to JCTVC-D600 [3]. However, for the sake of saving time, only 1/10 frames were tested. In order to generate nonzero dQP values, three dQP values (-1, 0, 1) were tested during encoding (the xCompressCU function in the software), and the one with the lowest rate-distortion cost was chosen. The results are shown in Table 2, Table 3, Table 4, and Table 5. All bitstreams can be decoded correctly, which indicates that our proposed sub-LCU dQP signaling is correctly implemented.

# Conclusions

MediaTek proposed a sub-LCU dQP signaling approach combining desirable features from JCTVC-D038 and JCTVC-D258. In this method, a QpMinCuSize is first defined. For each leaf CU that is larger than or equal to the QpMinCuSize and has at least one nonzero coefficient, a dQP is sent. For each non-leaf CU that is equal to the QpMinCuSize and is split into children and has at least one nonzero coefficient, a dQP is also sent. In this way, the fine granularity in JCTVC-D038 and the pipelining friendliness in JCTVC-D258 can be both achieved. A software implementation based on HM-2.0-dev (rev 609) was verified and no mismatch was found between the encoder and the decoder.

Table 1  Proposed syntax modification

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_qp_max_depth | ue(v) |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| change_qp_max_depth_flag | u(1) |
| if( change_qp_max_depth_flag ) { |  |
| sh_qp_max_depth | ue(v) |
| QpMinCuSize = MaxCuSize >> sh_qp_max_depth |  |
| } else { |  |
| QpMinCuSize = MaxCuSize >> sps_qp_max_depth |  |
| } |  |
| } |  |

| coding_tree( x0, y0, log2CuSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CuSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CuSize ) <= PicHeightInSamplesL && log2CUSize > Log2MinCuSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| if( CurrCuSize = = QpMinCuSize ) |  |
| QpAlreadyCoded = 0 |  |
| coding_tree( x0, y0, log2CuSize – 1 ) |  |
| … |  |
| } else { |  |
| if( CurrCuSize >= QpMinCuSize ) |  |
| QpAlreadyCoded = 0 |  |
| coding_unit( x0, y0, log2CuSize ) |  |
| } |  |
| } |  |

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && coded_block_flag_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && coded_block_flag_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && coded_block_flag_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( QpAlreadyCoded = = 0 ){ |  |
| delta_qp | se(v)¦ae(v) |
| QpAlreadyCoded = 1 |  |
| } |  |
| … |  |
| } |  |
| } |  |

Table 2  Experimental results of QpMinCuSize equal to 64x64

|  | HE-AI | LC-AI | HE-RA | LC-RA | HE-LD | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.7 | -0.3 | 0.2 | -1.1 |  |  |
| Class B | -0.7 | -0.6 | -0.8 | -0.8 | -0.7 | -0.9 |
| Class C | -0.5 | -0.4 | -1.2 | -1.3 | -1.0 | -1.2 |
| Class D | -0.4 | -0.4 | -1.0 | -0.7 | -0.8 | -0.9 |
| Class E | -1.0 | -1.0 |  |  | -1.1 | -0.8 |
| All | -0.6 | -0.5 | -0.7 | -1.0 | -0.9 | -0.9 |
| Enc Time[%] | 250% | 294% | 283% | 299% | 286% | 298% |
| Dec Time[%] | 100% | 101% | 98% | 97% | 97% | 96% |

Table 3  Experimental results of QpMinCuSize equal to 32x32

|  | HE-AI | LC-AI | HE-RA | LC-RA | HE-LD | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.9 | -0.6 | 0.2 | -0.8 |  |  |
| Class B | -0.9 | -0.7 | -0.7 | -0.4 | -0.4 | -0.2 |
| Class C | -0.9 | -0.7 | -1.2 | -0.9 | -0.9 | -0.8 |
| Class D | -0.7 | -0.6 | -1.2 | -0.6 | -0.8 | -0.4 |
| Class E | -1.2 | -0.9 |  |  | -1.0 | -0.3 |
| All | -0.9 | -0.7 | -0.7 | -0.7 | -0.8 | -0.4 |
| Enc Time[%] | 251% | 293% | 285% | 299% | 285% | 297% |
| Dec Time[%] | 99% | 100% | 98% | 97% | 96% | 97% |

Table 4  Experimental results of QpMinCuSize equal to 16x16

|  | HE-AI | LC-AI | HE-RA | LC-RA | HE-LD | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.9 | -0.2 | 0.3 | 0.0 |  |  |
| Class B | -0.8 | -0.2 | -0.4 | 0.4 | -0.2 | 0.7 |
| Class C | -0.9 | -0.5 | -1.0 | -0.3 | -0.6 | 0.1 |
| Class D | -0.8 | -0.5 | -0.9 | -0.1 | -0.5 | 0.4 |
| Class E | -0.7 | 0.1 |  |  | -0.6 | 0.7 |
| All | -0.8 | -0.3 | -0.5 | 0.0 | -0.5 | 0.5 |
| Enc Time[%] | 249% | 292% | 284% | 298% | 286% | 299% |
| Dec Time[%] | 100% | 101% | 98% | 97% | 98% | 97% |

Table 5  Experimental results of QpMinCuSize equal to 8x8

|  | HE-AI | LC-AI | HE-RA | LC-RA | HE-LD | LC-LD |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate | Y BD-rate |
| Class A | -0.6 | 0.4 | 0.4 | 0.5 |  |  |
| Class B | -0.5 | 0.7 | -0.3 | 1.0 | -0.3 | 1.1 |
| Class C | -0.5 | 0.6 | -0.7 | 0.6 | -0.6 | 1.0 |
| Class D | -0.4 | 0.5 | -0.7 | 0.7 | -0.4 | 1.2 |
| Class E | -0.2 | 1.1 |  |  | -1.2 | 1.0 |
| All | -0.5 | 0.6 | -0.3 | 0.7 | -0.6 | 1.1 |
| Enc Time[%] | 248% | 287% | 282% | 296% | 285% | 298% |
| Dec Time[%] | 99% | 100% | 98% | 98% | 98% | 98% |

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Budagavi and M. Zhou, “Delta QP signaling at sub-LCU level,” Document of Joint Collaborative Team on Video Coding, JCTVC-D038, Jan. 2011.

- L. Dong, W. Liu, and K. Sato, “CU Adaptive Quantization Syntax Change for Better Decoder Pipelining,” Document of Joint Collaborative Team on Video Coding, JCTVC-D258, Jan. 2011.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.