| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E052 |
| --- | --- |

| Title: | Removal of Duplicated Combinations of Reference Picture Indices for Forward Bi-Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Tzu-Der Chuang Jian-Liang Lin Yu-Wen Huang Shawmin Lei | Emails: | peter.chuang@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution described MediaTek’s proposal on reducing duplicated combinations of reference picture indices for bi-prediction in the forward B-slices whose LIST_0 reference pictures and LIST_1 reference pictures are identical. CABAC and LCEC are modified accordingly to avoid syntax redundancy, and motion estimation is skipped for the removed combinations of reference picture indices. The encoding time was reportedly reduced by 15-26% with 0.2-0.6% bit rate increase in comparison with the anchor in JCTVC-D600.

# Introduction

In the forward B-slices, the reference picture list 0 (LIST_0) and the reference picture list 1 (LIST_1) are identical, and there exists prediction redundancy in the uni-prediction (i.e. single list prediction) and the bi-prediction. In order to reduce the prediction redundancy, the concept of combined reference picture list (LIST_C) was adopted in HM2.0 to unify the reference picture list for the uni-prediction. However, there still exists prediction redundancy in the forward bi-prediction which should be further removed. The rest of this contribution is organized as follows. The prediction redundancy will be explained in Section 2, reduced combinations of reference picture indices will be proposed in Section 3, simulation results will be shown in Section 4, and conclusions will be given in Section 5.

# Prediction redundancy

Since the reference pictures in LIST_0 and LIST_1 are the same in forward B-slices, there exists prediction redundancy. Figure 1 shows two bi-prediction situations in forward B-slices. In Figure 1 (a), the motion vector (MV) in LIST_0 points to reference picture A (ref_A) and the MV in LIST_1 points to reference picture B (ref_B), and in Figure 1 (b), the MV in LIST_0 points to ref_A and the MV in LIST_1 points to ref_B. However, the motion compensation results of these two situations are the same, which means one of them is redundant.

Figure 1  Example of prediction redundancy in forward B-slices

The concept of the prediction redundancy can be further explained in Figure 2, which shows the combinations of reference picture indices (refIdx) of bi-prediction in forward B-slices. In the current common test configurations in JCTVC-D600 [1], each list in a B-slice has four reference pictures. In the table shown in Figure 2, the refIdx combination A is equivalent to the refIdx combination B, which means either A or B is redundant. Furthermore, the refIdx combinations in the bottom-left (BL) triangle are equivalent to those in the upper-right (UR) triangle. The motion estimation procedure for one of these triangles of refIdx combinations can be skipped to reduce encoding time.

Figure 2  Combinations of reference picture indices of bi-prediction in forward B-slices

# Reduced combinations of reference picture indices

In this contribution, it is proposed to skip the motion estimation for the BL-triangle in Figure 2 to reduce encoding time. The enabled refIdx combinations are shown in Figure 3, which is composed of the uni-prediction of LIST_C (which is equal to LIST_0) and the bi-prediction in the UR-triangle of Figure 2.

Figure 3  Proposed combinations of reference picture indices

Since the BL-triangle of refIdx combinations is removed, less refIdx combinations are required to be encoded, and the code words of remaining refIdx combinations should be re-designed to avoid the syntax redundancy. In CABAC, the code words of refIdx of LIST_1 are modified in this contribution. After determining the refIdx of LIST_0, the refIdx range of LIST_1 is limited. Figure 4 shows the modified binarized code word of LIST_1. The code word length is reduced for (LIST_0 refIdx, LIST_1 refIdx) equal to (0,0), (1,1), (2,2).

Figure 4  Binarized code words of refIdx of LIST_1

For LCEC, a look-up table is used to jointly encode the inter prediction direction and the reference picture index. This look-up table is modified to avoid syntax redundancy as well. Figure 5 (a) shows the original look-up table for combinations of inter prediction direction and reference picture index. The entry #5 in Figure 5 (a) is inside the disallowed triangle of this proposal and is moved to represent another combination as shown in Figure 5 (b). The entry #7 is moved as well for better coding efficiency.

Figure 5  LCEC look-up tables

# Simulation results

The proposed approach was implemented on HM2.0. The HE-LD and LC-LD configurations were simulated according to JCTVC-D600. Table 1 shows the experimental results of coding only the inter-prediction combinations in Figure 3. The encoding time is reduced by 19% and 26% with 0.6% and 0.4% bit rate increases for HE-LD and LC-LD, respectively.

Table 1  Results of skipping the BL-triangle of refIdx combinations with syntax change

|  | HE-LD |  |  | LC-LD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.9 | 0.8 | 0.6 | 0.9 | 1.1 |
| Class C | 0.6 | 0.6 | 0.7 | 0.3 | 0.4 | 0.2 |
| Class D | 0.5 | 0.5 | 0.3 | 0.4 | 0.3 | 0.3 |
| Class E | 0.6 | 0.6 | 1.0 | 0.4 | 0.6 | 0.3 |
| All | 0.6 | 0.7 | 0.7 | 0.4 | 0.5 | 0.5 |
| Enc Time[%] | 81% |  |  | 74% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |

Because we removed reference index combinations from the BL-triangle as shown in Figure 3, we further modified the derivation of spatial MV or MVP candidates based on JCTVC-D125 [2] to allow spatial candidates to be derived from LIST_0 or LIST_1. The results are shown in Table 2. The encoding time is reduced by 15% and 21% with 0.5% and 0.2% bit rate increases for HE-LD and LC-LD, respectively.

# Conclusions

MediaTek proposed a method to reduce duplicated combinations of reference picture indices for forward bi-prediction. The encoding time is reportedly reduced by 15-26% with 0.2-0.6% BD-rate increases in comparison with the JCTVC-D600 anchor. It is suggested to adopt this technique in HM.

Table 2  Results of further modifying the derivation of spatial MV or MVP candidates

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.6 | 1.0 | 0.8 | 0.4 | 0.9 | 1.2 |
| Class C | 0.4 | 0.6 | 0.5 | -0.1 | 0.1 | 0.1 |
| Class D | 0.5 | 0.4 | -0.1 | 0.2 | 0.8 | 0.6 |
| Class E | 0.5 | 0.5 | 0.5 | 0.2 | 0.8 | 0.4 |
| All | 0.5 | 0.6 | 0.4 | 0.2 | 0.6 | 0.6 |
| Enc Time[%] | 85% |  |  | 79% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-D600, Jan. 2011.

- J.-L. Lin, Y.-P. Tsai, Y.-W. Huang, and S. Lei, “Improved Advanced Motion Vector Prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-D125, Jan. 2011.