| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E053 |
| --- | --- |

| Title: | Unified Syntax of Reference Picture List Reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Chih-Wei Hsu Jian-Liang Lin Yu-Wen Huang Shawmin Lei | Emails: | cw.hsu@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a unified syntax for reference picture list reordering was proposed. The syntax can support reference picture list reordering not only for the list 0 and the list 1 in the bi-prediction mode but also for the combined list in the single list prediction mode in HEVC.

- Introduction

In AVC, there are two lists, list 0 and list 1, for B-slices. Each list can contain multiple reference pictures. In the slice header, it is allowed to specify how to map each reference picture index to a reference picture of a list by users instead of using the default mapping. This is called reference picture list reordering. In HEVC, in addition to the list 0 and the list 1, a new list called “combined list” was proposed in JCTVC-D421 [1] and adopted. The combined list interleaves list 0 and list 1 starting with list 0 and eliminating all redundant reference pictures. When the single list prediction mode (i.e. one motion vector, only one reference picture can be used) is used for a prediction unit (PU), the combined list is used. When the bi-prediction mode (i.e. two motion vectors, up to two reference pictures can be used) is used for a PU, the list 0 and the list 1 are used. In this contribution, a unified syntax of reference picture list reordering is proposed for the combined list as well as the two individual lists. The rest of this document is organized as follows. Details of the syntax will be described in Section 2. Conclusions will be given in Section .

# Proposed syntax

## Reference picture list reordering for combined list

The following is an example of a group of pictures in hierarchical B structure.

POC denotes “picture order count”, which is the same as display order. Take picture 2 (POC 2) for instance, for list 0 of picture 2, the first reference picture index maps to picture 0, and the second reference picture index maps to picture 4. For list 1 of picture 2, the first reference picture index maps to picture 4, and the second reference picture index maps to picture 8. In this example, the default method is used for reconstruction of list 0 and list 1. In the combined list, picture 0 (the first reference picture in list 0) is mapped to index 0, picture 4 (the first reference picture in list 1) is mapped to index 1, and then picture 4 (the second reference picture in list 0) is not mapped to index 2 due to its redundancy, and finally picture 8 (the second reference picture in list 1) is mapped to index 2. The above is the default reconstruction of the combined list. However, reference picture list reordering has not been clearly defined for the combined list in HEVC. Therefore, we propose a syntax design and process of reference picture list reordering for the combined list.

## Syntax description

The proposed syntax is shown in Table 1. Modifications are highlighted with yellow background. The “ref_pic_list_reordering_flag_lc” has the similar meaning to “ref_pic_list_reordering_flag_l0” or “ref_pic_list_reordering_flag_l1” in AVC except that it is applied to the combined list instead of the list 0 or the list 1. Accompanying with reordering_of_pic_nums_idc, which is a command to specify the reordering of reference picture lists, the reordering process could be represented more efficiently by this syntax than that in JCTVC-D421, which was just to straightforwardly specify the modified reference picture lists after reordering. The commands of reordering_of_pic_nums_idc are shown in Table 2. Moreover, our proposed syntax for the combined list is unified with those for the list 0 and the list 1, which have been used in AVC since 2003.

With this syntax, reference picture list reordering for combined list will proceed as follow. First, reference picture list reordering is applied for the list 0 and the list 1, respectively. Next, the initial reconstruction of the combined list interleaves reordered list 0 and list 1 starting with list 0 and eliminating all redundant reference pictures. Finally, reference picture list reordering is applied for the combined list.

# Conclusions

In this contribution, a unified syntax for reference picture list reordering was proposed. The syntax can support reference picture list reordering not only for the list 0 and the list 1 in the bi-prediction mode but also for the combined list in the single list prediction mode in HEVC.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y. Suzuki, TK Tan, “Extension of Uni-prediction Simplification in B Slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-D421, Jan. 2011.

Table 1  Syntax design for reference picture list reordering

| ref_pic_list_reordering( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) { |  |  |
| ref_pic_list_reordering_flag_l0 | 2 | u(1)¦ae(v) |
| if( ref_pic_list_reordering_flag_l0 ) { |  |  |
| do { |  |  |
| reordering_of_pic_nums_idc | 2 | ue(v)¦ae(v) |
| if( reordering_of_pic_nums_idc = = 0 ¦ ¦ reordering_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v)¦ae(v) |
| else if( reordering_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v)¦ae(v) |
| } while( reordering_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |
| if( slice_type = = B ) { |  |  |
| ref_pic_list_reordering_flag_l1 | 2 | u(1)¦ae(v) |
| if( ref_pic_list_reordering_flag_l1 ) { |  |  |
| do { |  |  |
| reordering_of_pic_nums_idc | 2 | ue(v)¦ae(v) |
| if( reordering_of_pic_nums_idc = = 0 ¦ ¦ reordering_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v)¦ae(v) |
| else if( reordering_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v)¦ae(v) |
| } while( reordering_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| ref_pic_list_reordering_flag_lc | 2 | u(1)¦ae(v) |
| if( ref_pic_list_reordering_flag_lc ) { |  |  |
| do { |  |  |
| reordering_of_pic_nums_idc | 2 | ue(v)¦ae(v) |
| if( reordering_of_pic_nums_idc = = 0 ¦ ¦ reordering_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v)¦ae(v) |
| else if( reordering_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v)¦ae(v) |
| } while( reordering_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

ref_pic_list_reordering_flag_lc equal to 1 specifies that the syntax element reordering _of_pic_nums_idc is present for specifying combined list, equal to 0 specifies that this syntax element is not present.

reordering_of_pic_nums_idc together with abs_diff_pic_num_minus1 or long_term_pic_num specifies which of the reference pictures are re-mapped. The values of reordering _of_pic_nums_idc are specified in Table 2. The value of the first reordering_of_pic_nums_idc that follows immediately after ref_pic_list_reordering_flag_lc shall not be equal to 3.

Table 2  Operations specified by reordering_of_pic_nums_idc

| reordering_of_pic_nums_idc | reordering specified |
| --- | --- |
| 0 | abs_diff_pic_num_minus1 is present and corresponds to a difference to subtract from a picture number prediction value |
| 1 | abs_diff_pic_num_minus1 is present and corresponds to a difference to add to a picture number prediction value |
| 2 | long_term_pic_num is present and specifies the long-term picture number for a reference picture |
| 3 | End loop for modification of the initial reference picture list |

abs_diff_pic_num_minus1 plus 1 specifies the absolute difference between the picture number of the picture being moved to the current index in the list and the picture number prediction value.

long_term_pic_num specifies the long-term picture number of the picture being moved to the current index in the list. When decoding a coded frame, long_term_pic_num shall be equal to a LongTermPicNum assigned to one of the reference frames or complementary reference field pairs marked as "used for long-term reference". When decoding a coded field, long_term_pic_num shall be equal to a LongTermPicNum assigned to one of the reference fields marked as "used for long-term reference".