| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E054 |
| --- | --- |

| Title: | Preliminary complexity assessment on ARM |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | Daniele Alfonso STMicroelectronics Via Tolomeo 1 Cornaredo, 20010 MI, Italy Andrea C. Ornstein STMicroelectronics Via Tolomeo 1 Cornaredo, 20010, MI, Italy | Tel: +39.02.93519755 Email: daniele.alfonso@st.com Tel: +39.02.93519743 Email: andrea.ornstein@st.com |  |  |
| Source: | Complexity assessment AHG |  |  |  |

_____________________________

# Abstract

This contribution presents the results of a preliminary assessment of HM 2.0 decoder complexity on ARM architecture, in particular a Dual Cortex-A9 CPU integrated in the ST-Ericsson U8500 System-on-Chip. The authors assert that ARM platforms are relevant in the current market status and they propose to conduct further complexity analysis on ST-Ericsson experimental boards.

# Problem Statement

Until now, the complexity efficiency of the HM reference software was mainly assessed by measuring the encoding and decoding execution time on PC platform, meaning that the Intel Architecture was implicitly selected as reference platform to evaluate the complexity of HEVC.

Considering the current market situation, the authors would like to consider alternative hardware platforms for complexity measurements, in particular the ARM CPU architecture. For this purpose, the HM 2.0 software was compiled and executed on a prototype board of the ST-Ericsson U8500, a System-on-Chip for mobile devices that integrates an ARM Dual Cortex-A9 CPU. The board, which runs at a clock frequency of 450 Mhz (slower than the actual final product), mounted a Linux operating system with kernel 2.6.29 and g++ compiler version 4.4.1.

Fig.1: the U8500 evaluation board.

Table 1: system characteristics.

# Results

The HM 2.0 decoder was compiled with Linux makefile (using ‘make release’ option) and executed to process the reference anchor bitstreams on two different systems: the U8500 experimental board and a PC platform. For this first test we considered only the “low delay” streams, in both “low complexity” and “high efficiency” configurations.

Since the U8500 board mounts a total of 200 MB of physical memory, it was not possible to decode sequences with spatial resolution greater than 1280x720, so that the bitstreams containing the sequences BQTerrace, BasketballDrill, Cactus, Kimono1and ParkScene - all having 1920x1080 spatial resolution – were not processed. It is remarked that in the “low delay” configuration the bit-streams are encoded using 4 reference frames, which means that at 1920x1080 resolution the decoder must allocate more than 
15 MB for the 4 reference pictures plus the current one. Furthermore, it is reported that the Vidyo4 bitstreams resulted in a segmentation fault for QP = 32 and 37 in the LC-HE configuration, for reason not yet investigated. For the sake of a fair comparison, all the “low delay” bitstreams not decoded on the board were excluded from the final evaluation.

Table 1 summarizes the main hardware and software characteristics of the two systems used for this experiment, whereas Table 2 shows the results obtained as user execution time reported by the HM decoder. On Intel architecture the average time ratio between HE and LC configuration resulted 1.77 (minimum 1.16, maximum 2.23), whereas on ARM architecture it resulted 1.60 (minimum 1.12, maximum 2.1).

# Conclusions

Considering the relevance of the ARM architecture in the market of multimedia devices, the authors would like to continue with further complexity assessment experiments on ST-Ericsson evaluation boards and they would like to present new results at the coming JCT-VC meetings, in the scope of the Complexity Assessment AHG.

Table 2: HM 2.0 decoding time results on Intel (at 2.4 GHz) and ARM (at 450 MHz) platforms.

# References

- D.Alfonso, “Proposals for video coding complexity assessment”, JCTVC-B020, 2nd JCT-VC meeting, July 2010, Geneva, CH.

- ST-Ericsson NOVATHORTM U8500, http://www.stericsson.com/platforms/U8500.jsp

- ARM CortexTM-A9 Processor, http://www.arm.com/products/processors/cortex-a/cortex-a9.php

- Intel® Core™2 Duo Processor P8600, http://ark.intel.com/Product.aspx?id=35568