| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E055 |
| --- | --- |

| Title: | Evaluation of decoder-side motion estimation within HM 2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sven Klomp Jörn Ostermann Institut für Informationsverarbeitung Appelstraße 9a 30167 Hannover Germany | Tel: Email: | +49-511-762-19590 klomp@tnt.uni-hannover.de |
| Source: | Leibniz Universität Hannover |  |  |

_____________________________

# Abstract

Decoder-side motion estimation (DSME) is used to reduce the bit rate needed to transmit the motion information from the encoder to the decoder. Simulation results based on the JM reference software were previously presented (JCTVC-B026). This contribution presents the coding performance within the HM 2.0 reference software. Bit rate reduction of 2.27% and 1.76%  is achieved in RA-HE mode for Class A and Class B, respectively.

Furthermore, the performance of DSME in conjunction with the DMVD approach proposed in JCTVC-E294 is evaluated in this contribution. The bit rate is reduced by 2.60% and 2.34% using the combined approach compared to the HM-2.0 anchors.

# Introduction

Decoder-side motion estimation (DSME) is an approach to reduce the rate used to code motion information. Temporal interpolation is used to generate a prediction of the current frame at the decoder. This DSME frame is stored and can be used as additional reference for the conventional coding tools. The motion estimated by the conventional tools should be zero, as the DSME frame is already motion compensated and thus, the motion vectors are very efficient to code. A detailed description of the architecture can be found in [1]. However, those results base on JM 16.2 [2] and the CfP anchors [3]. To get an evaluation of the performance in current HEVC test model, DSME was implemented into HM 2.0.

## Combination of DMVD and DSME

DSME bases on the assumption that motion between frames is homogeneous. The motion compensated prediction proposed in [1] can only handle objects, which perform the same motion from one frame to the second and from the second to a third frame as depicted in Figure 1.

Figure 1: Wrong motion compensation for nonlinear motion

The DSME algorithm estimates the true motion between the two reference frames F− and F+ and inserts the interpolated block in the centre of F0 because of the assumption of constant motion. However, the true motion drawn with dashed arrows is accelerated, and the correct position of the block is in the upper left corner. Thus, DSME is not able to create a valid prediction of the current frame.

In DMVD, the motion of neighbouring pixel is estimated and used as motion information for the current block. Thus, spatial constant motion is assumed.

Although both approaches, DMVD and DSME, try to reduce the rate used for transmitting the motion vectors, different assumptions on the motion are made as mentioned before. DMVD assumes constant motion of spatial neighbouring pixel, whereas DSME expects constant motion over time. A combination of both approaches can lead to additional rate gains. Therefore, the combination of the tools is also evaluated within this contribution.

A simplified block diagram of a decoder containing DMVD as well as DSME is shown in Figure 2. Since the interpolation for DSME is frame-based, it can be separated from DMVD by inserting the DSME frame into the reference picture buffer before coding the current frame. DMVD and the other coding tools only get a modified reference list without having to know anything about the DSME implementation.

Figure 2: Simplified decoder block diagram for the combination of DMVD and DSME

# Performance Evaluation

The algorithms were implemented in HM-2.0. The performance of DSME is evaluated according to [5] for Random Access – High Efficiency for Class A and B. The BD-Rate [6] is calculated for each sequence.

## Class A

|  | BD-Rate Reduction [%] |  |  |
| --- | --- | --- | --- |
| Sequence | DSME | DMVD | Combined |
| PeopleOnStreet | -5.69 | -1.82 | -5.93 |
| Traffic | -2.98 | -1.61 | -3.24 |
| NebutaFestival | -0.23 | -0.12 | -0.32 |
| SteamLocomotiveTrain | -0.19 | -0.97 | -0.91 |
| Average | -2.27 | -1.13 | -2.60 |

## Class B

|  | BD-Rate Reduction [%] |  |  |
| --- | --- | --- | --- |
| Sequence | DSME | DMVD | Combined |
| BasketballDrive | -2.10 | -1.06 | -2.62 |
| BQTerrace | -0.60 | -0.43 | -0.78 |
| Cactus | -3.01 | -1.54 | -3.52 |
| Kimono | -1.49 | -1.49 | -2.41 |
| ParkScene | -1.59 | -1.39 | -2.36 |
| Average | -1.76 | -1.18 | -2.34 |

## Complexity

The proposed DSME architecture introduces additional computational efforts at the encoder and also at the decoder. Until now, almost no particular efforts were made to optimise the algorithms complexity. However, this contribution intends to highlight the possibilities auf motion estimation at the decoder. Furthermore, motion estimation algorithms are used for temporal up scaling in nowadays flat screen TVs with 100 and 200 Hz techniques which may be combined with DSME.

# Conclusions

The performance of DSME within the current HM software was evaluated. Rate reduction of 2.27% is achieved for Class A. For Class B, the rate is reduced by 1.76%. The gain emphasizes further studies in this field to reduce the additional complexity.

Furthermore, the combination of DMVD and DSME was tested and evaluated. Although both approaches pursue the concept of reducing the rate for the motion information, additional gain is achievable by combining DMVD and DSME. Rate reduction of 2.60% and 2.34% are achieved for Class A and B while using both, DMVD and DSME.

# References

[1]	S. Klomp, J. Ostermann, “Decoder-Side Motion Estimation with Modified Reference List”, 2nd JCT-VC Meeting, Geneva, CH, July 2010, Doc. JCTVC-B026.

[2]	ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology”, N11113, January 2010, Kyoto, JP.

[3]	H.264/AVC Reference Software, http://iphome.hhi.de/suehring/tml/download/ version 16.2, 12th November 2009.

[4]	S. Lin, M. Yang, H. Yu, “CE1: Huawei report on DMVD in HM2.0”, 5th JCT-VC Meeting, Geneva, CH, March 2011, Doc. JCTVC-E294

[5]	Frank Bossen, “Common test conditions and software reference configurations”, 4th JCT-VC Meeting, Daegu, KR, January 2011, Doc. JCTVC-D600

[6]	G. Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, 16th - 18th July, 2008, Doc.VCEG-AI11.

# Patent rights declaration(s)

Leibniz Universität Hannover may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).