| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E056 |
| --- | --- |

| Title: | Necessity of Quantization Matrices Compression in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Hironari Sakurai W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

At the last JCT-VC mtg there has been a proposal on Quantization Matrices compression by TI [3]. 16x16 and 32x32 transforms are employed  in HEVC in addition to 4x4 and 8x8, and the expected bitrate of HEVC should be half of the one with AVC, it is necessary with HEVC to compress Quantization Matrices information more than with AVC. Therefore we would like to encourage this effort.

At that time question was raised how much bits have been typically spent in actual AVC streams but nobody had the answer. This contribution will provide information on the data extracted from the AVC streams in actual products to show the necessity of Quantization Matrices compression in HEVC.

# Introduction

Quantization Matrices are tools for subjective quality control as well as for avoiding HRD overflow in the past video codec standard like MPEG-2 or AVC. Especially it is an important tool for FRExt in AVC for the coding of contents with film grain noise. HEVC should also have Quantization Matrices functionality.

16x16 and 32x32 transforms in addition to 4x4 and 8x8 can be used in HEVC, and expected bitrate of the HEVC stream is half of the one with AVC, so overhead with Quantization Matrices in the bitstream would get larger without Quantization Matrices compression algorithm whose efficiency is better than the one with AVC.

# Necessity of Quantization Matrices in HEVC

Necessity of Quantization matrices has already been discussed during the standardization of AVC FRExt [1] [2].

In addition there has been consensus that basically HEVC should have functionalities that AVC has, unless it has never or rarely used in actual AVC products like FMO/ASO. Although not implemented on the HM reference software yet, Quantization Matrices should be regarded as tools inherited from AVC.

# Necessity of Quantization Matrices Compression in HEVC

At the last JCT-VC mtg there has been a proposal on Quantization Matrices compression by TI [3]. Question was raised how much bits have been typically spent in actual AVC streams then but nobody had the answer.

In this section bit statistics extracted from AVC streams in the actual product are provided to show that Quantization Matrices compression with more efficiency will be necessary in HEVC.

In AVC streams Quantization Matrices are used for subjective quality control as well as for avoiding HRD overflow. Change in QM typically occurs at scene-changes or in difficult scenes where HRD overflow might occur.

In Table 3.1 bit statistics of AVC streams in actual products are shown. Image format is 1920/1440x1080_60i, and the bitrate is 2Mbps. In Table 3.2 bitrate specified in HEVC CfP [4] are shown as a reference. In terms of bit/pixel, the data shown in Table 3.1 are close to “Rate 2” of “B1: 1080p24” or “Rate 3” of “B2: 1080p50-60”.

Most of the data in PPS is occupied with Quantization Matrices information. The amount of bits for other information is very little.

As estimated percentage for HEVC (shown in the last column) is calculated as

assuming that

- Bits spent for QM of each size is proportional to the size of the matrix

- Expected bitrate for HEVC is half of the one for AVC

The first assumption is rather conservative, as the larger the matrix is, the larger dynamic range of each component is, therefore more bits will be required.

Table 3.1

|  | TOTAL_BITS | TOTAL_PPS | [%] | Est. [%] for HEVC |
| --- | --- | --- | --- | --- |
| Stream 00 | 2837360984 | 1939880 | 0.068369 | 2.324546 |
| Stream 01 | 1635051032 | 3821104 | 0.233699 | 7.945766 |
| Stream 02 | 1394607704 | 861512 | 0.061775 | 2.10035 |
| Stream 03 | 2018347992 | 861360 | 0.042676 | 1.450984 |
| Stream 04 | 3005247144 | 3953536 | 0.131554 | 4.472836 |
| Stream 05 | 3561879640 | 2651136 | 0.074431 | 2.530654 |
| Stream 06 | 818722592 | 2391568 | 0.29211 | 9.93174 |
| Stream 07 | 3797624992 | 2278520 | 0.059999 | 2.039966 |
| Stream 08 | 3235327760 | 4378328 | 0.135329 | 4.601186 |
| Stream 09 | 1369271392 | 873536 | 0.063796 | 2.169064 |
| Stream 10 | 2302538408 | 1677600 | 0.072859 | 2.477206 |
| Stream 11 | 412361688 | 312856 | 0.075869 | 2.579546 |

Table 3.2

| Class | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| --- | --- | --- | --- | --- | --- |
| A: 2560x1600p30 | 2.5 Mbit/s | 3.5 Mbit/s | 5 Mbit/s | 8 Mbit/s | 14 Mbit/s |
| B1: 1080p24 | 1 Mbit/s | 1.6 Mbit/s | 2.5 Mbit/s | 4 Mbit/s | 6 Mbit/s |
| B2: 1080p50-60 | 2 Mbit/s | 3 Mbit/s | 4.5 Mbit/s | 7 Mbit/s | 10 Mbit/s |
| C: WVGAp30-60 | 384 kbit/s | 512 kbit/s | 768 kbit/s | 1.2 Mbit/s | 2 Mbit/s |
| D: WQVGAp30-60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |
| E: 720p60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |

As can be seen, “Stream 06” is the worst case in Table 3.1: percentage of PPS over total bits is 0.29%. This looks small, but estimated percentage for HEVC would become around 10% with this stream. This also means that if the bits for Quantization Matrices become half, as much as 5% gain can be obtained.

Therefore we would like to encourage efforts on Quantization Matrices compression.

In addition it may be difficult to compare one method with another if each proposal employs different matrices for simulation. So we propose that “Common Test Matrices” be defined (probably by the Quantization AHG), to evaluate each proposal for Quantization Matrices compression.

# Conclusion

Quantization Matrices are a useful tool for subjective quality control and avoiding HRD overflow. 16x16 and 32x32 transforms are employed in HEVC in addition to 4x4 and 8x8, and the expected bitrate of HEVC should be half of the one with AVC, so occupancy of Quantization Matrices in HEVC would become much larger with HEVC than with AVC. Therefore it is necessary with HEVC to compress Quantization Matrices information more.

Estimated from AVC bitstreams in the actual product, bit occupancy of Quantization Matrices in HEVC would be as much as 10% in the worst case. It means that if the compression ratio for Quantization Matrices becomes twice, as much as 5% gain can be obtained.

To make a fair comparison between proposals, “Common Test Matrices” should be defined, probably by the Quantization AHG.

# References

[1] “H.264/AVC for Next Generation Optical Disc:  A Proposal on FRExt Profiles”, JVT-K025, Mar 2004

[2] “Proposal of quantization weighting for H.264/MPEG-4 AVC Professional Profiles”, JVT-K029, Mar 2004

[3] “Compact representation of quantization matrices for HEVC”, JCTVC-D024, Jan 2011

- [4] “Joint Call for Proposals on Video Compression Technology”,ISO/IECJTC1/SC29/WG11, N11113, Jan 2010

- [5] “Quantization matrix for HEVC”, JCTVC-E073, Mar 2011