| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E057-r3 WG11 Number: m19565 |
| --- | --- |

| Title: | Pulse code modulation mode for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents HM2.0-based simulation results of Pulse Code Modulation (PCM) mode coding which conditionally transmits single-bit syntax in 2Nx2N intra PU header in order to indicate the use of PCM mode coding. This contribution also proposes a new SPS syntax that specifies the intra CU sizes which convey I_PCM_flags; the new SPS syntax allows encoders to choose the sizes of PCM mode CUs by their preference, that is, it allows encoders to omit I_PCM flag transmission at non-PCM CUs and save some bits. Simulation results for common test sequences show average BD-rate losses of 0.00% (Y), 0.00% (U), 0.01% (V) for all intra high efficiency setting, 0.08% (Y), 0.10 % (U), 0.09 % (V) for all intra low complexity setting, -0.01% (Y), 0.07% (U), -0.05% (V) for random access high efficiency setting, 0.04% (Y), 0.03% (U), 0.00% (V) for random access low complexity setting, -0.04% (Y), 0.02% (U), 0.08% (V) for low delay high efficiency setting, and 0.02% (Y), 0.17% (U), 0.08% (V) for low delay low complexity setting, when PCM coding CU sizes are 64x64 and 32x32. Additional simulation results for a synthesized sequence demonstrate that the use of PCM mode coding avoids yielding the number of bits which is prohibitively greater than that of input video data and reduces decode time significantly. It is proposed that the PCM mode coding is adopted in HM ver.3.

- Introduction

The PCM mode coding in the H.264/MPEG-4 AVC standard transmits sample values of its associated MB without prediction, transform coding, and entropy coding, and thus it allows encoders to adjust the number of bits of MB to a predetermined value or less without complicated computation. Since current HM does not have such a means, it would be desirable to adopt PCM mode coding in HM 3.0.

The purpose of this contribution is to present HM2.0-based simulation results of PCM mode coding methods for HM 3.0 adoption. Section 2 reviews the PCM mode coding methods in the H.264/MPEG-4 standard and JCTVC-D044. Section 3 and 4 present the modified versions of the JCTVC-D044 and their simulation results, respectively

# Prior PCM mode encoding methods

2.1. PCM mode encoding in the H.264/MPEG-4 AVC standard

The use of PCM mode coding is indicated by a MB mode syntax, and when PCM mode coding is used, the associated MB’s sample values are transmitted without prediction, transform coding, and entropy coding. Note that when entropy coding is CABAC, BAC is terminated and initialized prior to transmitting sample values.

2.2. PCM mode encoding in JCTVC-D044

The use of PCM mode coding is indicated by a new single-bit syntax, I_PCM_flag, which is conditionally signaled in the PU header layer when the associated CU is Intra. If I_PCM_flag presents and its value is one, the associated Intra CU is encoded by PCM mode coding; otherwise the associated Intra CU is encoded by intra prediction.

Entropy coding of I_PCM_flag for CABAC is the same as that of end_of_slice_flag, and therefore BAC is terminated and initialized prior to transmitting sample values when I_PCM_flag = 1. On the other hand, no entropy coding is applied to I_PCM_flag for LCEC, that is, fixed-length one bit is transmitted.

PCM sample values are transmitted in a way similar to that of the H.264/MPEG-4 AVC standard except that those values are quantized to the original bit-depth prior to the transmission when IBDI is used. The LSBs of samples extended by IBDI do not have any information about their original sample values and increase the amount of PCM data significantly. Therefore the quantization is applied when IBDI is used.

# Proposed PCM mode encoding methods

Proposed PCM encoding methods are on the top of a modified version of JCTVC-D044 (hereafter referred to as the PCM operation mode 1). The differences between the JCTVC-D044 and the PCM operation mode 1 are as follows:

- Since current HM supports 2Nx2N and NxN partitions in SCUs, the modified version transmits I_PCM_flags when intra partition size is 2Nx2N in order to save some bits at SCUs.

- Since current HM does not have the concept of IBDI, the bit-depth of PCM sample values is that of internal sample values. (Note that although attached software transmits PCM sample values by the original bit-depth, it can transmit PCM sample values by internal bit-depth by enabling “FULL_NBIT” macro.)

The PCM operation mode 1 signals I_PCM_flag when CU is 2Nx2N intra, that is, I_PCM can be used in any intra CUs. However, signaling I_PCM_flags in all intra CUs affects coding performance of HM2.0, especially when LCEC is used for entropy coding, due to the fact that PCM mode coding is a non-compression technique and is not used in the common test sequences with the common coding conditions.

Therefore it would be useful to have a new SPS syntax that specifies the intra CU sizes which convey the I_PCM_flags in order to allow encoders to omit I_PCM_flag transmission and save some bits at non-PCM CUs. PCM operation mode 2 and 3 are PCM encoding methods using such a new SPS syntax.

The PCM operation mode 2 transmits I_PCM flags in large intra CUs defined by a new SPS Syntax. Specifically, it signals the minimum PCM CU size, log2_min_I_PCM_coding_block_size_minus3, in SPS and transmits I_PCM_flags in 2Nx2N intra PU headers when their CU size is larger than or equal to (1 << (log2_min_I_PCM_coding_block_size_minus3 + 3)).

The PCM operation mode 3 is the opposite approach of the PCM operation mode 2. The PCM operation mode 3 signals the maximum PCM CU size, log2_max_I_PCM_coding_block_size_minus3, in SPS, and it signals I_PCM flags in 2Nx2N intra PU headers when their CU size is smaller than or equal to (1 << (log2_max_I_PCM_coding_block_size_minus3 + 3)).

The above PCM operation modes are summarized as follows:

- PCM operation mode 1: Signal I_PCM flag when CU is 2Nx2N intra.

- PCM operation mode 2: Signal I_PCM flag when CU is 2Nx2N intra and its size is larger than or equal to the CU size indicated by SPS.

- PCM operation mode 3: Signal I_PCM flag when CU is 2Nx2N intra and its size is smaller than or equal to the CU size indicated by SPS.

This contribution recommends the PCM operation mode 1 or 2 for HM3.0 adoption by considering pros and cons of the proposed PCM operation modes listed below; the PCM operation mode 3 seems to be undesirable for HM3.0 adoption due to its disadvantage in decreasing coding performance and throughput of large CUs.

In addition, as deduced from simulation results presented in section 4 (BD-rate loss of the PCM operation mode 2 is greatly smaller than other PCM operation modes), it seems that HM's RD-optimized CU size decision method selects small CUs more frequently than large CUs and that large CU serves a secondary purpose as some kind of bit regulating unit that can be used when necessary. The PCM operation mode 2 seems to be more appropriate for HM3.0 adoption than the PCM operation mode 1 because it allows encoders to omit I_PCM flag and save some bits at small CUs.

[PCM operation mode 2 against 1]

Pros:

- Bigger minCUsize, less I_PCM flag signaling overhead on average, where minCUsize is the minimum I_PCM CU size indicated by SPS.

Cons:

- Excessive bits can be used for coding the sub-CUs smaller than minCUsize

For example, suppose that there is a CU comprising of four sub-CUs: one sub-CU is a non-textured image area, and the other three sub-CUs are complicated-textured image areas. Further suppose that the coded non-textured image area needs 1-bit and that the each of coded complicated-textured image areas needs more bits than its row data bits. If the sub-CUs are smaller than minCUsize, an encoder needs to code the CU by I_PCM coding. It results in using excessive bits for the non-textured image area.

[PCM operation mode 3 against 1]

Pros:

- Smaller maxCUsize, less I_PCM flag signaling overhead on average, where maxCUsize is the maximum I_PCM CU size indicated by SPS.

Cons:

- Coding performance and throughput are decreased for CUs larger than maxCUsize

For example, suppose that there is a CU of a complicated-textured image area and the coded CU needs more bits than its row data bits. Further suppose that even if the CU split into small sub-CUs, each of coded sub-CUs needs more bits than its row data bits.  If the CU is larger than maxCUsize, an encoder needs to split the CU into small sub-CUs so that each of the small sub-CUs can be coded by I_PCM. Since BAC termination and PCM-alignment associated with I_PCM coding produce overhead bits significantly, the total size of the I_PCM-coded small sub-CUs is larger than that of I_PCM-coded CU. Furthermore, as the number of processing’s associated with I_PCM coding increase, it decreases the coding throughput of the CU. Especially if an encoder does not keep backup CABAC context parameters in local memory, the coding throughput is prohibitively decreased.

[PCM operation mode 3 against 2]

Pros:

- Excessive bits is not used for coding the sub-CUs smaller than minCUsize

Cons:

- Coding performance and throughput are decreased for CUs larger than maxCUsize

- Require more I_PCM flag signaling overhead

- Simulation

3.1. Experimental setup

PCM mode coding methods described in 2.2 are implemented into the HM2.0. (“JCTVC-E057-NEC-PCM-Software1.zip” supports the PCM operation mode1 and 2. “JCTVC-E057-NEC-PCM-Software2.zip” supports the PCM operation mode1 and 3.) Their 64-bit encoder and decoder executables were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platform listed below. Note that as inferred from the spec. of the computing platform, CPU automated over-clocking happened for some results. Individual encode and decode times can be significantly varied.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Simulation for common test sequences were conducted according to JCTVC-D600 and the results are described in 3.2. Since PCM mode is not selected in the common test sequences with the common coding conditions, additional simulation results for a synthesized sequence using lower QP values are described in 3.3.

In PCM operation mode 2, the minimum PCM CU size is set to 32x32. In PCM operation mode 3, the maximum PCM CU size is set to 16x16.

3.2. Results for common test sequences

Attached three Excel sheets, JCTVC-E057-NEC-HM-2.0-Anchor-vs-AllCU-PCM.xls, JCTVC-E057-NEC-HM-2.0-Anchor-vs-LargeCU-PCM.xls, and JCTVC-E057-NEC-HM-2.0-Anchor-vs-SmallCU-PCM.xls show the detailed results of proposed PCM operation mode 1, 2, and 3, respectively. Table 1 shows the summary results.

The followings are observed from the Table 1:

- In HE results, all PCM operation modes do not increase BD-rates significantly in all coding cases, as expected from the fact that I_PCM_flag’s MPS probability is about 0.99;

- In LC results, PCM operation mode 1 and 3 moderately increase BD-rates in All intra case, and they slightly increases BD-rates in other cases.

- In LC results, PCM operation mode 2 does not increase BD-rates significantly in all coding cases.

In terms of BD-rates, PCM operation mode 2 with the minimum PCM CU size 32x32 performs better than other PCM operation modes. In terms of BD-rates, PCM operation mode 2 with the minimum PCM CU size 32x32 performs better than other PCM operation modes. As it may deduced from Table1, the minimum PCM CU size should be greater than or equal to 32x32 in order to achieve negligible BD-rate losses.

Table 1: Summary of PCM results.

3.3. Additional results for synthesized noisy sequence

PCM mode is not selected in the common test sequences with the common coding conditions. Therefore this subsection provides additional results for a synthesized sequence depicted in Figure 1. The synthesized sequence is Gaussian noise, which is difficult to compress by using prediction, transform coding, and entropy encoding. Some videos can partially contain textures whose nature is similar to Gaussian noise. (“JCTVC-E057-NEC-Sandstorms-Software.zip” is a software that generates the sequence. )

Figure 1: Synthesized sequence, Sandstorms, (CIF 30Hz 30frames)

Table 2 and Table 3 show intra coding results for Sandstorms by using QP values 0, 2, 7, 12, 17, 22, 27, 32, and 37. “Comp. Ratio” is the ratio between the data sizes of input video and its compressed bitsream. “Relative dec.” is the ratio expressed in percentage between the decode times of HM2.0 and Proposal (PCM operation mode 1). From the tables, it is observed that Proposal avoids yielding the number of bits which is significantly greater than that of input video for values ranging from 0 to 12. In addition, the tables show that Proposal saves its decode time significantly for QP values ranging from 0 to 17 by restricting the amount of entropy-coded bits.

Table 2: High efficiency intra results. Note that “99.99” in PSNR means that decode video is loss less.

Table 3: Low complexity intra results. Note that “99.99” in PSNR means that decode video is loss less.

Since current HM does not have the concept of IBDI, the bit-depth of PCM sample values should be that of internal sample values. Therefore Table 4 shows the simulation results in which Proposal transmits PCM sample values represented by internal sample bit-depth. The results for QP values 0, 2, and 7 demonstrate that PCM mode coding avoids yielding the number of bits which is prohibitively greater than that of internal video data. Note that since current PCM mode decision is conducted without using Rate-Distortion cost, the results for QP values 12 and 17 are worse than HM2.0 in terms of the balance of PSNR and Bit-rates. As it can be seen from Table 2 and Table 4, the bit-depth of PCM sample values should be that of the original sample values. The issue of PCM sample bit-depth for the use of BDI is discussed in another contribution JCTVC-E192 [3].

Table 4: High efficiency all intra results. Note that “99.99” PSNR means that decode video is loss less.

- Conclusions

This contribution has presented HM2.0-based simulation results of Pulse Code Modulation (PCM) mode coding methods. It is recommended that the PCM operation mode 2 with the minimum PCM CU size 32x32 or 64x64 is adopted in HM ver.3.

- Reference

- Keiichi Chono, Kenta Senzaki, Hirofumi Aoki, Junji Tajime, and Yuzo Senda, "Pulse code modulation mode for HEVC", JCTVC-D044, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Keiichi Chono, Hirofumi Aoki, Viktor Wahadaniah, and ChongSoon Lim, “Proposal of enhanced PCM coding in HEVC", JCTVC-E192, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed syntax and semantics w/o the new SPS syntax (PCM operation mode 1)

(The modified portion of JCT-VC D503 is highlighted with red colour.)

### A.1. Change in CU syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_mode | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if(( PredMode == MODE_INTRA ) && ( PartMode == PART_2Nx2N ) ){ |  |
| pcm_flag | u(1) |
| } |  |
| if( PartMode == I_PCM ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( ( 1 << ( log2CUSize << 1 ) ) >> 1 ); i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |

pcm_flag specifies if associated intra coding unit is coded by I_PCM: If the pcm_flag value is equal to 1, associated intra coding unit is coded by I_PCM and the variable PredMode is set to I_PCM; otherwise the pcm_flag value is equal to 0 or the pcm_flag syntax is not present, associated intra coding unit is coded by MODE_INTRA.

pcm_alignment_zero_bit is a bit equal to 0.

pcm_sample_luma[ i ] is a sample value. The pcm_sample_luma[ i ] values represent luma sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthY.

pcm_sample_chroma[ i ] is a sample value. The first currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cb sample values in the raster scan within the coding unit and the remaining currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cr sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthC.

A.2. Change in “8.3	Decoding process for coding units coded in intra prediction mode”

The following text

"Depending on IntraSplitFlag, the following applies:"

is replaced by

" If PredMode is equal to I_PCM, the sample construction process for I_PCM CUs as specified in subclause 8.5.6 is invoked with the luma location ( xB, yB ) and the size of the current coding unit log2CUSize. Otherwise, depending on IntraSplitFlag, the following applies:

"

A.3. Addition of new subsection “8.5.6		Array reconstruction process for I_PCM CUs”

This process is invoked when PredMode is equal to I_PCM.

The variable currCodingUnitSize is set to ( 1 <<  log2CUSize).

The reconstructed luma samples R'L prior to the deblocking process are generated as specified by

for( i = 0; i < currCodingUnitSize*currCodingUnitSize; i++ )

R'L[ xB + ( i % currCodingUnitSize ), yB + ( i / currCodingUnitSize ) ] = pcm_sample_luma[ i ]

The variable currCodingUnitSizeC is set to (( 1 << log2CUSize) >> 1).

The reconstructed chroma samples R'Cb and R'Cr prior to the deblocking process are generated

as specified by

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cb[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i ]

}

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cr[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i + currCodingUnitSizeC * currCodingUnitSizeC]

}

### A.4. Change in “8.6.1.4.2	Filtering process for a sample"

The following text

" A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input."

is replaced by

" A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input, where qPL is zero for I_PCM blocks."

Appendix B: Proposed syntax and semantics with the new SPS syntax (PCM operation mode 2)

#### B.1. Change in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| adaptive_loop_filter_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

log2_min_pcm_coding_block_size_minus3 specifies the minimum size of I_PCM coding blocks.

The variable Log2MinIpcmCUSize is set equal to log2_min_I_PCM_coding_block_size_minus3 + 3.

B.2. Change in CU syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_mode | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if(( PredMode == MODE_INTRA ) && ( PartMode == PART_2Nx2N ) && ( log2CUSize >= Log2MinIpcmCUSize ) ){ |  |
| pcm_flag | u(1) |
| } |  |
| if( PartMode == I_PCM ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( ( 1 << ( log2CUSize << 1 ) ) >> 1 ); i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |

pcm_flag specifies if associated intra coding unit is coded by I_PCM: If the pcm_flag value is equal to 1, associated intra coding unit is coded by I_PCM and the variable PredMode is set to I_PCM; otherwise the pcm_flag value is equal to 0 or the pcm_flag syntax is not present, associated intra coding unit is coded by MODE_INTRA.

pcm_alignment_zero_bit is a bit equal to 0.

pcm_sample_luma[ i ] is a sample value. The pcm_sample_luma[ i ] values represent luma sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthY.

pcm_sample_chroma[ i ] is a sample value. The first currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cb sample values in the raster scan within the coding unit and the remaining currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cr sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthC.

i_pcm_flag specifies if associated intra coding unit is coded by I_PCM: If the i_pcm_flag value is equal to 1, associated intra coding unit is coded by I_PCM and the variable PredMode is set to I_PCM; otherwise the i_pcm_flag value is equal to 0 or the i_pcm_flag syntax is not present, associated intra coding unit is coded by MODE_INTRA.

B.3. Change in “8.3	Decoding process for coding units coded in intra prediction mode”

The following text

"Depending on IntraSplitFlag, the following applies:"

is replaced by

" If PredMode is equal to I_PCM, the sample construction process for I_PCM CUs as specified in subclause 8.5.6 is invoked with the luma location ( xB, yB ) and the size of the current coding unit log2CUSize. Otherwise, depending on IntraSplitFlag, the following applies:"

B.4. Addition of new subsection “8.5.6		Array reconstruction process for I_PCM CUs”

The following texts are added after 8.5.5.

8.5.6		Array reconstruction process for I_PCM CUs The constructed luma samples prior to

This process is invoked when PredMode is equal to I_PCM.

The variable currCodingUnitSize is set to ( 1 <<  log2CUSize).

The reconstructed luma samples R'L prior to the deblocking process are generated as specified by

for( i = 0; i < currCodingUnitSize*currCodingUnitSize; i++ )

R'L[ xB + ( i % currCodingUnitSize ), yB + ( i / currCodingUnitSize ) ] = pcm_sample_luma[ i ]

The variable currCodingUnitSizeC is set to (( 1 << log2CUSize) >> 1).

The reconstructed chroma samples R'Cb and R'Cr prior to the deblocking process are generated

as specified by

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cb[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i ]

}

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cr[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i + currCodingUnitSizeC * currCodingUnitSizeC]

}

### B.5. Change in “8.6.1.4.2	Filtering process for a sample"

The following text

" A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input."

is replaced by

" A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input, where qPL is zero for I_PCM blocks."

Appendix C: Proposed syntax and semantics with the new SPS syntax (PCM operation mode 3)

#### C.1. Change in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_max_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| adaptive_loop_filter_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

log2_max_pcm_coding_block_size_minus3 specifies the maximum size of I_PCM coding blocks.

The variable Log2MaxIpcmCUSize is set equal to log2_max_pcm_coding_block_size_minus3 + 3.

C.2. Change in CU syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_mode | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if(( PredMode == MODE_INTRA ) && ( PartMode == PART_2Nx2N ) && ( log2CUSize <= Log2MaxIpcmCUSize ) ){ |  |
| pcm_flag | u(1) |
| } |  |
| if( PartMode == I_PCM ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( ( 1 << ( log2CUSize << 1 ) ) >> 1 ); i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |

pcm_flag specifies if associated intra coding unit is coded by I_PCM: If the pcm_flag value is equal to 1, associated intra coding unit is coded by I_PCM and the variable PredMode is set to I_PCM; otherwise the pcm_flag value is equal to 0 or the pcm_flag syntax is not present, associated intra coding unit is coded by MODE_INTRA.

pcm_alignment_zero_bit is a bit equal to 0.

pcm_sample_luma[ i ] is a sample value. The pcm_sample_luma[ i ] values represent luma sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthY.

pcm_sample_chroma[ i ] is a sample value. The first currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cb sample values in the raster scan within the coding unit and the remaining currCodingUnitSize*currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent Cr sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthC.

C.3. Change in “8.3	Decoding process for coding units coded in intra prediction mode”

The following text

"Depending on intra_split_flag, the following applies:"

is replaced by

" If PredMode is equal to I_PCM, the sample construction process for I_PCM CUs as specified in subclause 8.5.6 is invoked with the luma location ( xB, yB ) and the size of the current coding unit log2CUSize. Otherwise, depending on intra_split_flag, the following applies:

"

C.4. Addition of new subsection “8.5.6		Array reconstruction process for I_PCM CUs”

The following texts are added after 8.5.5.

8.5.6		Array reconstruction process for I_PCM CUs The constructed luma samples prior to

This process is invoked when PredMode is equal to I_PCM.

The variable currCodingUnitSize is set to ( 1 <<  log2CUSize).

The reconstructed luma samples R'L prior to the deblocking process are generated as specified by

for( i = 0; i < currCodingUnitSize*currCodingUnitSize; i++ )

R'L[ xB + ( i % currCodingUnitSize ), yB + ( i / currCodingUnitSize ) ] = pcm_sample_luma[ i ] << bit_depth_luma_increment

The variable currCodingUnitSizeC is set to (( 1 << log2CUSize) >> 1).

The reconstructed chroma samples R'Cb and R'Cr prior to the deblocking process are generated

as specified by

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cb[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i ] << bit_depth_chroma_increment

}

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cr[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i + currCodingUnitSizeC * currCodingUnitSizeC] << bit_depth_chroma_increment

}

Note: Since current HM design does not have the concept of IBDI, the above descriptions should be replaced by the following ones:

8.5.6		Array reconstruction process for I_PCM CUs The constructed luma samples prior to

This process is invoked when PredMode is equal to I_PCM.

The variable currCodingUnitSize is set to ( 1 <<  log2CUSize).

The reconstructed luma samples R'L prior to the deblocking process are generated as specified by

for( i = 0; i < currCodingUnitSize*currCodingUnitSize; i++ )

R'L[ xB + ( i % currCodingUnitSize ), yB + ( i / currCodingUnitSize ) ] = pcm_sample_luma[ i ]

The variable currCodingUnitSizeC is set to (( 1 << log2CUSize) >> 1).

The reconstructed chroma samples RCb and R'Cr prior to the deblocking process are generated

as specified by

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

R'Cb[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i ]

}

for( i = 0; i < currCodingUnitSizeC * currCodingUnitSizeC; i++ ) {

S'Cr[ (xB >> 1) + ( i % currCodingUnitSizeC ),

(yB >> 1) + ( i / currCodingUnitSizeC ) ]

= pcm_sample_chroma[ i + currCodingUnitSizeC * currCodingUnitSizeC]

}

### C.5. Change in “8.6.1.4.2	Filtering process for a sample"

The following text

"2.	If bS is not equal to 2, the variables β and tC are specified as Table 8 8 with luma quantization parameter qPL as input otherwise the variables β and tC are specified as Table 8 8 with luma quantization parameter ( qPL + 4 ) as input."

is replaced by

"2.	If bS is not equal to 2, the variables β and tC are specified as Table 8 8 with luma quantization parameter qPL as input otherwise the variables β and tC are specified as Table 8 8 with luma quantization parameter ( qPL + 4 ) as input, where qPL is zero for I_PCM blocks."

Note: If the qPL value means the average of luma quantization parameter qPLP0 of po and luma quantization parameter qPLQ0 of qo, the text should be replaced by the following one:

“2.	The variable qPL is set to (qPLp0 + qPLq0+1)>>1, where qPLP0 and qPLQ0 are luma quantization parameters of po and qo. If bS is not equal to 2, the variables β and tC are specified as Table 8 8 with luma quantization parameter qPL as input otherwise the variables β and tC are specified as Table 8 8 with luma quantization parameter ( qPL + 4 ) as input. Note that luma quantization parameters of I_PCM samples are zero. "