| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E059 |
| --- | --- |

| Title: | Modifications of temporal mv memory compression and temporal mv predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Park Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-10-3922-5896 seungwook.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes modifications of temporal motion vector memory compression scheme and position change of temporal motion vector predictor. Regarding temporal mv memory compression scheme, three modifications are proposed: 1) correcting inconsistency of mv and reference index, 2) changing representative mv position and 3) replacing zero mv of intra block by neighbor inter block. And 4) changing center position of temporal mv predictor is proposed. For a combination of all modifications, average DB-rate savings of 0.7% for RAHE, 0.7% for RALC, 0.8% for LDHE and 0.8% for LDLC are reported.

# Introduction

In the last meeting, temporal mv memory compression technique [1] was proposed and the results showed that average 0.4% ~ 0.7% coding loss was yielded with 16x memory buffer savings. In this scheme, a motion vector at an upper left corner block is used as a representative one. It means a single motion vector is assigned to all blocks within a predefined region. However this method may cause the inconsistency between motion vector and reference index, because there is no change in reference index memory when motion vector is compressed. This inconsistency results in wrong scaling of temporal mv predictor and this makes a coding loss. And, besides upper left corner position is not efficient as much as center position when it is used as a temporal mv predictor [2].

Accordingly, we propose some modifications on the previous mv compression scheme in order to compensate the coding loss when mv memory compression is applied, and better center position of temporal predictor is proposed than the conventional one.

# Modifications of temporal motion vector compression scheme

## Correcting inconsistency of mv and reference index

Current temporal mv compression method may cause the inconsistency between motion vector and reference index, because only mv memory is compressed without any change in reference index buffer (refer to figure 1). This inconsistency may induce wrong mv scaling and accuracy degradation of temporal mv predictor. So we propose reference index buffer compression method following the same manner of temporal mv memory compression (refer to figure 2). This method provides coding gain as well as memory buffer reduction for reference index storage.

Figure 1. Temporal mv memory compression (e.g. motion_vector_buffer_comp_ratio = 4)

Figure 2. Temporal mv memory compression with ref idx memory compression

## Changing representative mv position

As reported in JCTVC-D164, centered temporal mv predictor is better than the upper left corner in coding efficiency perspective. Reflecting this concept to mv memory compression scheme, we propose the position change of representative mv from the origin of blocks to the center of blocks. In this example, its position is changed from 0th block to 12th block in z-scan order (refer to Figure 3). As another option, we can use right bottom block (15th block in scan-order) as a representative block.

- Option 1

- Option 2

Figure 3. Temporal mv memory compression with new representative mv

## Replacing zero mv of intra block by neighbor inter block

In the current mv compression scheme, all blocks will have zero mv if a representative block is coded as Intra, even if there are neighboring Inter blocks having real motion vector. So we propose to use neighbor Inter block instead of using zero mv of Intra block. Around the center position, the right bottom block, left top block, right top block and left bottom block are checked in order until we find an Inter coded block. In this example, we use the left top block as a representative mv, because the right bottom block is coded as Intra. If we use option 2 in section 2.2, searching order will be 15th block  11th block  7th block  3rd block.

Figure 4. Replacing zero mv of intra block by neighbor inter block

# Modifications of temporal motion vector predictor

## Changing position of centered temporal predictor

Centered temporal predictor was already adopted in the last meeting. In that scheme, upper left position of the center in the block was used as a temporal mv predictor instead of the origin of the block in AMVP process. According to our test, right bottom block is better than the conventional one in terms of coding efficiency. So we propose to change the position of centered temporal mv predictor from the upper left block to the right bottom block around the center position in collocated PU.

Figure 5. Changing position of centered temporal predictor

# Experimental results

We implemented the proposed methods into HM 2.0 and followed the common test conditions. Our proposal was cross-checked by TI and MediaTek. Test results of each modification and its combination are shown from Table 1 to Table 6.

Table 1. Results of 2.1 (correcting inconsistency of mv and ref idx)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.2 | -0.1 | 0.0 | 0.1 |
| Class C | -0.1 | 0.1 | 0.0 | -0.2 | -0.1 | -0.2 |
| Class D | -0.2 | -0.2 | -0.3 | -0.1 | -0.4 | -0.1 |
| Class E | -0.1 | -0.5 | 0.2 | 0.1 | -0.3 | -0.2 |
| All | -0.1 | -0.1 | 0.0 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2. Results of 2.2 (changing representative mv position: option 1)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.3 | -0.4 | -0.4 | -0.3 |
| Class B | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | -0.3 | -0.3 | -0.4 | -0.3 | -0.2 | -0.3 |
| Class D | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.0 | -0.2 | -0.2 | -0.2 | 0.3 |
| Class C | -0.5 | -0.3 | -0.4 | -0.5 | -0.6 | -0.8 |
| Class D | -0.6 | -0.7 | -0.5 | -0.5 | -0.9 | -0.9 |
| Class E | -0.4 | -0.2 | -0.3 | -0.5 | -0.6 | -0.2 |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.6 | -0.4 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3. Results of 2.2+2.3 (replacing zero mv of intra block by neighbor inter block)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.7 | -0.4 | -0.3 | -0.3 |
| Class B | -0.2 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.4 | -0.3 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.6 | -0.2 | -0.3 | -0.2 |
| Class C | -0.6 | -0.4 | -0.6 | -0.6 | -0.7 | -0.7 |
| Class D | -0.6 | -0.4 | -0.8 | -0.6 | -0.8 | -0.5 |
| Class E | -0.7 | -1.1 | -0.2 | -0.4 | -0.4 | -0.5 |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4. Results of 2.1+2.2+2.3

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.6 | -0.5 | -0.4 | -0.4 |
| Class B | -0.3 | -0.3 | -0.2 | -0.3 | -0.1 | -0.2 |
| Class C | -0.6 | -0.5 | -0.5 | -0.6 | -0.5 | -0.5 |
| Class D | -0.8 | -0.8 | -0.7 | -0.9 | -0.9 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.5 | -0.4 | -0.5 | -0.3 |
| Class C | -0.7 | -0.6 | -0.5 | -0.9 | -1.0 | -1.0 |
| Class D | -0.9 | -0.8 | -1.0 | -0.8 | -1.1 | -0.7 |
| Class E | -0.6 | -0.8 | -0.6 | -0.4 | -0.9 | -0.5 |
| All | -0.6 | -0.6 | -0.6 | -0.6 | -0.8 | -0.6 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5. Results of 3.1 (changing position of centered temporal predictor)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | -0.4 | -0.2 | -0.3 | -0.2 |
| Class B | -0.2 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class C | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.1 | -0.2 | -0.1 | -0.1 | -0.2 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.1 | -0.2 | -0.1 | -0.2 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | -0.3 |
| Class D | -0.1 | -0.2 | -0.3 | 0.0 | -0.2 | 0.0 |
| Class E | -0.2 | -0.2 | 0.0 | -0.1 | -0.6 | -0.3 |
| All | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 6. Results of 2.1+2.2+2.3+3.1

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.5 | -0.7 | -0.7 | -0.7 | -0.7 |
| Class B | -0.4 | -0.4 | -0.3 | -0.4 | -0.3 | -0.3 |
| Class C | -0.7 | -0.6 | -0.7 | -0.7 | -0.6 | -0.7 |
| Class D | -1.0 | -0.9 | -1.0 | -1.0 | -0.9 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -0.6 | -0.7 | -0.7 | -0.6 | -0.6 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.4 | -0.6 | -0.6 | -0.6 | -0.4 |
| Class C | -0.8 | -0.7 | -0.9 | -1.0 | -1.1 | -1.2 |
| Class D | -0.9 | -0.3 | -0.7 | -0.9 | -0.9 | -0.6 |
| Class E | -0.9 | -1.0 | -0.9 | -0.7 | -0.5 | -0.8 |
| All | -0.8 | -0.6 | -0.7 | -0.8 | -0.8 | -0.7 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Figure 6. Overall performance of each modification and its combination

Overall coding gain is depicted in Figure 6 and we can check that each method gives additional coding gain on top of every combination.

# Conclusion

In this proposal, we have proposed three modifications for temporal mv compression method and one method for centered temporal mv predictor. All schemes give a meaningful coding gain without noticeable impact on complexity. Besides reference index memory buffer can be compressed resolving the inconsistency between mv and ref idx. So we propose inclusion of all modifications in the HM design.

# References

- Y. Su and A. Segall, “CE9: Reduced resolution storage of motion vector data”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D072, 4th Meeting: Daegu, KR, 20-28 January, 2011.

- J. Jung, G. Clare, “Temporal MV predictor modification for MV-Comp, Skip, Direct and Merge schemes”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D164, 4th Meeting: Daegu, KR, 20-28 January, 2011.

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).