| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E060 |
| --- | --- |

| Title: | CE8 Subtest 5: Luma ALF with reduced vertical filter size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | madhukar@ti.com sze@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution analyzes implementation complexity of Luma Adaptive loop filter (ALF) at decoder. Implementation complexity is determined by not just computations but also memory bandwidth/memory size (area). This contribution presents Luma ALF filter sets that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. This contribution is a continuation of work first presented in JCTVC-D039 and studies Luma ALF filters with maximum vertical size of 5. Among the filter sets studied in this contribution, filter set Nx5-Set1 is asserted to provide the best trade-off in terms of complexity reduction and coding performance. Nx5-Set1 filter is reported to reduce memory bandwidth/memory size requirements by 33% and worst case computations by 15%. Nx5-Set1 filter is reported to provide average Y-BD-Rate loss of: 0.1% HE_Intra, 0.2% HE_RA, 0.3% HE_LD.

# Introduction

HM 2.0 ALF uses three diamond shaped filters of sizes 9x7, 7x7, 5x5 as shown in Figure 1. A maximum of up to 16 filters can be signaled per slice. The worst case number of multiplications required is for 9x7 filter: 20 multiplications. The number of pixels that need to be read to carry out one filtering operation is also 39.

9x7-Diamond

7x7-Diamond

5x5-Diamond

Nx7 filter set

Figure 1: HM 2.0 Luma ALF filter set.

ALF decode filtering can be carried out in either frame-based or LCU-based fashion. In frame-based ALF filtering, the deblocking filter output is first stored in a frame buffer. The deblocked filter output is then read to carry out ALF filtering. Figure 2 shows an example frame-based ALF filtering with size-5 filter. The red-lines in Figure 2 are pixel lines. For a size-9 filter, the deblock frame buffer needs to be read 39 times which is not feasible since the memory bandwidth requirements go up to 39 frame buffer reads. Clearly a caching mechanism is needed to reduce memory bandwidth requirements.

Figure 2: Frame-based ALF decode.

1

2

3

4

5

6

7

Filtering at (4, x)

Filtering at (5, x+1)

1

2

3

4

5

6

7

Figure 3: Frame-based ALF decode filtering with line buffer cache.

Figure 3 shows an example of frame-based ALF decode filtering with line buffer cache. The figure on left shows filtering of line 4. The figure on right shows filtering of line 5. From the Figure 3, one can see that Lines 3, 4, 5, 6 get used in filtering of both lines 4 and 5. Data on these lines can be cached in line buffers to avoid reading them again from deblock frame output buffer. With the line buffer caching mechanism shown in Figure 3, the deblock frame output buffer needs to be read only once (also written once). However, this is still expensive. Memory bandwidth requirements can be further reduced by using LCU-based filtering instead as shown in Figure 4.

Input

Output

Input

Lines that need to be cached

Output

Figure 4: LCU-based ALF decode filtering with line buffer cache.

In LCU-based filtering, ALF filtering is carried out on an LCU basis as and when the deblock filter output gets generated. The left part of Figure 4 shows ALF filtering of LCU(0,0) and the right part shows ALF filtering of LCU(1,0). The red lines show the deblock filter pixel lines (which is input to ALF filters) and the blue lines are the ALF filtered output. Since the ALF filter is a non-causal filter and uses right and bottom LCU data (which are not yet available), not all pixels in a LCU can be ALF filtered. The ALF filtered pixels are shown in solid blue line and the LCU pixels that cannot be ALF filtered when the LCU is generated are shown in dotted blue lines.

From Figure 4 it should be noted that even for LCU-based filtering, the previous N-1 lines need to be cached in line buffers to avoid additional memory bandwidth as shown in Figure 4.

Assuming a 4Kx2K image with maximum ALF filter size = 9 and IBDI on, the line buffer memory size for luma is: 4K * 6 lines *12bits = 36 Kbytes which is a significant amount for single chip video codecs.

# ALF filters with reduced vertical size

For a given image size, the vertical size of ALF filters determines the size of line buffer/memory bandwidth requirements. These requirements can be reduced by reducing the vertical size of ALF filter. This contribution presents two ALF filter sets that reduce vertical size of filter. Figure 5 shows both these filter sets. Both filter sets have a maximum vertical size of 5. Since these filters operate on only five lines instead of the original seven, the size of the line buffers goes down from 6 lines to 4 lines. Nx5-Set1 reduces memory bandwidth/memory size requirements by 33% and worst case computations by 15%. Nx5-Set3 is 15% more computationally complex than existing HM ALF filters but reduces memory bandwidth/memory size requirements by 33%.

Nx5-Set 1

Nx5-Set3

Figure 5: ALF filters with reduced vertical size.

# Simulation results

Filters Nx5-Set1 and Nx5-Set3 were integrated into HM 2.0. Table below provides the BD-Rate savings of existing HM 2.0 ALF filter set, proposed Nx5-Set1 and Nx5-Set2. It can be seen from tables below the proposed ALF filter sets capture most of the ALF coding gains while reducing line buffer size/memory bandwidth by 33%.

# Conclusions

This contribution analyzes implementation complexity of Luma Adaptive loop filter (ALF) at decoder. Implementation complexity is determined by not just computations but also memory bandwidth/memory size (area). This contribution presents Luma ALF filter sets that reduce vertical size of filter thereby reducing the line buffer/memory bandwidth requirements. This contribution is a continuation of work first presented in JCTVC-D039 and studies Luma ALF filters with maximum vertical size of 5. Among the filter sets studied in this contribution, filter set Nx5-Set1 is asserted to provide the best trade-off in terms of complexity reduction and coding performance. Nx5-Set1 filter is reported to reduce memory bandwidth/memory size requirements by 33% and worst case computations by 15%. Existing HM ALF filters provide average BD-Rate savings in range of 3.1%-4.6%. Nx5-Set1 filter is reported to provide average BD-Rate savings in the range of 2.9%-4.4%. Nx5-Set1 filter captures most of the ALF coding gains. Hence we recommend that Nx5-Set1 filter be adopted in HM 3.0.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).