| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E064 |
| --- | --- |

| Title: | Improvement to AMVP/Merge process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Itani Shun-ichi Sekiguchi Tokumichi Murakami Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Kohtaro Asai Advanced Technology R&D Center Mitsubishi Electric Corporation 8-1-1, Tsukaguchi-honmachi, Amagasaki, JAPAN | Tel: Email: | +81-467-41-2463 Itani.Yusuke@dx.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

Abstract

In this contribution, two changes to AMVP/Merge process in HM-2 are proposed. First one is a modification to derivation process of predictor candidate list for AMVP by introducing motion vector scaling. The second one is to disable the use of temporally collocated motion information in both AMVP and merge mode when a PU is smaller than or equal to 8x8. The performance evaluation of the proposed changes based on HM-2 shows 0.1% and 0.3% coding gain for HE RA and LC RA, and 0.1% and 0.3% coding gain for HE LD and LC LD with no loss of coding efficiency in all inter test conditions, while keeping the same level of complexity relative to HM-2.

# Introduction

In HM-2, AMVP has been adopted as a tool for motion vector prediction and derivation of skip motion vectors. Also, merge mode in HM-2 supports adaptive selection of merge target from multiple candidate PUs. This contribution focuses on further optimization of these MV derivation tools without introducing additional complexity. This proposal consists of two changes to AMVP/Merge process in HM-2. First one is a modification to derivation process of predictor candidate list for AMVP by introducing motion vector scaling. The second one is to disable the use of temporally collocated motion information in both AMVP and merge mode when a PU is smaller than or equal to 8x8. The following sections provide their technical overviews and necessary changes to normative description are shown in the appendix part.

# Candidate list generation with MV scaling in AMVP

Predictor candidate list for AMVP in HM-2 is obtained with the steps below [1].

- For the left predictor, the first available inter PU is searched from left-neighbor PUs by doing the search from bottom-left to top-left position.

- If the available PU contains a motion vector for target RefPicList having the same ref_idx as that of current PU, the motion vector is added in candidate list.

- The top predictor is derived in a similar fashion with left predictor.

- The third predictor is derived from the collocated PU.

The principle of using motion vector candidates having the same ref_idx (i.e., the same reference picture) as that of current PU is coming from the fact that correlation in motion field is high in such case. However, in a rate-distortion sense, there should be a room to make use of scaled MV to increase number of predictor candidates even when the reference picture is not identical.

In this contribution, we propose a following simple modification to derivation process of predictor candidate list for AMVP.

- For the left predictor, the first available inter PU is searched from left-neighbor PUs by doing the search from bottom-left to top-left position.(same as HM-2)

- If the available PU contains a motion vector for target RefPicList having the same ref_idx as that of current PU, the motion vector is added in candidate list.

- If no motion vector is added in the candidate list, motion vector of the first inter PU in the left-neighbor PUs is scaled and added to the candidate list.

- The top predictor is derived in a similar fashion with left predictor.

- The third predictor is derived from the collocated PU.

It is noted that this does not require any syntax change.

# Adaptive candidate list generation for AMVP and Merge mode

D072 [2] has been adopted in HM-2 as a method to reduce the size of the motion vector buffer for storing temporal collocated MVs. With this technique, any motion vectors for smaller PUs than 16x16 are not directly stored as they are, which may result in inaccurate representation of collocated MV. In addition to that, both encoding and decoding process have to perform addition of temporally collocated motion vector to candidate list in both AMVP and Merge mode, which always requires MV scaling process. Given this observation, we propose to disable the use of collocated motion vector in PUs smaller than or equal to 8x8, for both AMVP and Merge process. This means that those PUs shall use only spatial candidates in its AMVP and Merge process. It is also noted that this does not require any syntax change.

# Simulation results

Simulations were conducted for the above proposal using the common conditions defined in [3]. Table1 and 2 show simulation results in the form of standard template for BD rate report.

As can be seen in these tables, the proposed changes result in 0.1% and 0.3% coding gain for HE RA and LC RA, and 0.1% and 0.3% coding gain for HE LD and LC LD with no loss of coding efficiency in all test conditions while keeping the same level of complexity in terms of simulation runtime. More specifically, disabling the use of temporally collocated MV in smaller PUs can simply reduce encoding complexity because of no need of checking it when generating candidate lists for AMVP and Merge modes, and can also reduce decoding complexity since the decoder does not need to perform addition of collocated MV into the candidate lists in smaller PUs. It is also noted that additional MV scaling for obtaining spatial MV candidate in AMVP should happen only when there is no MVP candidate in left/upper PUs having the same refIdx as that of the current PU.

Table 1: simulation result for random access condition

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.4 | -0.3 | -0.5 | -0.4 |
| Class B | -0.1 | 0.0 | 0.0 | -0.4 | -0.3 | -0.3 |
| Class C | -0.2 | -0.2 | -0.2 | -0.4 | -0.2 | -0.3 |
| Class D | -0.1 | -0.2 | -0.2 | -0.2 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2: Simulation result for low delay condition

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.3 | -0.4 | -0.4 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 |
| Class D | -0.2 | -0.3 | -1.0 | -0.2 | 0.0 | 0.2 |
| Class E | -0.3 | -0.6 | -0.5 | -0.3 | -0.1 | 0.2 |
| All | -0.1 | -0.2 | -0.5 | -0.3 | -0.2 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The performance of each proposed element is further shown in Table 3 to 6. It can be found that the most of the gain is coming from the simple MV scaling, and the removal of collocated MV in smaller PUs does not show noticeable performance decrease. More interestingly, the removal of collocated MV in smaller PUs sometimes brings coding gain which would mean that how to use of collocated MV would need more careful study in AMVP and Merge process.

Table 3: simulation result on proposed change in section 2 for random access condition

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.3 | -0.4 | -0.4 | -0.5 | -0.6 |
| Class B | -0.1 | 0.0 | 0.0 | -0.2 | -0.1 | -0.1 |
| Class C | -0.2 | -0.2 | -0.3 | -0.3 | -0.2 | -0.3 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 4: simulation result on proposed change in section 2 for low delay condition

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.1 | 0.0 | -0.1 | 0.0 | 0.3 |
| Class C | -0.1 | 0.0 | -0.3 | -0.2 | 0.0 | -0.2 |
| Class D | -0.2 | -0.1 | -0.4 | -0.1 | -0.1 | 0.2 |
| Class E | -0.4 | -0.5 | -0.3 | 0.0 | 0.2 | 0.4 |
| All | -0.2 | -0.1 | -0.2 | -0.1 | 0.0 | 0.2 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 5: simulation result on proposed change in section 3 for random access condition

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class C | 0.1 | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | 0.3 | 0.1 | 0.0 | 0.2 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6: simulation result on proposed change in section 3 for low delay condition

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.6 | -0.2 | -0.4 | -0.2 |
| Class C | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 | -0.1 |
| Class D | 0.0 | -0.5 | -0.4 | 0.1 | 0.0 | 0.0 |
| Class E | -0.4 | -0.7 | -0.2 | -0.3 | -0.1 | 0.0 |
| All | 0.0 | -0.3 | -0.3 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

We have proposed two changes to AMVP/Merge process that can further optimize HM-2 specification with straight-forward modifications. Our simulation results showed up to 0.3% bitrate reduction without finding any cases those cause loss of coding efficiency, while keeping the same level of complexity relative to HM-2. We recommend inclusion of the proposed changes into the HM design.

References:

[1] Akira Fujibayashi, Frank Bossen , “CE9 3.2d Simplified Motion vector prediction,” JCTVC-D231, Jan. 2011.

[2] Yeping Su, Andrew Segall, “CE9: Reduced resolution storage of motion vector data,” JCTVC-D072, Jan. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

(A) Add the following text with yellow marker into section 8.4.2.1.3.

##### 8.4.2.1.3  Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.4.2.1.4 is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- When one of the width and the height of the prediction unit nPSW and nPSH is larger than 8, the derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.7 is invoked with luma location ( xP, yP ), refIdxLXCol (with X being 0 or 1, respectively) equal to 0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

When both nPSW and nPSH are smaller than or equal to 8, availableFlagCol shall be set to 0.

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A, if availableFlagA is equal to 1

  - B, if availableFlagB is equal to 1

  - Col, if availableFlagCol is equal to 1

  - C, if availableFlagC is equal to 1

  - D, if availableFlagD is equal to 1

- If several merging candidates have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

(B) Add the following text with yellow marker into section 8.4.2.1.5.

##### 8.4.2.1.5  Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- When one of the width and the height of the prediction unit nPSW and nPSH is larger than 8, the derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.7 is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

When both nPSW and nPSH are smaller than or equal to 8, availableFlagLXCol shall be set to 0.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.

(C) Add the following text with yellow marker into section 8.4.2.1.5.

##### 8.4.2.1.6  Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-4 – Spatial motion vector neighbours

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of nA sample locations be (xAk, yAk), with k = 0 .. nA, specifies sample locations with xAk = xP – 1, yA0 = yP and yAk-1 = yAk - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nA = nPSH / MinPuSize]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xAnA, yAnA ) to ( xA0, yA0 ) where yAk-1 = yAk - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ].

- If availableFlagLXA is equal to 0 and if the first available prediction unit covering luma location ( xAk, yAk ) whose PredMode is not MODE_INTRA and predFlagLX[ xAk ][ yAk ] is equal to 1, the derivation process for motion vector scaling in subclause 8.4.2.1.8 is invoked with the motion vector mvLX[ xAk ][ yAk ], the reference index refIdxLX[ xAk ][ yAk ], and the current reference index refIdxLX as inputs and the the scaled motion vector mvLXScaled[ xAk ][ yAk ] as the output. The availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the scaled motion vector mvLXScaled[ xAk ][ yAk ].

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of nB sample location (xBk, yBk), with k = -1,0 .. nB, specifies sample locations with xB-1 = xP - MinPuSize, xB0 = xP , yBk = yP – 1, and xBk-1 = xBk - MinPuSize. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nB = nPSW / MinPuSize]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xBnB-1, yBnB-1 ) where xBk-1 = xBk - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ].

- If availableFlagLXB is equal to 0 and if the first available prediction unit covering luma location ( xBk, yBk ) whose PredMode is not MODE_INTRA and predFlagLX[ xBk ][ yBk ] is equal to 1, the derivation process for motion vector scaling in subclause 8.4.2.1.8 is invoked with the motion vector mvLX[ xBk ][ yBk ], the reference index refIdxLX[ xBk ][ yBk ], and the current reference index refIdxLX as inputs and the the scaled motion vector mvLXScaled[ xBk ][ yBk ] as the output. The availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the scaled motion vector mvLXScaled[ xBk ][ yBk ].

(D) Replace the description of section 8.4.2.1.8 to the following text.

##### 8.4.2.1.8  Derivation process for scaled luma motion vector prediction

Inputs to this process are

- the motion vectors mvLXN of the neighbouring prediction unit PuN,

- the reference index refIdxLXN of the neighbouring prediction unit PuN,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the scaled motion vector prediction mvLXNScaled.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The mvLXNScaled is derived as follows.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-85)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-86)

mvLXNScaled = ClipMv( ( DistScaleFactor * mvLXN + 128 ) >> 8 )	(8-87)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLXN, L0 ) )	(8-88)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-89)

(E) Move the current content of section 8.4.2.1.8 to 8.4.2.1.9.