| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E065 |
| --- | --- |

| Title: | Report of CE8 Subtest4: Cross-verification on Low Complexity ALF design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi Kazuo Sugimoto Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

Abstract

This contribution reports cross-verification results on CE8 subtest4 that evaluates a low-complexity ALF design proposed in JCTVC-D270. The cross-verification work was performed using software that implements D270 technique into HM2.0 code base provided by the D270 proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that objective R-D performance produced in this work perfectly matches with CE report from the proponent. Similar tendency on encoding and decoding time was also observed.

# Introduction

Performance verification on CE8 subtest4 approved at the last Deagu meeting [1] was conducted. The verified method is Parametric ALF (PALF) technique proposed by Sony in JCTVC-D270 [2]. This work was conducted using modified HM2.0 software provided by Sony.

# Simulation results

Table 1 shows test conditions used in our experiment, which is to test configuration defined in the CE8 document [1]. We built executables from source code provided by Sony and ran it onto 64bit Linux operating system (more specifically Fedora 10 x86_64). Encoding and decoding time for both anchor and the test case were measured using “time” command in the Linux platform.

Table 1  Test Condition

| Codec software | Implementation of the tool proposed in JCTVC-D270[2] based on HM2.0 software |
| --- | --- |
| Test configuration | D600[3] |
| QP setting | QPI=22,37,32,37 |

Since the technique has been proposed as a tool for coding performance improvement in LC condition, we obtained results for all encoding configurations in LC condition. It is noted that, in this case, the HM2.0 anchor does not use ALF feature at all and the tested configuration adds PALF to the anchor configuration.

The objective performance data and simulation time obtained in this work are shown in Table 2 to 4. It has been confirmed that BD rate reduction results perfectly match with the CE report from the D270 proponent [4]. Similar tendency on encoding and decoding time was also observed. In summary, it can be said that the PALF brings about 3% coding gain in average at the expense of around 30% increase of decoding time in LC condition.

Table 2 Performance for All Intra / Low Complexity Condition

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.4 | -1.1 | -0.9 |
| Class B | -1.6 | -1.9 | -2.3 |
| Class C | -1.6 | -1.7 | -2.9 |
| Class D | -0.7 | -1.1 | -1.5 |
| Class E | -2.7 | -3.3 | -3.7 |
| All | -1.9 | -1.8 | -2.2 |
| Enc Time[%] | 133% |  |  |
| Dec Time[%] | 151% |  |  |

Table 3 Performance for Random Access / Low Complexity Condition

|  | Random access LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -5.3 | -4.0 | -4.3 |
| Class B | -3.8 | -4.0 | -2.9 |
| Class C | -2.4 | -1.4 | -2.4 |
| Class D | -2.1 | -0.1 | -0.1 |
| Class E |  |  |  |
| All | -3.4 | -2.5 | -2.5 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 123% |  |  |

Table 4 Performance for Low-Delay / Low Complexity Condition

|  | Low delay LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -5.1 | -6.7 | -6.1 |
| Class C | -3.8 | -3.8 | -5.8 |
| Class D | -2.9 | -1.4 | -1.5 |
| Class E | -4.5 | -11.6 | -9.5 |
| All | -4.1 | -5.6 | -5.5 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 124% |  |  |

As optional information, we also checked coding performance of the PALF in HM2.0 HE condition. In this optional test, the anchor includes the current HM2.0 ALF and the test cases replace the HM2.0 ALF to PALF. The results are shown in Table 5 to 7. The PALF decreases 13% of encoding time and 25% of decoding time in average at the expense of 1.2% performance loss.

Table 5 Performance for All Intra Conditions

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.6 | 0.4 | 0.1 |
| Class B | 1.7 | 0.2 | -0.8 |
| Class C | 1.2 | 1.0 | 0.9 |
| Class D | 0.8 | 1.8 | 1.9 |
| Class E | 2.6 | -0.5 | -0.2 |
| All | 1.7 | 0.6 | 0.3 |
| Enc Time[%] | 74% |  |  |
| Dec Time[%] | 74% |  |  |

Table 6 Performance for Random Access Conditions

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.7 | 0.6 | 0.5 |
| Class B | 2.1 | -1.8 | -3.1 |
| Class C | 0.8 | -1.9 | -1.6 |
| Class D | 0.3 | -0.4 | -0.6 |
| Class E |  |  |  |
| All | 1.7 | -0.9 | -1.3 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 74% |  |  |

Table 7 Performance for Low-Delay Conditions

|  | Low delay |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.7 | -4.7 | -6.4 |
| Class C | 0.0 | -4.0 | -5.0 |
| Class D | -0.8 | -2.2 | -2.2 |
| Class E | 1.8 | -6.3 | -8.8 |
| All | 0.3 | -4.2 | -5.4 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 76% |  |  |

References:

- Tomoo Yamakage, et.al. “Description of CE8: Non-deblocking loop filtering,” JCTVC-D608, Jan. 2011.

- E. Maani, et.al., “Low Complexity Parametric Adaptive Loop Filter,” JCTVC-D270, Jan. 2011.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- D600, Jan. 2011.

- E. Maani, et.al., “Parametric Adaptive Loop Filter,” JCTVC-E320, March 2011.