| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E067 WG11 Number: m19577 |
| --- | --- |

| Title: | CE6.b1: Cross-verification report on Short Distance Intra Prediction (JCTVC-E278) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi 5-1-1 Ofuna, Kamakura-shi Kanagawa, 247-8501 Japan | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution is a cross-verification report on short distance intra prediction defined one of CE6 items described in JCTVC-D606[1]. There are three versions of software named as Normal version, 1x4_OFF version and 1x4_OFF_BUDI_OFF version. In this contribution, a verification result on 1x4_OFF_BUDI_OFF version is reported.

The verification tests have been conducted using common intra test conditions defined in JCTVC-D600. It is reported that average BD-rate gain for high efficiency and low complexity conditions were 2.0% and 3.2% respectively. It is also reported that the increase of encoding time for high efficiency and low complexity conditions were 16% and 28% respectively.

All test results reportedly matched those reported in JCTVC-E278.

# Experimental Conditions

## Software

We used HM-2.0 with short distance intra prediction provided by Huawei. There are three versions of the software as follows:

- Normal version

  - All Intra CU can signal I_PCM mode.

- 1x4_OFF version

  - I_PCM mode can be signaled in each CU larger than or equal to 32x32.

- 1x4_OFF_BUDI_OFF version

  - I_PCM mode can be signaled in each CU smaller than or equal to 32x32.

The 1x4_OFF_BUDI_OFF version of the software has been verified.

## Test Sequences, Bit Rates and Coding Conditions

All verification test are conducted under the default test conditions for intra defined in JCTVC-D600[2], using the reference configuration files also provided in [2]. Our tests are conducted on Linux OS with the same CPU/Memory equipped PCs.

# Test Results

The test results are shown in Table 1.

Table 1. test results on 1x4_OFF_BUDI_OFF version

# Conclusion

We confirmed that the implementation of short distance intra prediction in HM-2.0 is consistent, and all test results matched the results from the proponent.

# References

- Ali Tabatabai, “Core Experiment 6: Intra Prediction Improvement”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D606, 4th Meeting: Daegu, KR, Jan., 2011.

- Frank Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, KR, Jan., 2011.