| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E068 WG11 Number: m19578 |
| --- | --- |

| Title: | CE6.g: verification report on number of intra prediction directions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi 5-1-1 Ofuna, Kamakura-shi Kanagawa, 247-8501 Japan | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution is a verification report of Core Experiment 6.g on number of intra prediction directions. As described in JCTVC-C606, five test cases are verified using all intra conditions, and bit increase in BD-rate compared to the anchor are reported as follows:

(Test #1) using only 17 instead of 34 modes for 8x8

0.424% and 0.848% for high efficiency and low complexity conditions respectively.

(Test #2) using only 17 instead of 34 modes for 16x16

0.239% and 0.394% for high efficiency and low complexity conditions respectively.

(Test #3) using only 17 instead of 34 modes for 32x32

0.152% and 0.091% for high efficiency and low complexity conditions respectively.

(Test #4) using only 9 instead of 34 modes for 32x32

0.235% and 0.115% for high efficiency and low complexity conditions respectively.

(Test #5) using only DC prediction mode instead of 3 modes for 64x64

0.012% and 0.015% for high efficiency and low complexity conditions respectively.

It is proposed by the contributor that only DC prediction mode instead of 3 modes for intra 64x64 should be supported in the next version of the HEVC test model.

# Experimental Conditions

## Software

We modified HM-2.0 by limiting the number of intra prediction directions according to Table 1. For test #5, intra prediction mode is not signaled for 64x64 block as there is only DC prediction mode. The software patch used in this CE is attached to this contribution.

Table 1: Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |
| 4x4 | 17 | 17 | 17 | 17 | 17 | 17 |  |  |  |  |  |  |
| 8x8 | 34 | 17 | 34 | 34 | 34 | 34 |  |  |  |  |  |  |
| 16x16 | 34 | 34 | 17 | 34 | 34 | 34 |  |  |  |  |  |  |
| 32x32 | 34 | 34 | 34 | 17 | 9 | 34 |  |  |  |  |  |  |
| 64x64 | 3 | 3 | 3 | 3 | 3 | 1 |  |  |  |  |  |  |

## Test Sequences, Bit Rates and Coding Conditions

Test conditions are the intra-only configuration on both High Efficiency and Low Complexity conditions and test sequences as defined in JCTVC-D600[1], using the reference configuration files also provided in [1]. Our tests are conducted on Linux OS with the same CPU/Memory equipped PCs.

# Test Results

The summary of test results is shown in Table 2, and detailed test results for test #1 to #5 are shown in Table 3-7 respectively.

Table 2. The summary of the test results

|  |  |  |  |  |  | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | Enc Time | BDrate | Enc Time |
| Reference | 17 | 34 | 34 | 34 | 3 |  |  |  |  |
| Test#1 | 17 | 17 | 34 | 34 | 3 | 0.424 | 98.46% | 0.848 | 96.41% |
| Test#2 | 17 | 34 | 17 | 34 | 3 | 0.239 | 99.01% | 0.394 | 97.33% |
| Test#3 | 17 | 34 | 34 | 17 | 3 | 0.152 | 99.29% | 0.091 | 97.71% |
| Test#4 | 17 | 34 | 34 | 9 | 3 | 0.235 | 98.68% | 0.115 | 96.45% |
| Test#5 | 17 | 34 | 34 | 34 | 1 | 0.012 | 98.32% | 0.015 | 99.03% |

Table 3. test #1 results

Table 4. test #2 results

Table 5. test #3 results

Table 6. test #4 results

Table 7. test #5 results

# Analysis

According to the test results, all test patterns except #5 show some bit increase in terms of BD-rate. According to the test #5 result, using only DC prediction mode for 64x64 performs almost the same as reference with 1-2% complexity reduction for encoding.

# Conclusion

For 64x64 block size, limiting number of intra prediction mode from 3 to 1 does not affect coding performance but gives small encoder complexity reduction. We propose that only DC prediction mode instead of 3 modes for intra 64x64 shall be supported in the next HEVC test model.

# References

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, KR, Jan., 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).