| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E069 WG11 Number: m19579 |
| --- | --- |

| Title: | CE6.f: LUT-based adaptive filtering on intra prediction samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Akira Minezawa Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Kazuhisa Iguchi Yoshiaki Shishikui NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-467-41-2463 Kazuo.Sugimoto@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation and NHK(Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

In this contribution, a test result on LUT-based adaptive filtering on intra prediction samples defined in Core Experiment 6.f is reported. It is reported that the BD-rate gain of the tested scheme were 0.2% and 0.4% with regards to HM-2.0 all intra default configurations of high efficiency and low complexity settings respectively with 1% lower encoding complexity and 1-2% lower decoding complexity. A test result on using both the tested scheme and a planar intra prediction with DST proposed in JCTVC-D235[1] is also reported. In this case, it is reported that the BD-rate gain of the tested scheme was 0.2% for both high efficiency and low complexity settings.

# Introduction

A simplified version of intra prediction samples filtering proposed in [2] is tested in CE6.f. The proposed scheme applies 2 tap filtering on most upper and left edge samples of DC prediction samples with simplified MDIS. Filter strength is decided based on a lookup table.

# Tested scheme

The tested scheme applies filtering only on most upper and left edge samples of DC prediction as shown in Figure 1, where only left and upper most pixel lines of the prediction samples are filtered using 2 tap filters except top-left corner sample using one of the filters shown in Figure 2 according to the block size. The top-left corner sample in DC prediction samples is filtered using 3 tap filters shown in Figure 3. In our proposed scheme, instead of MDIS, only one time filtering is applied for intra smoothing as shown in Figure 4 instead of two times filtering in MDIS as shown in Figure 5. The LUT for the proposed scheme and MDIS are shown in Figure 6.

Figure 1. DC prediction samples to be filtered

Figure 2. filter coefficients for DC prediction edges except the top-left corner sample

Figure 3. filter coefficients for the top-left corner sample in DC prediction samples

Figure 4.  Reference samples to be filtered in the proposed scheme

Figure 5.  Reference samples to be filtered in MDIS

-

Proposed scheme

{0, 0,

A

, 1, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //4x4

{0, 0,

B

, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //8x8

{0, 0,

C

, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //16x16

{0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0

, 0}, //32x32

{0, 0, 0, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //64x64

-

MDIS

{0, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //4x4

{0, 0, 0, 1, 1, 2, 2, 2, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0}, //8x8

{0, 0, 0, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //16x16

{0, 0, 0, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2,

2, 2, 2, 2, 2, 2, 0, 0, 0, 0, 0, 0}, //32x32

{0, 0, 0, 2, 2, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //64x64

Figure 6.  LUT for each intra prediction unit size and intra prediction mode

# Simulations & Results

The proposed scheme is implemented on top of HM-2.0, and the simulations are conducted using the common test configurations[3] for all intra conditions. Table 1 shows the comparison results in high efficiency and low complexity intra coding respectively.

Table 1. BD Bitrate saving of the proposed scheme

We also conducted a simulation on the proposed scheme using software for CE6.e on Planar+DST mode proposed in [1]. The macro settings used in this test is shown in Figure 7.

Figure 7.  Macro settings for planar+DST mode

The BD-rate gain achieved with CE6.e software and CE6.e+proposed scheme using the macro settings described above are shown in tables 2 and 3 respectively.

Table 2. BD Bitrate saving of Planar+DST mode

Table 3. BD Bitrate saving of Planar+DST with the proposed scheme

We have also conducted simulations on simplification of MDIS adopted in HM2.0 and the part of DC prediction filtering separately on top of HM2.0. Table 4 and 5 show these comparison results with HM2.0 anchor respectively. In Table 4, it is shown that the part of simplification of MDIS causes slight loss of coding efficiency but there is a bit of complexity reduction. It can also be found in Table 5 that applying the part of DC prediction filtering to the current HM2.0 process shows more gain than what is reported in Table 1. This confirms that overall coding gain reported in Table 1 comes from the part of DC prediction filtering. More complete results per each test sequences are attached as additional Excel sheets.

Table 4. BD Bitrate saving of simplified reference samples filtering

Table 5. BD Bitrate saving of MDIS+DC prediction samples filtering

# Conclusion

According to the test results, the proposed scheme achieves 0.2% and 0.4% BD-rate gain with regards to HM-2.0 all intra default configurations of high efficiency and low complexity settings respectively with lower complexity.

The improvement using the proposed scheme on top of planar intra prediction was 0.2% BD-rate for both high efficiency and low complexity settings with lower complexity.

We propose the verified intra prediction filtering scheme to be adopted to HM-3.

# Reference

- Sandeep Kanumuri, et al., “Enhancements to Intra Coding”, Doc. JCTVC-D235, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011

- Kazuo Sugimoto, et al., “LUT-based adaptive filtering on intra prediction samples”, Doc. JCTVC-D109, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011

- JCT-VC, “Common test conditions and software reference configurations” , Doc. JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

The WD text for JCTVC-E069 on top of adopted solution in JCTVC-E488 (BoG report on padding of unavailable reference samples for intra prediction) is specified below.

Relevant changes/additions for E069 are indicated by Green marker.

8.3.3.1	Intra sample prediction

…

The nS*4+1 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y=-1, are derived as follows.

- The luma location ( xBN, yBN ) is specified by

xBN = xB + x 	(8-1)

yBN = yB + y 	(8-2)

- The derivation process for neighbouring locations in subclause XXX is invoked for sample location with ( xBN, yBN ) as input and the treeblock address tbAddrN as output.

- Each sample p[ x, y ] with x = -1, y= -1..nS*2-1  and x = 0..nS*2-1, y = -1 is derived as follows

  - If any of the following condition is true, the sample p[ x, y ] is marked as “not available for intra prediction”

- tbAddrN is not available

- the coding unit covering ( xBN, yBN ) is not coded as intra mode and constrained_intra_pred_flag is equal to 1

  - Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the sample at the location ( xBN, yBN ) inside the treeblock tbAddrN is assigned to p[ x, y ]

When any samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction,” samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction.”

When any samples p[ -1, y ], with y = 0..nS-1 are marked as “not available for intra prediction,” samples p[ -1, y ], with y = 0.. nS-1 are marked as “not available for intra prediction.”

When any samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ nS-1, -1 ] is marked as “available for intra prediction” the sample value of p[ nS-1, -1 ] is substituted for samples p[ x, -1 ], with x = nS..nS*2-1 and samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “available for intra prediction”.

When any samples p[ -1, y ], with y = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ -1, nS-1 ] is marked as “available for intra prediction” the sample value of p[ -1, nS-1 ] is substituted for samples p[ -1, y ], with y = nS..nS*2-1 and samples p[ -1, y ], with y = nS..nS*2-1 are marked as “available for intra prediction”.

Otherwise, if all samples p[ x, y ] are marked as “not available for intra prediction,” the following applies.

p[ x, y ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-16)

When at least one sample p[ x, y ] with x = -1, y= -1..nS-1 and x = 0..nS-1, y = -1 is marked as “available for intra prediction,” DCPredFilterFlag is set equal to 1. Otherwise, DCPredFilterFlag is set equal to 0.

When at least one sample p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 is marked as “not available for intra prediction,” the reference sample substitution process for intra sample prediction in subclause 8.3.3.1.1 is invoked with the samples p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 as input and the modified samples p[ x, y ] with x = -1, y= -1..nS*2-1 and x = 0..nS*2-1, y = -1 as output.

[Ed.: If necessary, specifying p[ x, y ] as both input and output can be changed to specifying p[ x, y ] as input and p'[ x, y ] as output (subsequent subclauses needs to be changed accordingly).]

[Ed.: After invoking subclause 8.3.3.1.1, all reference samples p[ x, y ] are marked as “available for intra prediction”.]

Table 8-1 specifies the value for intraPredMode and the associated names.

- Table 8-1 – Specification of intraPredMode and associated names

| intraPredMode | Name of intraPredMode |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33) | Intra_Angular |

Depending on intraPredMode, the following ordered steps apply:

- IfWhen cIdx is equal to 0, filtering process of neighbouring samples specified in Error: Reference source not found8.3.3.1.2 is invoked with the sample array p and the prediction size nS as the inputs and the output is a (2*nS+1)x(2*nS+1) filtered sample array pF.

- One of the intra prediction modes specified in subclause  to Error: Reference source not found8.3.3.1.3 to 8.3.3.1.6 is invoked with the sample array pF and the prediction size nS as the inputs and the output is the predicted sample array predSamples.

- If intra PredMode is equal to 2 and DCPredFilterFlag is equal to 1, filtering process of DC prediction samples specified in 8.3.3.1.7 is invoked with the prediction size nS and the sample array p and the DC prediction samples predSamples as the inputs and the output is assigned to a nSxnS filtered DC prediction sample array predSamples.

[Ed.: The above steps are phrased differently from AVC and may need some improvement.]

8.3.3.1.1		Reference sample substitution process for intra sample prediction

…… (as specified in E488)

8.3.3.1.18.3.3.1.2		Filtering process of neighbouring samples

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	filtered samples pF[ x, y ],. with x, y = -1..2*nS-1.

Table 8-2 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 0 |
| 9 | 1 | 1 | 1 | 1 | 0 |
| 10 | 0 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 1 | 1 | 0 |
| 12 | 0 | 0 | 1 | 1 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 |
| 14 | 0 | 0 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 1 | 0 |
| 17 | 0 | 0 | 1 | 1 | 0 |
| 18-33 | 0 | 0 | 0 | 1 | 0 |

Filtered sample array pF[ x, y ] with x = -1..nS*2-1 and y = -1..nS*2-1  are derived as follows:

–	If intraFilterType[ nS ][ IntraPredMode ] is equal to 1, the following applies:

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]		(8-3)

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]		(8-4)

pF[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-5)

pF[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2	(8-6)

pF[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-7)

–	Otherwise, intraFilterType[ nS ][ IntraPredMode ] is equal to 2, the following applies:

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]		(8-8)

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]		(8-9)

pF1[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-10)

pF1[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2	(8-11)

pF1[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-12)

pF[ -1, y ] = ( pF1[ -1, y+1 ] + 2*pF1[ -1, y ] + pF1[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-13)

pF[ -1, -1] = ( pF1 [ -1, 0 ] + 2*pF1 [ -1, -1] + pF1 [ 0, -1 ] + 2) >> 2	(8-14)

pF[ x, -1 ] = ( pF1 [ x-1, -1 ] + 2* pF1 [ x, -1 ] + pF1 [ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-15)

8.3.3.1.28.3.3.1.3		Specification of Intra_Vertical prediction mode

…

This mode shall be used only when the samples p[ x, -1 ] with x = 0..nS-1 are marked as “available for intra prediction”.

…

8.3.3.1.38.3.3.1.4		Specification of Intra_Horizontal prediction mode

…

This mode shall be used only when the samples p[ -1, y ] with y = 0..nS-1 are marked as “available for intra prediction”.

…

8.3.3.1.48.3.3.1.5		Specification of Intra_DC prediction mode

…

8.3.3.1.58.3.3.1.6		Specification of Intra_Angular prediction mode

…

8.3.3.1.7		 Filtering process of DC prediction samples

Inputs to this process are:

–	a variable nS specifying the prediction size,

–	neighbouring samples p[ x, y ], with x, y = -1..nS-1,

–	DC predicted samples predSamples[ x, y ], with x, y =0..nS-1.

Output of this process is:

–	filtered DC predicted samples predSamplesF[ x, y ], with x, y =0..nS-1.

Table 8-xx – Specification of DCFilterType[ nS ] for various prediction unit sizes

| intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- |
| 3 | 2 | 1 | 0 | 0 |

Filtered sample array predSamplesF [ x, y ] with x, y = 0..nS-1, are derived as follows:

–	If DCFilterType [ nS ] is equal to 0, the following applies:

PredSamplesF[ x, y ] = predSamples[ x, y] for x,y = 0..nS-1	(8-x0)

–	If DCFilterType [ nS ] is equal to 1, the following applies:

predSamplesF[ 0, 0 ] = ( 3*p[ -1, 0 ] + 2*predSamples[ 0, 0 ] + 3*p[ 0, -1 ] + 4 ) >> 3	 (8-x1)

PredSamplesF[ x, 0 ] = ( 3*p[ x, -1] + 5*predSamples[ x, 0] + 4 ) >> 3 for x = 0..nS-1	(8-x2)

PredSamplesF[ 0, y ] = ( 3*p[ -1, y] + 5*predSamples[ 0, y] + 4 ) >> 3 for y = 0..nS-1	(8-x3)

PredSamplesF[ x, y ] = predSamples[ x, y] for x,y = 1..nS-1	(8-x4)

–	If DCFilterType [ nS ] is equal to 2, the following applies:

predSamplesF[ 0, 0 ] = ( p[ -1, 0 ] + 2*predSamples[ 0, 0 ] + p[ 0, -1 ] + 2 ) >> 2	 (8-x5)

PredSamplesF[ x, 0 ] = ( p[ x, -1] + 3*predSamples[ x, 0] + 2 ) >> 2 for x = 0..nS-1	(8-x6)

PredSamplesF[ 0, y ] = ( p[ -1, y] + 3*predSamples[ 0, y] + 2 ) >> 2 for y = 0..nS-1	(8-x7)

PredSamplesF[ x, y ] = predSamples[ x, y] for x,y = 1..nS-1	(8-x8)

–	If DCFilterType [ nS ] is equal to 3, the following applies:

predSamplesF[ 0, 0 ] = ( p[ -1, 0 ] + 6*predSamples[ 0, 0 ] + p[ 0, -1 ] + 4 ) >> 3	 (8-x9)

PredSamplesF[ x, 0 ] = ( p[ x, -1] + 7*predSamples[ x, 0] + 4 ) >> 3 for x = 0..nS-1	(8-x10)

PredSamplesF[ 0, y ] = ( p[ -1, y] + 7*predSamples[ 0, y] + 4 ) >> 3 for y = 0..nS-1	(8-x11)

PredSamplesF[ x, y ] = predSamples[ x, y] for x,y = 1..nS-1	(8-x12)