| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E070 |
| --- | --- |

| Title: | CE12 Subset2: Cross-verification of Panasonic’s proposal JCTVC-Exxx |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-Exxx proposed by Panasonic. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-D600. Proposed tools have been evaluated on the common test condition. Detailed results are summarized in the attached excel sheet. In addition subjective quality is evaluated on test sequences, QP and coding conditions selected in CE12. The source code was provided by Panasonic to cross verify. It is confirmed that all results perfectly match ones provided by Panasonic.

# Introduction

Panasonic’s proposal [1] is included in subset2 of CE12 which is techniques related to parallelization for deblocking filter. The BD-rate, the measurement of enc/dec-time, and subjective evaluation is shown as below.

# Results

## Test condition and machine platform

Experiment was carried out based on HM-2.0 and the configuration and common test condition are as JCTVC-D600 [2].

Both encoding and decoding time were measured on the platform which has Core i7 CPU2.8GHz, 8GB memory and the Linux 64bit (called Cent OS).

## BD-rate and measurement of enc/dec-time

The experimental results are summarized in the following tables. The detailed results are shown in attached excel-file.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table2-1: Results for intra

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.3 | 0.0 | 0.0 | 0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-2: Results for randomaccess

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class E | -0.1 | 0.0 | 0.1 | 0.0 | -0.3 | 0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table2-3: Results for lowdelay

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1 randomaccess QP=37

- Kimono1 lowdelay_loco QP=32

- BQMall randomaccess QP=37

- BQMall lowdelay QP=32

- Vidyo3 lowdelay QP=37

- Vidyo3 lowdelay_loco QP=32

Equipments

- Monitor: EIZO FlexScan SX3031W

- Processor : Intel Core i5 750 BOX

- RAM : DCDDR3-4GB-1333

- Graphic Card : ENGTX285 2DI 1GD3

The difference between Panasonic’s proposal and anchor is invisible, and so Panasonic’s proposal has no impact on subjective quality.

# Conclusion

In this contribution, the cross verified results of deblocking filter by Panasonic are reported. It is confirmed that the results perfectly match the ones provided by Panasonic and that their proposal has no impact on subjective quality.

# References

[1] JCT-VC, “xxx”, JCTVC-Exxx,5th. JCT-VC Meeting, Geneva, CH, March 2011

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600,4th. JCT-VC Meeting, Daegu, KR, January 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).