| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E072 |
| --- | --- |

| Title: | CE5: Adaptive Coding of InterPredMode Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Zhang Ming Li Ping Wu | Tel: Email: | +86-13528803560 zhang.wen4@zte.com.cn li.ming42@zte.com.cn pwuzte@gmail.com |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In HM-2.0, several syntax elements, i.e. SkipFlag, MergeFlag and the flags in codePredMode and codePartSize function are used to signal different inter-frame prediction modes for CUs. In JCTVC-D275, an alternative representing method was proposed to utilize the varying statistical features of this InterPredMode related syntax elements among slices in the entropy coding process. This approach has been further studied based on HM2.0, and achieves average gains of -0.1% and -0.3% for RALC and LDLC configurations, respectively.

# Introduction

In HM-2.0, the representation of InterPredMode is related to several functions, i.e. codeSkipFlag, codeMergeFlag, and codePredMode combined with codePartSize which are used to write the signaling bits for inter-frame prediction mode. As proposed in JCTVC-D275, a syntax element named pred_mode_coding_type is introduced into P/B-slice header to indicate different coding types used in current slice to code the syntax elements of InterPredMode, where coding type is defined as type of coding order of the InterPredMode related elements. The encoder determines pred_mode_coding_type value for the current slice according to the statistical results of the occurrence of the inter-frame prediction modes obtained from the previous coded slices, which result in the ability of adaptive coding of the syntax elements related to InterPredMode at the slice or frame level.

Based on HM2.0, CU split flag is involved in this adaptive coding method. And since there are 4 flags for InterPredMode, Skip (denoted as SKP), Merge (MRG), Inter (TER) and Intra (TRA), coding types are defined by specifying the order of the first three coded flags. The defined coding types by pred_mode_coding_type are listed in Table 1.

In this proposal, CU split flag is jointly coded with the InterPredMode flags. Therefore, the coding types listed in Table 2 are designed to signal the coding order of the split flag based on the already determined coding order of InterPredMode flags among different quad-tree layers of CU. The corresponding modification of the slice header syntax is shown in Table 3.

Table 1 pred_mode_coding_type

| SKP_MRG_TER_TRA | Coding SkipFlag first, then MergeFlag, InterFlag |
| --- | --- |
| SKP_TER_MRG_TRA | Coding Skipflag first, then InterFlag, MergeFlag |
| SKP_TRA_MRG_TER | Coding Skipflag first, then IntraFlag, MergeFlag |
| MRG_SKP_TER_TRA | Coding MergeFlag first, then SkipFlag, InterFlag |
| MRG_TER_SKP_TRA | Coding MergeFlag first, then InterFlag, SkipFlag |
| MRG_TRA_SKP_TER | Coding MergeFlag first, then IntraFlag, SkipFlag |
| TER_SKP_MRG_TRA | Coding InterFlag first, then SkipFlag, MergeFlag |
| TER_MRG_SKP_TRA | Coding InterFlag first, then MergeFlag, SkipFlag |
| TER_TRA_SKP_MRG | Coding InterFlag first, then IntraFlag, SkipFlag |
| TRA_SKP_MRG_TER | Coding IntraFlag first, then SkipFlag, MergeFlag |
| TRA_MRG_SKP_TER | Coding IntraFlag first, then MergeFlag, SkipFlag |
| TRA_TER_SKP_MRG | Coding IntraFlag first, then InterFlag, SkipFlag |

Table 2 split_flag_coding_type

| SPT_FLAG_1ST | SplitFlag in 1st coding order of inter prediction mode |
| --- | --- |
| SPT_FLAG_2ND | SplitFlag in 2nd coding order of inter prediction mode |
| SPT_FLAG_3RD | SplitFlag in 3rd coding order of inter prediction mode |
| SPT_FLAG_END | SplitFlag in last coding order of inter prediction mode |

Table 3 Syntax elements added in slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| pred_mode_coding_type | 2 | ue(v) |
| split_flag_coding_type_depth0 | 2 | ue(v) |
| split_flag_coding_type_depth1 | 2 | ue(v) |
| split_flag_coding_type_depth2 | 2 | ue(v) |
| num_ref_idx_active_override_flag | 2 | u(1) |

- Experimental results

The proposed method was evaluated using the recommended common test condition presented in JCTVC-D600. Two configurations (RALC and LDLC) were employed in the experiments to get the anchors. The test platform is HP server E5506/2.13 GHz with Windows NT server 2003. The test results for all sequences from class A to E are shown in Table 3.

Table 3 Test results of PredMode adaptive coding

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | #VALUE! | #VALUE! | #VALUE! | -0.1 | -0.1 | -0.1 |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.2 | 0.1 | 0.1 |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.1 | 0.0 | 0.0 |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.4 | -0.3 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | #VALUE! | #VALUE! | #VALUE! | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.2 | -0.3 | 0.0 |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.5 | -0.3 | -0.4 |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.6 | -0.7 | -0.4 |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.1 | -0.4 | -0.3 |
| All | #VALUE! | #VALUE! | #VALUE! | -0.3 | -0.4 | -0.3 |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusions

In this document, an alternative adaptive coding method for InterPredMode information is proposed which further removed the redundancy of current design in HM2.0. As demonstrated by the experimental results, the proposed method achieves an average efficiency improvement of 0.2% with none computational complexity increment/decrement. It is recommended to further study this syntax elements adaptive coding at slice or frame level in a CE or AHG.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).