| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E074 WG11 Number: m19584 |
| --- | --- |

| Title: | CE9: Mode Dependent 2-step Transform for Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes a unification of MDDT and ROT, in which a mode dependent adaptive DCT and DST transform is used for 4x4 transform blocks and a mode dependent 2-step transforms are used for larger ones. The adaptive DCT and DST is same as the proposal in JCTVC-B024, except the fast implementation. The first one of the 2-step transform is either AVC transform or Chen-DCT in the HM version 2, and the second one is 4x4 KLT which is partially applied to the lower frequency components. Application of the second transform is depending on the intra prediction mode of the corresponding prediction unit. An experimental result reportedly shows that the proposed method provides 1.1% and 0.5% gain for Intra HE and Random Access HE, respectively, over the HM version 2 anchors. The encoding time (and decoding time) is 101% (102%) for Intra HE and no major difference was reportedly observed for Random Access HE.

- Introduction

The KLT which is designed for intra prediction residual is an effective technology in terms of coding efficiency. On the other hand, however, its complexity is higher than DCT’s in terms of number of operation and encoder/decoder software runtime. This contribution presents an approach for the unification of MDDT and ROT, which encoding/decoding runtimes are comparable with those of HM2.0.

# Solutions and experimental results

## Concept for the reduction of operation counts

We use 2-step transform to reduce the number of operation counts, especially for the multiplication. For large transform blocks including 8x8, 16x16 and 32x32, the first transform is either AVC transform or Chen-DCT in the HM2.0 and the second transform is 4x4 KLT which is applied to the lower frequency components. The second transform is mode dependent 4x4 KLT. The 4x4 KLT is applied only to the lower frequency components in order to reduce the number of operation. For 4x4 transform block, 1 step transform is used. This is because, the number of operation count per pixel increases significantly for larger blocks, but the complexity of 4x4 transform block is considered to be not critical.

## Decoding Process

The data flow of the decoding process is shown in the figure 1.It shows also the operation count in the worst case and the bit precision in an example worst case. We assumed the quantized coefficient could be maximum 16bit and the QP is set to 16 and IBDI extends 2 bit. The decoding process is separated into the low frequency component and the remaining high frequency component. The process for high frequency component is not changed from the conventional process which is in HM2.0,. As for the low frequency component, the dequantization process is between the inverse second transform and the inverse first (core) transform. The inverse second transform is applied to the quantized coefficient in 4x4 low frequency component (1st and 2nd block in right side of the figure). The output value of the inverse second transform is shifted down with the precision of the second transform matrix if both horizontal and vertical inverse second transform are applied (3rd block). Then the multiplication of dequantization table is applied (4th block) The dequantiztion table is identical for one defined for the core transform. There is no additional dequantiztion table. Then the multiplied value may be shifted up or down according to the relative number to be calculated by subtracting the precision of the second transform matrix (7 bit) from the amount of the shift up for dequantization “(QP % 6)” (5th block).

Figure 1. Data flow of the decoding process in the proposed method

## Transform Matrix

We designed one KLT matrix for large transform blocks. In order to get KLT matrix, we derived a covariance matrix for the output of the first transform by a equation M2=TM1TT, where M1 is the covariance matrix for 1-step transform presented in JCTVC-B24[1] and JCTVC-C108[2] and T is the transform matrix of the first transform, i.e. DCT. Since the second transform is applied only for lower frequency components and the transform block size is 4x4, the upper left part of M2ij (0≤i,j<4) is considered for derivation of transform matrix.

As an example, the covariance matrix for 1-step 8x8 transform presented in [1][2] is:

M18×8=1111111112222222123333331234444412345555123456661234567712345678

The covariance matrix for second transform is:

M28×8=1.000−0.357−0.087−0.037−0.020−0.011−0.006−0.003−0.3570.258000000−0.08700.06700000−0.037000.0320000−0.0200000.020000−0.01100000.01400−0.006000000.0110−0.0030000000.010

The transform matrix of second transform with 4x4 size for 8x8 transform block is:

T28×8=1128118−47−10−439109−53−152339109−50182839117

Then, for larger transform blocks including 8x8, we made the slope of M1 more gentle since such large blocks would be usually used for spatially uniform region. Specifically, we used 50% gradient relative to the above covariance matrix M1. The derived covariance matrix and second transform matrix are shown in bellows. The transform matrix for other large transform blocks, 16x16 and 32x32 use the one for 8x8 block in order to reduce the memory storage.

M18×8=1111111111.51.51.51.51.51.51.511.522222211.522.52.52.52.52.511.522.5333311.522.533.53.53.511.522.533.54411.522.533.544.5

T28×8=1128122−37−8−334119−33−101628122−2491421125

## Mode Dependent 2nd Transform

We used the same assignment of transform mode to each intra prediction mode as proposed in JCTVC-C39[5]. The assignment is shown in Table 1.

Table 1. Assignment of the second transform

| “Logical” Mode | Column Transform | Row Transform |
| --- | --- | --- |
| 0 | KLT | NO OPERATION |
| 1 | NO OPERATION | KLT |
| 2 | NO OPERATION | NO OPERATION |
| 3 | KLT | KLT |
| 4 | KLT | KLT |
| 5 | KLT | NO OPERATION |
| 6 | KLT | NO OPERATION |
| 7 | KLT | KLT |
| 8 | NO OPERATION | KLT |
| 9 | NO OPERATION | KLT |
| 10 | KLT | KLT |
| 11 | KLT | KLT |
| 12 | KLT | NO OPERATION |
| 13 | KLT | NO OPERATION |
| 14 | KLT | KLT |
| 15 | KLT | KLT |
| 16 | NO OPERATION | KLT |
| 17 | NO OPERATION | KLT |
| 18 | KLT | KLT |
| 19 | KLT | KLT |
| 20 | KLT | KLT |
| 21 | KLT | KLT |
| 22 | KLT | NO OPERATION |
| 23 | KLT | NO OPERATION |
| 24 | KLT | NO OPERATION |
| 25 | KLT | NO OPERATION |
| 26 | KLT | KLT |
| 27 | KLT | KLT |
| 28 | KLT | KLT |
| 29 | KLT | KLT |
| 30 | NO OPERATION | KLT |
| 31 | NO OPERATION | KLT |
| 32 | NO OPERATION | KLT |
| 33 | NO OPERATION | KLT |

## Complexity Evaluation

### Arithmetic operations

For 4x4 transform block, the AVC 4x4 transform or the 4x4 matrix multiplication is exclusively operated. For large transform blocks, the core transforms and 4x4 KLT are operated continuously. The operations count in the worst case is shown in the table 2. The average operations count and the other detail which are required in CE are included in the accompanying spread sheet “Supplement_2nd”. The derivation process of the operation count in the worst case is included in the spread sheet “Supplement_3rd” and the derivation process of the average operation count is included in the spread sheet “Supplement_4th”.

Table 2. Worst-Case Transform Arithmetic Operations (ops/block)

| Evaluation Metrics |  |  | Total of the proposal | Core Transform | Alternative Transform |
| --- | --- | --- | --- | --- | --- |
| Worst-Case Transform Arithmetic Operations (ops/block) | 4x4 Block | Mults | 128 | 0 | 128 |
|  |  | Adds | 144 | 80 | 144 |
|  |  | Shifts | 32 | 32 | 16 |
|  |  |  | Total of the proposal | Core Transform | 2nd Transform |
|  | 8x8 Block | Mults | 128 | 0 | 128 |
|  |  | Adds | 768 | 640 | 128 |
|  |  | Shifts | 368 | 352 | 16 |
|  | 16x16 Block | Mults | 1536 | 1408 | 128 |
|  |  | Adds | 2752 | 2624 | 128 |
|  |  | Shifts | 1616 | 1600 | 16 |
|  | 32x32 Block | Mults | 7552 | 7424 | 128 |
|  |  | Adds | 13568 | 13440 | 128 |
|  |  | Shifts | 7312 | 7296 | 16 |

### Bit precision

The maximum bit precision needed is 32 bit for all transform sizes.

### Memory requirements

- 32 bytes memory which stores the transform matrix of 1step transform used for 4x4 transform blocks.

- 32 bytes memory which stores the transform matrix of 2nd transform used for large transform blocks.

- 33 bytes memory which stores the map from the intra prediction mode to the alternative transform type. The type is selected in the list, {“no alternative transform”, “horizontal alternative transform only”, “vertical alternative transform only” and “both horizontal and vertical alternative transform”}.

### Others

- No additional de-quantization matrix.

- No additional scan order.

- No additional R-D optimization in the encoder and the decoder.

## Complexity of encoding

In order to make the encoder running faster, the implementation is not completely symmetric to the implementation of the decoder.

### Bit precision

The maximum bit precision needed is 64 bit for large transform sizes and 32 bit for 4x4 transform size.

### Additional memory requirements relative to the decoding

In order to make the encoder running faster, the additional quantization table and the additional estimation error table which is used in RDOQ have been allocated. The 4x4 low frequency components have a same scalar number which is identical to the DC position of the original quantization table. And the elements for high frequency components are identical to the same position of the original quantization table. The purpose is to avoid the additional cycles for conditional indexing of the only one dequantization table in the above manner. This implementation is just an example which is optimized for the runtime on the HM software. It does not suggest that the total memory has to be a double of the original in all other encoder software.

## Simulation Result

Table 4 shows RD performance and execution time of the software relative to HM version 2 software. Regarding high efficiency configuration, the coding gain is 1.1% and 0.5% for Intra HE and Random Access HE respectively. The encoding time (and decoding time) is 101% (102%) for Intra HE, but the significant increase was not observed for Random Access HE. As for low complexity configuration, the coding gain is 1.7% and 0.7% for Intra LC and Random Access LC respectively. The encoding time (and decoding time) is 104% (105%) for Intra LC, but the significant increase was not observed for Random Access LC. Cross-checkers, Toshiba and Samsung have been verified the performance in the table.

Table 4. The RD performance of proposed method (anchor: HM version 2)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr | Y psnr | U psnr | V psnr |
| Class A | -1.2 | -1.3 | -1.2 | -1.6 | -0.5 | -0.4 |
| Class B | -0.9 | -1.0 | -1.1 | -1.5 | -0.7 | -0.7 |
| Class C | -1.1 | -1.2 | -1.1 | -1.7 | -1.0 | -1.0 |
| Class D | -1.1 | -1.0 | -1.0 | -1.6 | -0.8 | -0.8 |
| Class E | -1.7 | -2.2 | -2.0 | -2.4 | -1.0 | -1.3 |
| All | -1.1 | -1.3 | -1.2 | -1.7 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr | Y psnr | U psnr | V psnr |
| Class A | -0.6 | -0.3 | -0.5 | -0.7 | -0.2 | 0.1 |
| Class B | -0.4 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Class C | -0.6 | -0.6 | -0.5 | -0.8 | -0.5 | -0.4 |
| Class D | -0.5 | -0.4 | -0.3 | -0.7 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, low-complexity unification of MDDT and ROT is proposed. An experimental result shows that the proposed method provides 1.1% and 1.7% gain for Intra HE and Intra LoCo and without significant increase of software execution time in comparison with HM version 2. And it shows that the proposed method provides 0.5% and 0.7% gain, respectively, for Random Access HE and LC. We recommend that this proposal be adopted in HM.

# References

- C. Yeo, Y. Tan, Z. Li, S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-B024, Geneva, CH, July 2010.

- A. Saxena, F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C108/m18131, Guangzhou, CN, October 2010

- C. Yeo, Y. Tan, Z. Li, S. Rahardja, “Choice of transforms in MDDT for Unified Intra Prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C039/m18060, Guangzhou, CN, October 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).