| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E075 WG11 Number: m19586 |
| --- | --- |

| Title: | CE7 tool 4: Cross-check report for the proposal JCTVC-E098 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution summarizes Panasonic’s cross-check for the proposal on a mode dependent transform and coefficient scanning for coding of intra prediction residuals proposed by Qualcomm, Toshiba, Institute for Infocomm Research and Huawei. The verification task has been done successfully and the results match those proposed in JCTVC-D392.

- Introduction

- At Deagu meeting, Core Experiment 7 has been decided to evaluate the proposal JCTVC-D392 [1].

# Cross-Checking

The software was complied fine and the cross-checking was performed on 64bit linux system. The simulation has been conducted under the common test conditions [2]. The configuration with default HM scans was tested for both high efficiency and low complexity conditions in the case of I-slice only coding structure and the random access coding structure. And another additional configuration with a directional scan was tested for low complexity conditions in the case of I-slice only coding structure and the random access coding structure. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by the proponents. Table 1 and Table 2 show the cross-checking results and it was confirmed that these results perfectly match with the one provided by the proponents.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.8 | -1.5 | -2.1 | -0.5 | -0.3 |
| Class B | -0.6 | -1.3 | -1.3 | -1.2 | -1.1 | -1.0 |
| Class C | -0.9 | -1.2 | -1.2 | -1.6 | -1.3 | -1.2 |
| Class D | -1.0 | -1.2 | -1.2 | -1.6 | -1.1 | -1.2 |
| Class E | -1.1 | -2.1 | -1.8 | -2.2 | -0.8 | -1.1 |
| All | -0.9 | -1.5 | -1.4 | -1.7 | -1.0 | -1.0 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.9 | -0.6 | -0.7 | -0.2 | 0.2 |
| Class B | -0.3 | -0.4 | -0.1 | -0.5 | -0.3 | -0.3 |
| Class C | -0.5 | -0.6 | -0.5 | -0.8 | -0.6 | -0.5 |
| Class D | -0.4 | -0.5 | -0.4 | -0.7 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.6 | -0.4 | -0.7 | -0.4 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1: Simulation results on the condition with HM default scans

|  | Intra LoCo |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.8 | -0.6 | -0.5 | -1.5 | -0.8 | -0.5 |
| Class B | -2.3 | -1.8 | -1.8 | -1.2 | -0.9 | -0.9 |
| Class C | -1.8 | -1.4 | -1.4 | -0.8 | -0.9 | -0.7 |
| Class D | -2.0 | -1.6 | -1.6 | -0.8 | -0.7 | -0.7 |
| Class E | -3.7 | -1.6 | -1.7 |  |  |  |
| All | -2.5 | -1.4 | -1.4 | -1.1 | -0.8 | -0.7 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 2: Simulation results on the condition with directional scans

# References

- R. Cohen, C. Yeo, R. Joshi and F. Fernandes, “Description of Core Experiment 7: Alternative Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D607, Daegu, KR, January 2011.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, Daegu, KR, January 2011.