| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E076 WG11 Number: m19587 |
| --- | --- |

| Title: | CE7: Cross-check report for Samsung’s proposal JCTVC- E380 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution summarizes Panasonic’s cross-check for JCTVC-E380 proposal on the Rotational transform (ROT). The results for several implementations of ROT and several combinations with a mode dependent DST/DCT transform have been verified by cross checkers. Panasonic has been verified the test 2, 3 and 4 of the following test sets and Qualcomm has been verified the test 1. Tests are, Test 1: ROT with matrix multiplication. Test 2: ROT with a lifting implementation. Test 3: a combination with a mode dependent adaptive DST/DCT transform. Test 4; ROT with restricted encoder search and a combination with mode dependent adaptive DST/DCT transform.

- Introduction

At Deagu meeting, Core Experiment 7 has been decided to evaluate the proposal the Rotational Transform (ROT) relating proposals, JCTVC-D357, JCTVC-D360 and JCTVC-C096. The tools in the documents have been implemented on HM version 2 and have been proposed in JCTVC-E380.

# Cross-Checking

The software was complied fine and the cross-checking was performed on 64bit linux cluster The simulation has been conducted under the common test conditions [2]. Following total 3 configurations have been tested by cross-checkers.

Test 1 = ROT with matrix multiplication

Test 2 = ROT with lifting

Test 3 = ROT + DCT/DST

Test 4 = ROT with restricted encoder search + DCT/DST

Panasonic verified test 2, 3 and 4 for both high efficiency and low complexity conditions in the case of I-slice only coding structure and the random access coding structure based on CE7 test conditions [1]. The encoded images match those of the decoded ones. The PSNR and bitrate values are confirmed identical to the result which is provided by the proponent. Table 1, 2 and 3 show the cross-checking results.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | 0.1 | 0.2 | -2.5 | 1.1 | 1.0 |
| Class B | -1.6 | 1.1 | 0.9 | -2.2 | 1.7 | 1.6 |
| Class C | -1.6 | 1.4 | 1.5 | -2.2 | 1.7 | 1.7 |
| Class D | -1.7 | 1.6 | 1.6 | -2.3 | 1.8 | 1.8 |
| Class E | -1.4 | 0.3 | 0.3 | -2.1 | 1.0 | 1.1 |
| All | -1.6 | 0.9 | 0.9 | -2.3 | 1.5 | 1.5 |
| Enc Time[%] | 163% |  |  | 204% |  |  |
| Dec Time[%] | 106% |  |  | 110% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.1 | 0.1 | -1.1 | 1.4 | 1.4 |
| Class B | -0.9 | 0.7 | 0.7 | -1.4 | 1.1 | 1.1 |
| Class C | -1.1 | 0.8 | 0.8 | -1.4 | 0.8 | 1.0 |
| Class D | -0.9 | 0.5 | 0.5 | -1.2 | 0.9 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | 0.5 | 0.6 | -1.3 | 1.1 | 1.0 |
| Enc Time[%] | 109% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 1: Simulation results for test 2 (ROT with lifting)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.3 | -1.0 | -0.8 | -3.8 | 0.7 | 0.7 |
| Class B | -1.7 | 0.4 | 0.2 | -2.6 | 1.0 | 0.9 |
| Class C | -1.8 | 0.7 | 0.7 | -2.8 | 1.0 | 1.0 |
| Class D | -2.0 | 0.9 | 0.9 | -3.0 | 1.2 | 1.2 |
| Class E | -1.6 | -0.8 | -0.6 | -2.9 | 0.3 | 0.3 |
| All | -1.9 | 0.1 | 0.1 | -3.0 | 0.9 | 0.9 |
| Enc Time[%] | 157% |  |  | 198% |  |  |
| Dec Time[%] | 105% |  |  | 110% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.5 | -0.4 | -1.5 | 1.1 | 1.1 |
| Class B | -0.9 | 0.4 | 0.6 | -1.4 | 0.9 | 0.8 |
| Class C | -1.3 | 0.5 | 0.5 | -1.6 | 0.6 | 0.8 |
| Class D | -1.1 | 0.3 | 0.4 | -1.4 | 0.7 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | -1.0 | 0.2 | 0.3 | -1.5 | 0.8 | 0.8 |
| Enc Time[%] | 108% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2: Simulation results for test 3 (ROT + DCT/DST)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -1.3 | -1.1 | -2.9 | 0.0 | 0.1 |
| Class B | -1.3 | -0.4 | -0.4 | -2.0 | -0.2 | -0.1 |
| Class C | -1.3 | -0.3 | -0.3 | -2.1 | -0.4 | -0.4 |
| Class D | -1.4 | -0.2 | -0.2 | -2.1 | -0.3 | -0.3 |
| Class E | -1.3 | -1.4 | -1.2 | -2.5 | -0.3 | -0.5 |
| All | -1.4 | -0.7 | -0.6 | -2.3 | -0.2 | -0.2 |
| Enc Time[%] | 123% |  |  | 130% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.3 | -0.3 | -1.1 | 0.6 | 0.7 |
| Class B | -0.6 | 0.2 | 0.3 | -1.0 | 0.3 | 0.3 |
| Class C | -0.9 | -0.1 | -0.1 | -1.2 | -0.1 | 0.0 |
| Class D | -0.7 | -0.1 | 0.1 | -1.0 | -0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | 0.0 | 0.0 | -1.0 | 0.2 | 0.3 |
| Enc Time[%] | 104% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Simulation results for test 4 (ROT with restricted encoder search + DCT/DST)

# References

- R. Cohen, C. Yeo, R. Joshi and F. Fernandes, “Description of Core Experiment 7: Alternative Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D607, Daegu, KR, January 2011.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, Daegu, KR, January 2011.