| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E078 |
| --- | --- |

| Title: | CE3: Region-based adaptive interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Yukihiro Bandoh Takeshi Ito Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document reports the performance of a region-based adaptive interpolation filter that adjusts filter coefficients based on the characteristic of the input video signal. In the proposed method, a frame is segmented into multiple regions, and filter coefficients for each region are derived. The basic idea is that filter coefficients are designed for each region in a frame when the frame consists of multiple regions with different characteristics. The proposal was implemented in HM2.0 software to evaluate its performance. Compared to the anchor of HM2.0, the average BD-rate gain for HE-RA, HE-LD, LC-RA and LC-LD were 0.03 %, 0.10 %, 0.42% and 1.13%, respectively. The maximum gain was about 5.6 % for the sequence “vidyo3” in LC-LD case.

# Introduction

Interpolation filter that supports motion estimation with sub-pel accuracy is one of important coding tools for improving efficiency of inter frame coding. In the previous JCT-VC 4th meeting, it was decided that HM2.0 adopts the 8-tap DCT-IF as an interpolation filter for both HE and LC cases. The coefficient values of the filter are constant regardless of the characteristics of the input video sequences. To improve the performance of such a fixed interpolation, adaptive interpolation filters (AIF) were proposed, for example, 2D-AIF [1], SAIF [2]. Each AIF derives the optimal filter coefficients for each frame. However, single set of filter coefficients may not be the optimal option, when the image has plural regions each of which has different characteristics (motion, texture, etc.). In this contribution, a region-based adaptive approach is introduced into AIF. This approach is called Region-based AIF (RBAIF) [3, 4]. RBAIF generates multiple sets of filter coefficients that are optimized for each region.

# Region-Based Adaptive Interpolation Filter (RBAIF)

RBAIF designs the optimal multiple sets of filter coefficients for each region in a frame. For example, let us consider a frame which consists of two regions as shown in Figure 1. In this case, RBAIF generates two sets of filter coefficients, while AIF provides single set of filter coefficients on a frame-by-frame basis.

Figure 1: Conventional AIF (left) and Region-based AIF (right).

The current RBAIF support the segmentation modes which Table.1 shows. Based on criteria shown in Table.1, every pixel in each frame is classified into one of multiple regions. As shown in Table 1, RBAIF also includes DCT-IF and AIF. The parameters  and  for mode 2 and 3 represent the threshold values of motion vectors. The thresholds are determined based on the histogram of motion vectors. In mode 2 and 3, the minimum unit of this classification is 4x4 block. In the mode 5 and 6, the input frame is dual-partitioned. The division line is set each CU size (e.g. 64 pixels). RBAIF needs two kinds of overhead information; indices that identifies selected mode and filter coefficients of each region in the case of mode 1 to mode 6.

Note that the proposed scheme can support arbitrary number of regions. The proposal is currently an extension of SAIF in terms of filter coefficient coding. Of course, the concept of the proposal can be also coupled with other AIF methods.

Table 1: Definition of segmentation modes

When an RBAIF encoder selects the optimum mode among modes listed in Table 1, the encoder takes a two-step approach as follows. In the first step, the encoder selects the best mode among modes that support multi-region whose mode ID in Table 1 are two to six, in terms of MC prediction error. In this step, all candidate modes share common motion vectors. So, there are no needs of additional motion estimation. Then, filter coefficients are provided by solving the linear equation systems. Using the motion vectors and filter coefficients, MC prediction error is derived. Finally, encoder selects the mode that minimizes the sum of MC prediction error, as “best multi-region mode”.

In the second step, the encoder selects the optimum mode among DCT-IF, AIF and the best multi-region mode which is set in the first step. The criterion in this selection is RD cost that consists of rate value derived by entropy coder and distortion value of a local decoded image. In this step, motion vectors derived using DCT-IF is reused.

# Experimental results

The proposal has been implemented in HM 2.0. It is noted that the proposed method is only applied to luminance signal. The configurations of RBAIF are subject to the following conditions:

- Filter type: separable filter

- Tap length: 8 taps

- algorithm to generate filter coefficients of each region: that of SAIF [2]

The other configuration is subject to the common test condition [5].

The objective performance is shown in Table 2. The gains (BD-rate [6]) with the anchor of HM 2.0 are calculated. In the HE configuration, the average gains were about 0.03 % and 0.1 % for RA and LD, respectively. In the LC configuration, those were 0.4 % and 1.3 % for RA and LD, respectively. The maximum gain was about 5.6 % for the sequence “vidyo3” in LC-LD case. It is confirmed that the proposal is effective for high resolutions such as Class A, B and E.

Table 2: RD-performance compared to HM2.0 anchor

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.4 | -0.5 | -1.5 | -0.7 | -0.6 |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | -0.3 | -0.4 |
| Class C | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class D | 0.0 | 0.2 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.4 | -0.2 | -0.3 |
| Enc Time[%] | 152% |  |  | 146% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.1 | 0.0 | -1.3 | 0.2 | 0.6 |
| Class C | 0.0 | 0.1 | 0.1 | -0.5 | -0.1 | -0.2 |
| Class D | -0.1 | 0.1 | 0.0 | -0.1 | 0.4 | 0.2 |
| Class E | -0.2 | -0.5 | 0.5 | -3.0 | 0.1 | -0.1 |
| All | -0.1 | 0.0 | 0.1 | -1.1 | 0.2 | 0.1 |
| Enc Time[%] | 136% |  |  | 125% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

In this contribution, a region-based AIF technique was proposed. Average gains of coding efficiency against HM2.0 of HE case were about 0.03 % and 0.1 % for RA and LD, respectively. Those of LC case were about 0.4 % and 1.1 % for RA and LD, respectively. The proposal can be applied to other AIF methods such as 2D-AIF and EAIF that have already been employed in KTA software. More effective region-dividing methods will be investigated.

It is recommended to incorporate the proposal into HEVC as interpolation method.

# References

[1] Y. Vatis, B. Edler, D. T. Nguyen and J. Osterman., “Two-dimensional non-separable adaptive wiener interpolation filter for H.264/AVC,” ITU-T SG16 Q.6 Document, VCEG-Z17r1, 26th VCEG Meeting, Busan, Korea, April 2005.

[2] S. Wittmann and T. Wedi., “Separable adaptive interpolation filter for video coding,” in Proc. International Conference on Image Processing (ICIP) 2008, San Diego, California, USA, October 2008.

[3] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa., “Region-based adaptive interpolation filter,” JCTVC-B051, 2nd. JCT-VC Meeting, Geneva, CH, 21-28 July, 2010.

[4] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa., “Response to CE3: Region-based adaptive interpolation filter,” JCTVC-D150, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[5] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[6] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).