| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E079 |
| --- | --- |

| Title: | CE12 Subtest1: Improved Deblocking Filter |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Jicheng An, *Qian Huang, *Xun Guo, #Yu-Wen Huang, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | jicheng.an@mediatek.com qian.huang@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes a deblocking filter which is based on the deblocking filter in HM2.0 and the previous contribution of JCTVC-D163. Some modifications are made in luma filter and chroma filter including filtering decision, filter selection and weak filter operation. It is reported that the average BD-rate reductions of 1.2% and 1.7% for six configurations (HE-AI, LC-AI, HE-RA, LC-RA, HE-LD and LC-LD) can be observed for luma and chroma component, respectively. Moreover, it is also reported that the encoding and decoding time of proposed deblocking filter is essentially the same to that of the anchor.

# Introduction

The HEVC deblocking filter shows good coding performance and is computationally simpler than the H.264 deblocking filter. In order to further improve the efficiency of the HEVC deblocking filter, we propose to make some modifications for both luma filter and chroma filter. Specifically, for luma filter, the filtering decision, filter selection and weak filter operations are modified; for chroma filter, the filtering unit size, filtering decision and weak filter operations are modified. Compared to our previous contribution in [1], the directional deblocking filter is removed so that there is no complexity increase for deblocking module.

The rest of this contribution is organized as follows. The algorithm description will be described in Section 2, compression performance will be discussed in Section 4, complexity analysis will be shown in Section 5, and finally Section 6 draws our conclusion.

# Algorithm description

Fig. 1 shows an example of Fig. 1 shows an example of a vertical boundary to be filtered between two luma blocks A and B. The deblocking filter in HM2.0 operates on each 8x8 block in a CU no matter the shapes of A and B. In this example, A and B can be two 8x8 prediction units (PU) or transform units (TU). The major modifications of proposed improved deblocking filter (IDF) are described as the following sub-sections.

Fig. 1. An example of a vertical boundary for deblocking

## Modified Luma Filter

### Modified filtering decision for PU boundary

In HEVC deblocking filter, the luma boundary can be filtered if one of the two neighboring blocks has nonzero transform coefficients no matter the boundary is a PU boundary or TU boundary. That condition is inherited from the H.264 deblocking filter directly, in which the transform size is always equal to prediction size. However, in HEVC, TU size can be larger than PU size so that the PU boundary is not always the TU boundary, which is not the same to H.264.

Therefore, it is proposed that the condition, i.e. one of the two neighboring blocks has nonzero transform coefficients, is only checked for TU boundary. And it is always false when the boundary is only a PU boundary, since in that case both of the two blocks are in the same TU so that there are no artifacts resulting from the block-based transform.

### Luma filtering conditions and operations

The filtering conditions in HEVC deblocking include two parts: conditions for filtering decision and conditions for filter selection. The conditions for filtering decision are used to determine whether a boundary should be filtered or not. If a boundary is determined to be filtered, the conditions for filter selection are used to determine whether a strong filter or a weak filter should be used. We modified both parts and the corresponding thresholds as described in the following sub-sections.

#### Luma filtering decision

We modified the pixel value inequality for filtering decision as follows. In the original inequality, all the pixels in one line shares the same threshold values. We separate the original one inequality into three according to pixel positions. Different threshold values are used for these three inequalities. The modified conditions are as follows:

For the third line and the sixth line, i.e. i=2 and 5, respectively, the three conditions are checked:

,			                           			 	  (1)

,                                                 			 	  (2)

,                                                 				   (3)

where the thresholds α and β can be derived as follows:

,                           			   (4)

,                              			   (5)

where and  are specified in Table 1 with dependency on QP. The A and B are the two neighboring blocks of the boundary.

#### Luma filter selection

If the 8-pixel boundary between A and B has been determined to be filtered, one of the two filters (weak or strong filter) need to be selected for each pixel line. We modified the filter selection conditions as in (6) and (7).

(6)

(7)

And in (7), .

For each line, the strong filter is used if both (6) and (7) are true; otherwise, the weak filter is used.

      - Luma weak filter operation

The filtered values are calculated according to the following equations:

(9)

(10)

(11)

(12)

(13)

(14)

(15)

The clipping value  can be found in Table 1.

The same filter coefficients are applied to three positions to obtain ,, and as defined in (9), (12), and (13).

Table 1. Proposed threshold values , and clipping value

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 4 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| QP | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
|  | 5 | 6 | 7 | 8 | 9 | 10 | 12 | 13 | 15 | 17 | 20 | 22 | 25 | 28 | 32 | 36 | 40 | 45 |
|  | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 10 | 10 |
|  | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 |
| QP | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |
|  | 50 | 56 | 63 | 71 | 80 | 90 | 101 | 113 | 127 | 144 | 162 | 182 | 203 | 226 | 255 | 255 |  |  |
|  | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 |  |  |
|  | 4 | 5 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |  |

## Modified Chroma Filter

### Filtering Unit

In HEVC deblocking filter, the smallest filtering unit is 8x8 for both luma and chroma. i.e. only the PU and TU boundaries lying on 8x8 grid can be filtered for both luma and chroma blocks. In this proposal, we set the smallest chroma deblocking size according to the sampling format. For 4:2:0 format, the smallest chroma deblocking size is 4x4. For 4:2:2 format, the smallest chroma deblocking size is 4x8. And for 4:4:4 format, the smallest chroma deblocking size is 8x8.

### Chroma filtering decision

Compared to HEVC chroma deblocking filter, filtering decision is added for the proposed chroma filter. The chroma boundary between A and B can be filtered if all the following conditions are true:

- A or B is intra coded

- conditions in (1), (2) and (3)

For chroma filter, the second pixel line, i.e. i=1 in (1), (2) and (3) is used in the filtering decision when filtering process is performed on 4-pixel edges (for example, for 4:2:0 format). The third and sixth pixel lines, i.e. i=2 and 5, are used when the chroma filtering is performed on 8-pixel edges (for example, 4:4:4 format). The thresholds α and β are also derived by (4) and (5), and and  are specified in Table 1 with dependency on QP of chroma.

### Chroma filter operation

For chroma filtering, only p0 and q0 values are modified. The filter operation is the same as luma weak filter, as in (9), (10), (11).

- Combination test

In CE14 [3], the deblocking filter is classified into different tools, such as luma filter and chroma filter. In order to verify that different tools can be combined to achieve additional benefits. We also test the combination of the luma filter from [4] and the chroma filter from this contribution.

# Experimental results

In order to verify the performance of the proposed IDF, we have run simulations according to the test conditions of JCTVC-D600 [2]. Table 2 shows the experimental results of IDF. It can be seen that the proposed IDF can achieve an average BD-rate reductions of 1.2% for luma and 1.7% for chroma, respectively. Table 3 shows the combination results of luma filter from [3] and chroma filter from this contribution. Similar gains can be achieved by

Table 2. Experimental results for IDF compared with anchor of HM2.0

|  | Intra |  |  | Intra_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.9% | -1.0% | -1.3% | -2.4% | -2.1% | -2.0% |
| Class B | -1.2% | -1.2% | -1.8% | -1.4% | -2.8% | -3.0% |
| Class C | -1.0% | -2.1% | -2.1% | -1.2% | -3.6% | -4.1% |
| Class D | -0.9% | -2.0% | -2.2% | -1.1% | -4.0% | -4.4% |
| Class E | -1.3% | -0.3% | -0.5% | -1.5% | -2.5% | -3.3% |
| All | -1.3% | -1.4% | -1.6% | -1.5% | -3.0% | -3.4% |
| Enc Time[%] | 99.9% |  |  | 97.1% |  |  |
| Dec Time[%] | 99.1% |  |  | 100.8% |  |  |
|  |  | RandomAccess |  |  | RandomAccess_LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2% | -0.9% | -1.2% | -1.6% | -1.4% | -1.4% |
| Class B | -0.9% | -1.6% | -1.7% | -1.2% | -1.7% | -1.6% |
| Class C | -0.8% | -2.5% | -2.4% | -0.8% | -2.3% | -2.5% |
| Class D | -0.7% | -2.2% | -2.4% | -0.7% | -2.0% | -2.2% |
| Class E |  |  |  |  |  |  |
| All | -0.9% | -1.8% | -1.9% | -1.1% | -1.8% | -1.9% |
| Enc Time[%] | 100.1% |  |  | 98.1% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.7% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -0.7% | -0.9% | -1.2% | -0.6% | -0.3% |
| Class C | -0.9% | -1.0% | -1.1% | -1.1% | -1.0% | -1.2% |
| Class D | -1.1% | -0.7% | -1.2% | -1.0% | -0.7% | -0.6% |
| Class E | -0.9% | -0.3% | -0.2% | -1.4% | -1.9% | -1.7% |
| All | -0.9% | -0.7% | -0.9% | -1.2% | -1.0% | -0.9% |
| Enc Time[%] | 100.5% |  |  | 97.6% |  |  |
| Dec Time[%] | 98.7% |  |  | 100.2% |  |  |

The chroma filter improvements presented in Section 2.2 have also been tested independently based on HM2.0 and the results are shown in Table 3. For the consistency with HEVC luma filter, in this experiment, the inequations of chroma filtering decision in Section 2.2.2 have been modified according to HEVC luma filtering decision inequations. It can be seen that around 1.8% chroma BD-rate reduction can be achieved, which is almost the same to that shown in the whole experiment (Table 2).

Table 3. Experimental results for modified chroma filter compared with anchor of HM2.0

|  | Intra |  |  | Intra_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -1.1% | -1.3% | 0.0% | -2.2% | -2.1% |
| Class B | 0.0% | -1.4% | -2.0% | 0.0% | -2.7% | -2.8% |
| Class C | 0.0% | -2.3% | -2.3% | 0.0% | -3.4% | -3.9% |
| Class D | 0.0% | -2.1% | -2.4% | 0.0% | -3.6% | -4.0% |
| Class E | 0.0% | -0.3% | -0.5% | 0.0% | -2.5% | -3.3% |
| All | 0.0% | -1.5% | -1.8% | 0.0% | -2.9% | -3.2% |
| Enc Time[%] | 100.4% |  |  | 98.6% |  |  |
| Dec Time[%] | 99.2% |  |  | 99.6% |  |  |
|  |  | RandomAccess |  |  | RandomAccess_LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -0.9% | -1.3% | -0.1% | -2.2% | -2.1% |
| Class B | 0.0% | -1.6% | -1.8% | 0.0% | -1.8% | -1.7% |
| Class C | -0.1% | -2.7% | -2.4% | -0.1% | -2.7% | -3.0% |
| Class D | 0.0% | -2.0% | -2.4% | 0.0% | -2.1% | -2.3% |
| Class E |  |  |  |  |  |  |
| All | 0.0% | -1.8% | -2.0% | 0.0% | -2.2% | -2.2% |
| Enc Time[%] | 100.0% |  |  | 102.4% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.1% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.7% | 0.0% | -0.8% | -0.5% |
| Class C | 0.0% | -1.0% | -1.1% | 0.0% | -1.2% | -1.4% |
| Class D | -0.1% | -0.6% | -1.0% | 0.0% | -0.9% | -0.7% |
| Class E | -0.1% | -0.3% | -0.4% | 0.0% | -1.6% | -1.7% |
| All | -0.1% | -0.6% | -0.8% | 0.0% | -1.1% | -1.0% |
| Enc Time[%] | 99.9% |  |  | 99.2% |  |  |
| Dec Time[%] | 99.9% |  |  | 98.8% |  |  |

Table 4. Experimental results for the combination

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.1 | -1.3 | -1.5 | -2.2 | -2.1 |
| Class B | -0.7 | -1.4 | -2.0 | -0.8 | -2.7 | -2.8 |
| Class C | -0.6 | -2.3 | -2.3 | -0.8 | -3.4 | -3.9 |
| Class D | -0.6 | -2.1 | -2.4 | -0.6 | -3.6 | -4.0 |
| Class E | -0.9 | -0.3 | -0.5 | -1.0 | -2.5 | -3.3 |
| All | -0.7 | -1.5 | -1.8 | -0.9 | -2.9 | -3.2 |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -1.5 | -1.5 | -1.4 | -2.1 | -1.8 |
| Class B | -1.0 | -2.2 | -2.3 | -1.1 | -2.5 | -2.1 |
| Class C | -0.9 | -3.3 | -3.1 | -0.8 | -3.2 | -3.4 |
| Class D | -0.6 | -2.9 | -3.0 | -0.6 | -2.8 | -2.7 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -2.5 | -2.5 | -1.0 | -2.6 | -2.5 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -2.7 | -2.5 | -1.3 | -2.4 | -1.5 |
| Class C | -1.1 | -2.6 | -2.9 | -0.9 | -2.3 | -2.8 |
| Class D | -0.9 | -1.9 | -2.2 | -0.6 | -1.9 | -1.6 |
| Class E | -3.0 | -3.8 | -4.2 | -2.6 | -7.1 | -5.7 |
| All | -1.5 | -2.7 | -2.9 | -1.3 | -3.1 | -2.7 |

# Complexity Issue

In order to evaluate the complexity of the proposed deblocking filter, the decoding time on a single machine has been measured for both the proposed algorithm and the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time. Table 5 shows the decoding time of the version with only chroma improvements in Section 2.2 and the version with the whole proposed improvements, compared with HM2.0. It shows that complexity is not increased for both chroma and luma improvements. Therefore, the proposed IDF has a similar complexity compared with the HM2.0 anchor.

Table 5. Geometry mean of decoding time compared with HM2.0

|  | Dec. time HM 2.0 [s] | Dec. time Section 2.2 [s] |  | Dec. time. Proposed IDF[s] |  |
| --- | --- | --- | --- | --- | --- |
| Intra HE | 69.3 | 68.7 | 99.1% | 68.1 | 98.3% |
| Intra LoCo | 30.0 | 30.0 | 100.0% | 29.9 | 99.7% |
| Random Access HE | 33.7 | 33.4 | 99.2% | 33.9 | 100.6% |
| Random Access LoCo | 18.5 | 18.5 | 99.8% | 18.6 | 100.3% |
| Low Delay HE | 28.1 | 28.1 | 100.0% | 27.8 | 99.1% |
| Low Delay LoCo | 15.9 | 15.9 | 100.0% | 16.0 | 100.5% |
| Average |  |  | 99.7% |  | 99.7% |

# Conclusion

The contribution proposed a deblocking filter that is a further development based on the HEVC deblocking filter. Some modifications were made to further improve the coding performance, which includes filter decision, filter selection and filtering operations for luma and chroma. The average BD-rate reductions of 1.2% and 1.7% can be observed for luma and chroma, respectively. In addition, a combination of tools is also tested, which consists of the luma filter in [4] and the chroma filter from this contribution. The results show that the coding gain can be additive. Moreover, all of the improvements have no complexity increase.

# References

[1] J. An, X. Guo, Q. Huang, Y.-W. Huang and S. Lei, “CE8 Subtest1: Improved deblocking filter,” JCTVC-D163, JCTVC meeting, Daegu, KR. Jan. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC meeting, Daegu, KR, Jan. 2011.

[3] A. Norkin, B. Jeon and M. Narroschke, “CE12: Summary report of core experiment on deblocking filtering,” JCTVC-E032, JCTVC meeting, Geneva, 2011.

[4] A. Norkin, K. Andersson and R. Sjoberg, “CE12.1: Ericsson deblocking filter,” JCTVC-E276, JCTVC meeting, Geneva, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).