| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E080 |
| --- | --- |

| Title: | Adaptive Inter Mode Coding for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ximin Zhang Shan Liu Shawmin Lei 2860 Junction Ave. San Jose, CA95134, USA | Emails: | Ximin.zhang@mediatek.com Shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This contribution reports methods and corresponding results for coding the CU mode in inter slice with low complexity entropy coding (LCEC). In the proposed methods, the codeword length of each inter prediction mode is adjusted based on accumulated statistics and probabilities of a content dependent interval in a slice. The accumulated statistics and probabilities are measured by a set of counters. An average of 0.2% BD-rate savings are observed for low complexity configurations with random access setting, and an average of 0.3% BD-rate savings are observed for low complexity configurations with low delay setting. No encoding or decoding time increase.

- Introduction

Similar to other conventional video coding standards, the proposed video coding standard HEVC (High Efficiency Video Coding, a successor to H.264/AVC, i.e. Advanced Video Coding standard) utilizes a fixed order to code various inter prediction modes. According to the most up-to-date HEVC working draft (JCTVC-D503), the coding tree, coding unit syntax and prediction unit syntax are shown in Table 1a, Table 1b and Table 1c, respectively.

Table 1a Coding tree syntax in HEVC HM2.0

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| ... |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| ... |  |
| } |  |

Table 1b Coding unit syntax in HEVC HM2.0

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| … |  |
| if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_mode | u(v) ¦ ae(v) |
| ... |  |
| } |  |

Table 1c Prediction unit syntax in HEVC HM2.0

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| … |  |
| } |  |

From above tables, the following number of syntax bits will be needed to code each mode in a given coding unit level.

|  | Split | SKIP | 2Nx2N | 2NxN | Nx2N | NxN | INTRA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Furthermore, within each prediction unit (PU) there might be a merge_flag which specifies whether this PU is merged to neighbors or is self-coded. Additional bit may have to be used for 2Nx2N, 2NxN, Nx2N and NxN.

# Proposed Methods

This proposal proposes a method for adapting the codeword length of each inter prediction mode, based on accumulated statistics and probabilities, such that the overall coding efficiency is improved.

In the proposed method, the code word is adapted for each mode in each depth. It is shown in Table 2. In this table, an index value “Index()” is used to link the specified mode to a specified code word. The value of Index() is from 0 to the number of total coding modes (flags) minus one. The small index will use shorter code word and big index will use longer code word. For each Index value, the corresponding coding word can be shown as following table 3. During the coding process, a counter is assigned to each coding mode at each depth, and the counter is reset to zero at the beginning of each content dependent interval in each slice. At the beginning of each interval, a certain codeword table (or order of coding prediction modes as in HEVC HM2.0, as an example) is used to specify the code word for each prediction mode. Then, during the encoding process, when each time a prediction mode is selected, the corresponding counter is incremented by one. The codeword table keeps adapting to ensure the prediction modes with greater counter values use shorter code words. For example, when the counter of Mode2 is increased to the number greater than the counter of Mode1, The index value of Mode1 and Mode2 is exchanged. Their code word will be swapped correspondingly.

Table 2 Coding syntax design for CU with depth “uiDepth”

|  | Coding syntax |
| --- | --- |
| Mode1[uiDepth] | Syntax[Index (Mode1 [uiDepth])] |
| Mode2[uiDepth] | Syntax[Index (Mode2 [uiDepth])] |
| Mode3[uiDepth] | Syntax[Index (Mode3 [uiDepth])] |
| Mode4[uiDepth] | Syntax[Index (Mode4 [uiDepth])] |
| Mode5[uiDepth] | Syntax[Index (Mode5 [uiDepth])] |
| Mode6[uiDepth] | Syntax[Index (Mode6 [uiDepth])] |
| Mode7[uiDepth] | Syntax[Index (Mode7 [uiDepth])] |
| Mode8[uiDepth] | Syntax[Index (Mode8 [uiDepth])] |
| … | … |

Table 3 Example Syntax coding table with different Index value

|  | Coding word syntax |
| --- | --- |
| Syntax[0] | 1 |
| Syntax[1] | 01 |
| Syntax[2] | 001 |
| Syntax[3] | 0001 |
| Syntax[4] | 00001 |
| Syntax[5] | 000001 |
| Syntax[6] | 0000001 |
| Syntax[7] | 00000001 |
| … | … |

During the encoding process, the counters keep accumulating, the counters are adjusted at coding unit (CU) level and thus, the prediction mode syntax coding is adapted at CU level as well. Consequently, the decoder will use the same counting mechanism to update the inter prediction mode syntax coding.

The corresponding syntax change is yellow highlighted in Table 4.

Table 4a Proposed Coding tree syntax for HEVC HM

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| ... |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(v) ¦ ae(v) |
| ... |  |
| } |  |

Table 4b Proposed Coding unit syntax for HEVC HM

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(v) ¦ ae(v) |
| … |  |
| if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_mode | u(v) ¦ ae(v) |
| ... |  |
| } |  |

Table 4c Proposed Prediction syntax for HEVC HM

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| merge_flag[ x0 ][ y0 ] | u(v) ¦ ae(v) |
| … |  |
| } |  |

# Simulation results

Simulations follow common test conditions defined in JCTVC-600 Error: Reference source not found. Results are shown in table 5.

Table 5	 Results for proposed global counters scheme compared with anchor.

|  |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.2 | -0.2 | -0.1 |  |
|  | Class B | 0.0 | -0.1 | 0.0 |  |
|  | Class C | -0.2 | -0.1 | -0.1 |  |
|  | Class D | -0.3 | -0.2 | -0.1 |  |
|  | Class E |  |  |  |  |
|  | All | -0.2 | -0.1 | -0.1 |  |
|  | Enc Time[%] | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |
|  |  | Low delay LoCo |  |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A |  |  |  |  |
|  | Class B | -0.3 | -0.1 | 0.4 |  |
|  | Class C | -0.4 | -0.1 | -0.3 |  |
|  | Class D | -0.3 | 0.0 | 0.2 |  |
|  | Class E | -0.3 | 0.2 | 0.2 |  |
|  | All | -0.3 | 0.0 | 0.1 |  |
|  | Enc Time[%] | 101% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |

Table 6	 Results for proposed global counters scheme compared with fixed table1.

|  |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -1.8 | -1.1 | -1.0 |  |
|  | Class B | -1.9 | -1.9 | -1.7 |  |
|  | Class C | -1.5 | -1.5 | -1.5 |  |
|  | Class D | -1.2 | -1.1 | -1.1 |  |
|  | Class E |  |  |  |  |
|  | All | -1.6 | -1.4 | -1.4 |  |
|  | Enc Time[%] | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |
|  |  | Low delay LoCo |  |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A |  |  |  |  |
|  | Class B | -1.9 | -2.5 | -2.1 |  |
|  | Class C | -1.0 | -1.3 | -1.3 |  |
|  | Class D | -1.0 | -1.5 | -1.3 |  |
|  | Class E | -3.7 | -3.7 | -3.8 |  |
|  | All | -1.8 | -2.2 | -2.0 |  |
|  | Enc Time[%] | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |

Note1: anchor used HM2.0 software under common test conditions [2] with fixed codeword table (CU adaptive LCEC inter mode coding (QC_LCEC_INTER_MODE) disabled).

# Conclusions

This contribution proposes methods for adaptive inter mode coding on LCEC. Based on the results, we recommend the proposed counter based scheme to be adopted into HM for LCEC inter mode coding.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.