| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E081 WG11 Number: m19595 |
| --- | --- |

| Title: | Syntax and structure for coding and utilizing partition types |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu Ximin Zhang Shawmin Lei 2860 Junction Ave. San Jose, CA95134, USA | Emails: | shan.liu@mediatek.com ximin.zhang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This contribution reports a couple of methods and syntax modifications for coding and utilizing partition types. First, a modification to the current inter partition type code word table is proposed, which aligns the syntax of coding partition types for SCU (smallest coding unit) to other CUs in all depth. Moreover, an average of 0.1% BD-rate savings are observed for Low Complexity configurations (coding gain/loss is negligible for HE in overall) with no encoding or decoding time increase. Second, a hierarchical partition type structure is proposed, which reduces encoding time by around 13% in average, with an average 0.4% BD-rate increase. No increase for decoding time.

- Introduction

The current HEVC (High Efficiency Video Coding) defines coding unit (CU) and prediction unit (PU) where a CU may consists of one or multiple PU. Up to this meeting (HM2.0), four partition types and PU sizes have been defined and used, as shown in Figure 1. Among them, an INTRA coded 2Nx2N CU may select 2Nx2N or NxN partition type if it is a smallest CU (SCU); otherwise it can only select 2Nx2N partition type. For an INTER coded 2Nx2N SCU, all four partition types may be selected, but excluding NxN if CU is greater than SCU.

Figure 1 Various partition types in a 2Nx2N CU.

Consequently, a set of code words are used to represent these partition types, as shown in Table 1 and Table 2.

Table 1 Codeword table for various partition types in HEVC HM2.0, when CU is greater than SCU.

Table 2	Codeword table for various partition types in HEVC HM2.0 when CU is SCU.

In summary, the codeword table for INTER frames and slices in current HEVC HM2.0 is illustrated as in Table 3.

Table 3 Codeword table for Inter partition types in HEVC HM2.0.

| Partition type | CU>SCU | CU==SCU |
| --- | --- | --- |
| INTER 2Nx2N | 1 | 1 |
| INTER Nx2N | 01 | 01 |
| INTER 2NxN | 001 | 001 |
| INTER NxN | NA | 0001 |
| INTRA 2Nx2N | 000 | 00001 |
| INTRA NxN | NA | 00000 |

# Proposed Methods

## Syntax modification for SCU partition types

During the investigation towards the adopted contribution JCTVC-D432[1], we found that removing INTER NxN partition type from SCU partition types only resulted in average 0.1% BD-rate increase. Inspired by this observation we propose to shuffle the INTER NxN prediction type to the last, i.e. least significant code word in the codeword table, as shown in Table 4.

Table 4	 Proposed codeword table for Inter partition types in HEVC.

| Partition type | CU>SCU | CU==SCU |
| --- | --- | --- |
| INTER 2Nx2N | 1 | 1 |
| INTER Nx2N | 01 | 01 |
| INTER 2NxN | 001 | 001 |
| INTRA 2Nx2N | 000 | 0001 |
| INTRA NxN |  | 00001 |
| INTER NxN |  | 00000 |

The corresponding syntax change is shown in Table 5.

Table 5	 Proposed CU syntax modification.

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( slice_type != I ) |  |
| inter_partitioning_idc | ue(v) ¦ ae(v) |
| ... |  |
| if( CU == SCU ) { |  |
| scu_split_flag | ue(1) ¦ ae(v) |
| if(scu_split_flag) { |  |
| scu_intra_inter_flag | ue(1) ¦ ae(v) |
| ... |  |
| } |  |
| ... |  |
| } |  |

Semantics

scu_split_flag specifies whether the current CU (i.e. an SCU) is split into four prediction blocks with half horizontal and vertical size.

scu_intra_inter_flag specifies whether the current CU (i.e. SCU) is predicted by using four INTRA or four INTER prediction units.

This syntax, compared with the existing one, has two advantages as follows.

- SCU codeword table better aligns with CU in other depths, thus the overall syntax and coding flow is more clean and tidier.

- It improves the coding efficiency slightly by reducing averagely 0.1% BD-rate for low complexity configurations. Both encoding and decoding time remain the same.

## Hierarchical partition type structure

We further propose a hierarchical partition type structure, in which the number of allowed partition types is associated with the depth, as shown in Table 6.

Table 6	 Proposed hierarchical partition type structure and codeword table.

| Partition type | Depth = 0 | Depth = 1 | Depth = 2 | Depth = 3 |
| --- | --- | --- | --- | --- |
| INTER 2Nx2N | infer | 1 | 1 | 1 |
| INTRA 2Nx2N | n/a | 0 | 000 | 000 |
| INTER 2NxN | n/a | n/a | 001 | 001 |
| INTER Nx2N | n/a | n/a | 01 | 01 |
| INTRA NxN | n/a | n/a | n/a | 00001 |
| INTER NxN | n/a | n/a | n/a | 00000 |

# Simulation results

Simulations follow common test conditions defined in JCTVC-600 Error: Reference source not found. Results are shown in Table 7 and Table 8.

Table 7	 Results for proposed method 2.1 compared with anchor.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 |
| Class B | 0.0 | 0.1 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.1 | -0.1 | -0.2 | -0.2 |
| Class D | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | -0.1 | -0.2 | -0.1 |
| Class C | 0.0 | 0.3 | 0.1 | -0.1 | -0.2 | -0.2 |
| Class D | -0.1 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 |
| Class E | 0.0 | -0.4 | 0.0 | -0.1 | -0.2 | -0.1 |
| All | 0.0 | 0.0 | 0.0 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 8	 Results for proposed method 2.2 compared with anchor.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 1.3 | 0.8 | 0.7 | 1.0 | 0.8 |
| Class B | 0.5 | 0.6 | 0.6 | 0.7 | 0.5 | 0.6 |
| Class C | 0.3 | 0.4 | 0.4 | 0.2 | 0.3 | 0.3 |
| Class D | 0.3 | 0.2 | 0.3 | 0.0 | 0.1 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.6 | 0.5 | 0.4 | 0.5 | 0.5 |
| Enc Time[%] | 88% |  |  | 88% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.0 | 0.9 | 0.4 | 0.5 | 0.9 |
| Class C | 0.4 | 0.8 | 0.7 | -0.1 | 0.0 | -0.1 |
| Class D | 0.2 | 0.5 | 0.3 | -0.1 | -0.1 | 0.1 |
| Class E | 0.9 | 1.6 | 2.8 | 1.1 | 2.1 | 2.0 |
| All | 0.5 | 0.9 | 1.1 | 0.3 | 0.5 | 0.6 |
| Enc Time[%] | 88% |  |  | 87% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution proposes a couple of methods for coding and utilizing partition types in HEVC. First a modification to the current inter partition type codeword table is proposed, which aligns the syntax for coding partition types for SCU to that for other CUs in all depth, and at the same time results in averagely 0.1% BD-rate reduction under Low Complexity configurations. Neither encoding nor decoding time is increased. Second, a hierarchical partition type structure is proposed, which reduced encoding time by 12% for HE, 13% for LC cases, with an average 0.4% BD-rate increase. We recommend the committee to consider including the first method in the future HM. We also recommend the committee to consider the second method for adoption or further study.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Shan Liu, Yu-Wen Huang and Shawmin Lei, “Remove Partition Size NxN”, JCTVC-D432, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.